# Appendix A-2

This appendix describes how the building type effective full-load hours (EFLH) and coincident factor (CF) values were developed using data gathered as part of the 2006-2008 Small Commercial Contract Group evaluation (Small Com evaluation).  It also describes how the activity area distributions were calculated.

## EFLH and CF Calculations

### Setup (applies to both the EFLH and CF calculations)

The Small Commercial evaluation involved the installation of nearly 7,000 lighting loggers in over 1,200 commercial buildings throughout California.  Extensive training and QC procedures were applied to the lighting logger installations, as is documented in the Small Com report.

The ON/OFF transition data from each lighting logger was processed into a percent ON per hour format.  Each logger was then extrapolated into an 8760 lighting use shape, as described in the Small Com report.  Since some sites saw multiple loggers installed in the same activity areas, the 8760 use shapes were averaged together to create a single 8760 for each activity area at each site.  The number of lamps represented by each logger was used as a weight to create this site level activity area shape.

The first step in calculating the EFLH and CF values for this DEER update was the creation of the weekly usage profiles.  Each site level activity area 8760 was collapsed to an 8-day use shape (five weekdays, 2 weekends and an eighth day for holidays).  These site level activity area 8-day shapes were then averaged together to make an 8-day shape for each building type - activity area that was reported in Small Com.  The total number of lamps logged within each activity area at each site was used as a weight to create these building type - activity area shapes.

In this way, we arrived at a percent ON value for each hour (0-23) of each weekday (1-8) at each building type – activity area.  To explain in symbols, if we let where HRh,w,b,a be the average hours ON during hour h on weekday w at building type b and activity area a, then we have:

HRh,w,b,a=sws,a∙HRs,h,w,asws,a

where the sums are taken over all sites s such that s has building type b and activity area a.

The term ws,a represents the total number of lamps at site s in activity area a.  So, in this case, sws,a would represent the total number of lamps in activity areas a in sites with building type b.  The term HRs,h,w,a represents the percent ON in activity area a site s during hour h on weekday w.

### EFLH Calculation

The calculation of the EFLH value at building type b (EFLHb) is most clearly described by the following equations:

EFLHb=aPctAAb,a∙EFLHb,a

where the sum is taken over all activity areas a such that building type b has activity area a as a final reported activity area.

The term PctAAb,a represents the percentage that activity area a takes up in building type b (so we have aPctAAb,a=1 for each b; the calculation of these PctAAb,a terms is described in the next section).

The term EFLHb, a is the EFLH at building type b and activity area a, as defined by:

EFLHb, a=354w=17DayTotw,b,a7+11DayTot8,b,a

The term DayTotw,b,a represents the average daily hours ON for weekday w (Sunday=1, Monday=2,…,Saturday=7, and Holiday=8), at building type b and activity area a.  It is defined as

DayTotw,b,a=h=023HRh,w,b,a

where HRh,w,b,a is the average hours ON during hour h (0≤h≤23) on weekday w at building type b and activity area a, as described in the section above.  Note, 0≤HRh,w,b,a≤1.

### CF Calculation

The DEER Peak Definition is as follows:

“The DEER demand impact is defined as the average demand impact, for an installed measure, as would be “seen” at the electric grid level, averaged over the nine hours, between 2PM and 5PM, during the three consecutive weekday period which contains the weekday with the highest temperature of the year.”

Since the profiles used for this update are 8-day profiles, we average the weekday hours between 2 and 5pm to find the CF.  In symbols, the CF at building type b is calculated as follows:

CFb=aPctAAb,a∙CFb,a

where CFb,a represents the CF at building type b and activity area a (the term PctAAb,a is the same as mentioned above and defined below).  The term CFb,a is calculated as

CFb,a=w=26h=1416HRh,w,b,a15

We divide by 15 here because we are taking the average over 15 hours.  The term  HRh,w,b,a is the same as used above.

## Activity Area Distributions

The Small Com onsite survey included an inventory of rebated technologies beyond the lamps that were actually logged.  These data were used to calculate the activity area distributions, which represent the percentage of building lighting attributable to each activity area.  These activity area distributions were created separately for CFL and linear lamps.

The first step in the calculation was to find the total lamps surveyed within each activity area at each site.  These site – activity area lamp counts were then summed to find the total number of lamps in each building type – activity area.

The lamp count from each building type – activity area was divided by the total number of lamps for that building type to find the percentage distribution.  Letting PctAAb,a represent the percentage that activity area a represents within building type b, this calculation is described by the following equations:

PctAAb,a=Lampsb,aLampsb

where Lampsb=aLampsb,a (with the sum being taken over all activity areas a contained within building type b) represents the total number of lamps found at all sites having building type b.  Likewise, the term Lampsb,a represents the  total number of lamps found in activity areas a at sites with building type b.  It is calculated as

Lampsb,a=sLampss,a

where the sum is taken over all sites s with building type b.  The term LAmpss,a represents the total number of lamps surveyed in activity area a at site s.

The numbers 11 and 354 appear in this calculation because we assume there are 11 holidays and 354 non-holidays in a year.

http://www.doe2.com/download/AvoidedCost/ConsultantsReport_2006-03-20/3-21-06%20Update%20Attach3.pdf