# Introduction - DEER 2011 Update Analysis

## Introduction

This report presents recommended updates to a portion of the estimated load impacts and net-to-gross of energy efficiency measures currently found within the DEER data base and for some new measures likely to be promoted in the investor-owned utilities’ energy efficiency programs starting in the bridge funding year of 2013. These updated values are intended for use as ex ante values, as directed by the October 25th, ACR in Rulemaking R09-11-014, for estimating likely program savings and the cost effectiveness of measures and programs for the 2013-2014 bridge-portfolio.

The process used to develop these updates included the following steps:

- Develop an analytical process to prioritize and to select which measure parameters both should and could be included in this cycle. This process includes a review of the existing uncertainties in the DEER estimates, ranking of the measures based on the forecast of their relative share of 2010-2012 overall energy savings at the statewide level, a consideration of the availability of new information from the 2006-2008 CPUC impact evaluation studies, and a consideration of technical, regulatory or  other factors related to the feasibility and requirements of including particular updates in the DEER update for the 2013-2014 bridge period.

- Select the measures for the development of analysis plans   based on a review of available data from the 2006-08 evaluation studies and more recent market shipments data. This includes an assessment of whether the samples and data used to develop new parameter estimates  were sufficiently large and representative of the population to support replacing the current parameter values.

- Execute the analysis plans to derive proposed updates for inputs and output for each measure parameter.

- Compare the resulting changes in DEER measure outputs, such as unit energy savings, with the outputs in the existing data base, to enhance quality control and identify and summarize the effect of the proposed changes on net and gross savings estimates

- Prepare the final documentation for the analysis and insert the new values into the updated DEER data base. All of the DEER files can be viewed at   the following link ftp://deeresources.com/pub/SPTdb/Latest-SPTdb-Viewer.zip.  This provides access to the Frozen Ex Ante Database Viewer.

In addition, the DEER team has fixed some software bugs and made changes to building prototype characteristics and lighting impact estimates in DEER version 3.02.  These changes are summarized   in Appendix A-1.

The analytical process used to select the measure specific parameters to be updated in 2011 is detailed in Chapter 2 of this document.  Below we provide an overview of the technology groups that were formed to execute the analysis steps described above.

The DEER Update analysis was performed by ten separate technology analysis teams. Each technology area was defined to cover a specific market segment, group of end users, and energy efficiency measures currently promoted by energy efficiency programs within the technology/market segment. Analysts were selected to serve on each technology group based on their prior experience and expertise in estimating baseline energy use, impact parameters, and net-to-gross values for measures in that area.  An overview of these technology group market segments and common measures installed in them is provided in Table 1-1.

Table 1-1:  Organization Structure of DEER Analysis Teams

| Technology Group | Common Efficiency Measures |
| --- | --- |
| 1. Nonresidential Lighting | CFL’s, Linear Fluorescent Lamps and Ballast, High intensity discharge (HID) fixtures |
| 2. Commercial HVAC | Water-Cooled Chillers, Quality Maintenance (refrigerant charge adjustment, coil cleaningretrocommisioning packages), “Frictionless” Chillers |
| 3. Residential & Nonresidential Hot Water Systems | Multifamily boilers, heaters, gas instantaneous water heaters, pool heaters |
| 4. Commercial Information Technology & Plug Loads | Computer Power Management programs, smart strips, CPU sleep programs |
| 5. Commercial Refrigeration | Fan Motors, Efficient Lighting in Refrigeration cases, gaskets and covers |
| 6. Industrial & Agriculture-All End Uses | Steam Traps, Greenhouse, process boilers and many custom measures |
| 7. Residential Lighting | CFLs, LEDs, Linear fluorescent fixtures |
| 8. Residential HVAC | Gas Furnaces, Room and Central Air Conditioners, duct sealing and refrigerant charge |
| 9. Residential Appliances & Plug Loads | Clothes Washer, Refrigerator Freezer, Television, smart power strips, etc. |
| 10. Commercial Foodservice | Gas Fryer, Gas Oven, Gas Convection Oven, Electric Oven, Electric Steam Cooker |

These ten technology groups reviewed the ranked list of energy efficiency measures within their sector and assessing the quality and availability of data from the 2006-08 evaluation studies for use in the update.  Based on this review, the DEER team concluded that there was not sufficient evaluation or market data available to provide any updates for energy efficiency measure in the Computer Information and Plug Loads and Commercial Food Service market segments as part of the 2011 update.

After reviewing the available evaluation data, the DEER team recommended that the 2011 update focus on revising the load impact estimates (unit energy saving and unit peak savings) for residential and non residential lighting and residential hot water systems. The team recommended  that the residential and non residential lighting analysis  concentrate  on using the available evaluation data for estimated hours of operation by building type, usage area or type, and differences in more efficient wattage and the baseline wattage (delta watts) for selected lighting technologies.  In addition, the DEER team decided that there was sufficient data available to provide updated estimates of net to gross ratio (NTGR) estimates for eight of the ten technology groups. The technology parameter areas selected for update in 2011 are listed in Table 1-2 below.

Table 1-2:  Technology Areas Selected for Update in 2011

| Load Impact Updates in 2011 | NTGR Updates in 2011 |
| --- | --- |
| Non Residential Lighting | Non Residential Lighting |
| Residential Lighting | Residential Lighting |
| Residential Water Heating Systems | Large Industrial and Commercial Custom Measures |
|  | Commercial HVAC |
|  | Commercial Refrigeration |
|  | Residential HVAC |
|  | Residential and Non Residential Water Heating |
|  | Residential Appliances |
|  | Defaults |

The remainder of this report is organized as follows:

Chapter 2 – Ranked List of Energy Efficiency Measures by Technology Group

Chapter 3 – Non Residential Lighting Impact Update

Chapter 4 – Residential Lighting Impact Update

Chapter 5 – Residential Hot Water Heating Impact Update

Chapter 6– Non Residential Lighting NTGR Update

Chapter 7– Residential Lighting NTGR Update

Chapter 8 – Large Industrial and Commercial Measures NTGR Update

Chapter 9 – Commercial HVAC systems and Building envelope NTGR Update

Chapter 10 – Commercial Refrigeration NTGR Update

Chapter 11 – Residential HVAC NTGR Update

Chapter 12 – Residential and Non Residential Hot Water System NTGR Update

Chapter 13 – Residential Appliances NTGR Update

Chapter 14 – Overall Summary NTGR Tables

Chapter 15 – Recommended Default NTGR Update

# Ranked List of Energy Efficiency Measures by Technology Group

The DEER team placed a higher priority on updating parameter estimates in technology areas which contain a higher relative share of the expected total savings for future programs. Table 2-1 shows the estimated electricity savings share for each technology group as estimated by the DEER team in mid 2010 compares to the more current share of portfolio savings based on the most recent utility tracking data from the second quarter of 2011.  The share estimates in the middle column were based on forecasts of likely savings at the measure level based on projected savings from 2010-2012 portfolio at the statewide level that were performed in mid 2010. The share estimates in the final column come from the latest quarter of utility program tracking data.

.

Table 2-1:  Comparisons of the Technology Group Share of Electricity Savings Over Time

| Technology Group | Share of 2010-2012 Portfolio Electric Savings (2010 Estimate) | Share of Portfolio Electricity Savings Based on Q2 2011 Tracking Data |
| --- | --- | --- |
| Non Res Lighting | 17.8% | 28.2% |
| Commercial HVAC & Bldg Envelope | 13.5% | 6.3% |
| Res and Non Res Water Heating | 1.1% | 0.3% |
| Commercial Plug Loads | 1.3% | 0.5% |
| Commercial Refrigeration | 8.4% | 5.9% |
| Large Commercial and Industrial | 23.2% | 9.3% |
| Residential Lighting | 12.5% | 30.1% |
| Residential HVAC & Bldg Envelope | 0.5% | 0.5% |
| Residential Appliances | 15.1% | 6.5% |
| Commercial Food Service | 1.0% | 0.2% |
| Whole House or Bldg/ Other Codes (Title 20/24) | 5.5% | 11.6% |
| Totals * | 98.90% | 99.4% |
| * Totals don't equal 100% due to impact of undefined and miscellaneous measures |  |  |

These technology group share estimates confirm that a focus on updating lighting parameter estimates will continue to be important given they represent between 30 and 50% of the expected energy savings in these two forecasts of future savings.  Comparison of the last two columns in the table highlights a shift in more recent program activity towards a greater share of electricity savings being reported for measures in the residential and non residential lighting technology group and lower shares of electricity savings for Commercial HVAC, Large Commercial and Industrial End Uses, and Residential Appliances.

The Energy Efficiency Measure (EEM) Categories under consideration for this update in 2011 for each technology group are summarized in Table 2-2 through Table 2-10.  The Technology Groups undertook the EEM prioritization analysis in late 2010 to identify which measure categories and associated parameters should be updated by the fall of 2011 or deferred to subsequent long term DEER updates.   The tables indicate whether each measure group is within one of the existing DEER datasets, either 2005 v2.01 or 2008 v2.05, or whether the measure is new for the Non-Residential Lighting area.  The prioritization was based upon rough estimates of the expected savings in the 2010-2012 utility portfolios, the relative need to improve the existing DEER parameters, and the available data to execute the updates, i.e., the 2006 - 2008 EM&V studies and other pertinent secondary sources. The EEM Measure categories listed in these tables appear in ranked order as determined by the DEER team members.

The rankings were derived after consideration of both the available EM&V data to affect an update, and the feasibility of completing the analysis within the study’s allocated timeframe and budget.  EEMs ranked “A” are the highest priority updates, “B” are medium priority, and “C” are considered the lowest priority updates.  The DEER Team Update only includes updates for measures in the “A” categories because they represent the highest priority. Measures placed in Category “B” or “C” will be included in the next formal DEER update.  Comments were provided by program administrators during the process on these lists and measures that should be included in the next update.

Table 2-2: Ranked Non-Residential Lighting EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | CFL, integral | A | 2008 v2.05 |
| 2 | Linear Fluorescent, de-lamping | A | 2005 v2.01 |
| 3 | Linear Fluorescent, lamp + ballast | A | 2008 v2.05 |
| 4 | Linear Fluorescent, fixture | A | 2008 v2.05 |
| 5 | HID, Mercury Vapor/HPS/LPS | B | 2008 v2.05 |
| 6 | HID, T5 HO | B | 2008 v2.05 |
| 7 | HID, Metal Halide | B | 2008 v2.05 |
| 8 | HID, LED | B | New |
| 9 | CFL, fixture | C | New |
| 10 | Side Daylighting Controls | C | 2005 v2.01 |
| 11 | Top Daylighting Controls | C | 2005 v2.01 |
| 12 | Exit, LED | C | 2008 v2.05 |
| 13 | Incandescent Lamp | C | N/A |

Table 2-3: Ranked Commercial HVAC and Building Envelope EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | HVAC Quality Maintenance: Refrigerant Charge Adjustment | A | 2008 v2.05 |
| 2 | HVAC Quality Maintenance: Airflow Adjustment | A | New |
| 3 | HVAC Quality Maintenance: Condenser Coil Cleaning | A | 2005 v2.01 |
| 4 | HVAC Quality Maintenance: Evaporator Coil Cleaning | A | New |
| 5 | HVAC Quality Maintenance: RCx Packages | A | New |
| 6 | Package Terminal Air Conditioner | A | 2005 v2.01 |
| 7 | Water-Cooled Centrifugal Chiller | A | 2008 v2.05 |
| 8 | Water-Cooled Screw Chiller | A | 2008 v2.05 |
| 9 | Water-Cooled Scroll Chiller | A | New |
| 10 | Water-Cooled Reciprocating Chiller | A | 2008 v2.05 |
| 11 | Water-Cooled Centrifugal Chiller with Variable Speed Drive | A | 2008 v2.05 |
| 12 | Water-Cooled Screw Chiller with Variable Speed Drive | A | New |
| 13 | Water-Cooled “Frictionless” Chiller | A | New |
| 14 | Water-Cooled “Frictionless” Chiller with Variable Speed Drive | A | New |
| 15 | Air-Cooled Screw Chiller | A | 2008 v2.05 |
| 16 | Air-Cooled Scroll Chiller | A | New |
| 17 | Air-Cooled Reciprocating Chiller | A | 2008 v2.05 |
| 18 | Air-Cooled Screw Chiller with Variable Speed Drive | A | New |
| 19 | Air-Cooled “Frictionless” Chiller | A | New |
| 20 | Air-Cooled “Frictionless” Chiller with Variable Speed Drive | A | New |
| 21 | Central Cooling Plant Reset Controls | A | 2005 v2.01 |
| 22 | Central Heating Plant Reset Controls | A | 2005 v2.01 |
| 23 | Air-Cooled Package Heat Pump | B | 2005 v2.01 |
| 24 | Water Source Heat Pump | B | 2005 v2.01 |
| 25 | Air-Cooled Split System Air Conditioner | B | 2008 v2.05 |
| 26 | Air Cooled Split System Heat Pump | B | 2005 v2.01 |
| 27 | Water-Cooled Package Air Conditioner | B | New |
| 28 | Water-Cooled “Frictionless” Compressor | B | New |
| 29 | Air-Cooled “Frictionless” Compressor | B | New |
| 30 | HVAC Quality Maintenance: Duct Sealing | B | 2008 v2.05 |
| 31 | Chilled Water Variable Frequency Drive Pump | B | 2005 v2.01 |
| 32 | Hot Water Variable Frequency Drive Pump | B | 2005 v2.01 |
| 33 | Water Loop Heat Pump Variable Frequency Drive Pump | B | New |
| 34 | Water Loop Heat Pump Variable Flow Loop | B | 2005 v2.01 |
| 35 | Variable Air Volume Distribution | B | 2005 v2.01 |
| 36 | Air-Cooled Package Air Conditioner | B | 2008 v2.05 |
| 37 | Steam Traps in HVAC Applications | B | New |
| 38 | Furnace | B | 2008 v2.05 |
| 39 | Package Terminal Heat Pump | B | 2005 v2.01 |
| 40 | Hot Water Boiler | B | 2005 v2.01 |
| 41 | Steam Boiler | B | 2005 v2.01 |
| 42 | Central Cooling Plant Water-side Economizer/Pre-cooler | B | 2005 v2.01 |
| 43 | Room Air Conditioner | C | New |
| 44 | Hot/Dry Climate Air-Cooled Package Air Conditioner | C | New |
| 45 | Hot/Dry Climate Air-Cooled Split System Air Conditioner | C | New |
| 46 | Room Heat Pump | C | New |
| 47 | Chilled Water Circulation Pump Motor | C | 2005 v2.01 |
| 48 | Hot Water Circulation Pump Motor | C | 2005 v2.01 |
| 49 | Air Distribution Variable Frequency Drive Fan | C | 2005 v2.01 |
| 50 | Air-side Heat Recovery | C | 2005 v2.01 |
| 51 | Evaporative-Cooled Package Air Conditioner | C | 2005 v2.01 |
| 52 | Evaporative-Cooled Split System Air Conditioner | C | New |
| 53 | Chilled Water Circulation Pipe Insulation | C | New |
| 54 | Hot Water Circulation Pipe Insulation | C | New |
| 55 | Steam Circulation Pipe Insulation | C | New |
| 56 | Condenser Water Circulation Pump Motor | C | 2005 v2.01 |
| 57 | Water Loop Heat Pump Circulation Pump Motor | C | 2005 v2.01 |
| 58 | Air Distribution Fan Motor | C | 2005 v2.01 |
| 59 | Central Cooling Plant Cooling Tower Variable Frequency Drive Fan | C | 2005 v2.01 |
| 60 | Central Cooling Plant Cooling Tower Fan Motor | C | 2005 v2.01 |
| 61 | Evaporative Coolers | C | 2005 v2.01 |

Table 2-4:  Residential and Commercial Hot Water Heating EEM Categories Parameter Updates

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Residential Aerator | A | 2005 v2.01 |
| 2 | Residential Showerheads | A | 2005 v2.01 |
| 3 | Residential Pool Heaters | A | New |
| 4 | Commercial Gas Instantaneous Water Heater | A | 2005 v2.01 |
| 5 | Commercial Electric Instantaneous Water Heater | A | 2005 v2.01 |
| 6 | Residential Multifamily Boiler controller | A | New |
| 7 | Residential Gas Storage Water Heater | B | 2008 v2.05 |
| 8 | Residential Electric Storage Water Heater | B | 2008 v2.05 |
| 9 | Residential Gas Instantaneous Water Heater | B | 2008 v2.05 |
| 10 | Residential Electric Instantaneous Water Heater | B | 2008 v2.05 |
| 11 | Residential Pipe Insulation w/Gas Water Heat | B | 2005 v2.01 |
| 12 | Residential Pipe Insulation w/Electric Water Heat | B | 2005 v2.01 |
| 13 | Commercial Gas Storage Water Heater | B | 2008 v2.05 |
| 14 | Commercial Electric Storage Water Heater | B | 2008 v2.05 |
| 15 | Commercial Gas Water Heating Boiler | B | 2005 v2.01 |
| 16 | Commercial Electric Water Heating Boiler | B | New |
| 17 | Residential Gas Water Heating Boiler | B | New |
| 18 | Residential Electric Water Heating Boiler | B | New |
| 19 | Automatic Closing Thermostatic Shower Valve | B | New |
| 20 | Commercial Pipe Insulation w/Gas Water Heat | C | New |
| 21 | Commercial Pipe Insulation w/Electric Water Heat | C | New |
| 22 | Residential Tank Insulation w/Gas Water Heat | C | 2005 v2.01 |
| 23 | Residential Tank Insulation w/Electric Water Heat | C | 2005 v2.012 |
| 24 | Commercial Tank Insulation w/Gas Water Heat | C | 2005 v2.012 |
| 25 | Commercial Tank Insulation w/Electric Water Heat | C | 2005 v2.012 |

Table 2-5:  Ranked Commercial Information Technology Equipment and Plug Loads EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Computer Power Management Network Software | B | New |
| 2 | Computer CPU | B | New |
| 3 | Personal Computer Monitors/Displays | B | New |
| 4 | Televisions | B | New |
| 5 | Photocopiers | C | 2005 v2.01 |
| 6 | Multi Function Peripherals | C | New |
| 7 | Printers | C | New |
| 8 | Occupancy Sensor Controls | C | 2005 v2.01 |
| 9 | Efficient Vending Machine | C | New |
| 10 | Vending Machine Time Clock | C | 2005 v2.01 |
| 11 | Server Farms | C | New |
| 12 | Laptop Computer | C | New |

Table 2-6: Ranked Commercial Refrigeration EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Vertical Reach-in Refrigerated Display Case Fan Motor | A | 2005 v2.01 |
| 2 | Horizontal Reach-in Refrigerated Display Case Fan Motor | A | 2005 v2.01 |
| 3 | Vertical Open Refrigerated Display Case Lighting | A | New |
| 4 | Horizontal Reach-in Refrigerated Display Case Lighting | A | New |
| 5 | Horizontal Open Refrigerated Display Case Lighting | A | New |
| 6 | Vertical Reach-in Refrigerated Display Case Lighting | A | New |
| 7 | Vertical Reach-in Refrigerated Display Case | B | 2005 v2.01 |
| 8 | Horizontal Reach-in Refrigerated Display Case | B | New |
| 9 | Vertical Reach-in Refrigerated Display Case Door Gasket | B | New |
| 10 | Horizontal Reach-in Refrigerated Display Case Door Gasket | B | New |
| 11 | Walk-in Cooler Door Gasket | B | New |
| 12 | Walk-in Freezer Door Gasket | B | New |
| 13 | Vertical Open Refrigerated Display Case Night Cover | B | 2005 v2.01 |
| 14 | Vertical Open Refrigerated Display Case Fan Motor | B | 2005 v2.01 |
| 15 | Horizontal Open Refrigerated Display Case Fan Motor | B | 2005 v2.01 |
| 16 | Strip Curtains | B | New |
| 17 | Walk-in Cooler Door Closer | C | 2005 v2.01 |
| 18 | Walk-in Freezer Door Closer | C | 2005 v2.01 |
| 19 | Occupancy Sensor Controls for Refrigerated Display Case Lighting | C | New |
| 20 | Anti-Sweat Heater Controller | C | New |

Table 2-7: Ranked Industrial and Agricultural EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Steam Traps | A | New |
| 2 | Greenhouse Envelope | A | 2005 v2.01 |
| 3 | Premium Efficiency Motors | A | 2005 v2.01 |
| 4 | Storage Tank Insulation | A | New |
| 5 | Micro Irrigation Systems | B | 2005 v2.01 |
| 6 | Pump Off Controllers | B | New |
| 7 | Process Boilers | B | New |
| 8 | Variable Frequency Drive (VFD) on System Pumps | B | 2005 v2.01 |
| 9 | Pipe Insulation | B | New |
| 10 | Air Compressor | C | New |
| 11 | Reverse Osmosis | C | New |
| 12 | Injection Molding | C | New |
| 13 | Waste Heat Recovery | C | New |
| 14 | Direct Digital Controls | C | New |
| 15 | Heat Recovery Devices | C | New |
| 16 | Condensing Economizers | C | New |
| 17 | Process Boiler Controls | C | New |

Table 2-8: Ranked Residential Lighting EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Indoor Lighting – LED Exit Sign | A | New (to Residential) |
| 2 | Indoor Lighting – CFL, Integral | A | 2008 v2.05 |
| 3 | Exterior Lighting – CFL, Integral | A | 2008 v2.05 |
| 4 | Indoor Lighting – CFL, Specialty | A | 2008 v2.05; some New |
| 5 | Exterior Lighting – CFL, Specialty | A | 2008 v2.05; some New |
| 6 | Exterior Lighting – LED Seasonal Lights | B | New |
| 7 | Indoor Lighting – LED Task | B | New |
| 8 | Indoor Lighting – LED Ambient | B | New |
| 9 | Indoor Lighting – CFL, Fixture | B | 2008 v2.05 |
| 10 | Exterior Lighting – CFL, Fixture | B | 2008 v2.05 |
| 11 | Indoor Lighting – LED Night Light | C | New |
| 12 | Indoor Lighting -- Linear Fluorescent, Lamp + Ballast | C | New (to Res) |
| 13 | Indoor Lighting -- Linear Fluorescent, Fixture | C | New (to Res) |
| 14 | Indoor Lighting -- Linear Fluorescent, De-Lamping | C | New (to Res) |

Table 2-9: Ranked Residential HVAC and Building Envelope EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Programmable Thermostats | A | 2005 v2.01 |
| 2 | Furnaces | A | 2008 v2.05 |
| 3 | Room Air Conditioners | A | New |
| 4 | Quality Maintenance: Duct Sealing | A | 2008 v2.05 |
| 5 | Quality Maintenance: Refrigerant Charge Adjustment | A | 2008 v2.05 |
| 6 | Quality Maintenance: Airflow Adjustment | A | New |
| 7 | Quality Maintenance: Condenser Coil Cleaning | A | New |
| 8 | Quality Maintenance: Evaporator Coil Cleaning | A | New |
| 9 | Quality Maintenance: RCx Packages | A | 2008 v2.05 |
| 10 | Roof Insulation | A | 2005 v2.01 |
| 11 | Wall Insulation | A | 2005 v2.01 |
| 12 | Integral Vertical Fenestration (Windows) | A | 2005 v2.01 |
| 13 | Building Envelope Air Leakage | A | 2005 v2.01 |
| 14 | Water Cooled Packaged Air Conditioners | A | New |
| 15 | Floor Insulation | A | 2005 v2.01 |
| 16 | Air Cooled Packaged Air Conditioners | A | New |
| 17 | Air Cooled Split System Air Conditioners | A | 2008 v2.05 |
| 18 | Evaporative Cooling Systems | B | 2005 v2.01 |
| 19 | Room Heat Pumps | B | New |
| 20 | Evaporatively Cooled Packaged Air Conditioners | B | New |
| 21 | Evaporatively Cooled Split System Air Conditioners | B | 2008 v2.05 |
| 22 | Whole House Fans | B | 2005 v2.01 |
| 23 | Water Source Heat Pumps | B | New |
| 24 | Air Cooled Packaged Heat Pumps | B | New |
| 25 | Split System Heat Pumps | B | 2005 v2.01 |
| 26 | Hot Water Boiler (space heating) | B | New |
| 27 | Central Heating Plant Reset Controls | B | New |
| 28 | Hot Water Circulation VFD Pump | B | New |
| 29 | Hot Water Circulation Pump Motor | B | New |
| 30 | Hot Water Heating Pipe Insulation | B | 2005 v2.01 |
| 31 | Low Solar Heat Gain Coefficient (SHGC) Coatings (window films) | B | 2005 v2.01 |
| 32 | Cool Roofs | C | New |
| 33 | Packaged Terminal Air Conditioner | C | New |
| 34 | Packaged Terminal Heat Pump | C | New |
| 35 | Hot/Dry Climate Air Cooled Packaged Air Conditioner | C | New |
| 36 | Hot/Dry Climate Air Cooled Split System Air Conditioner | C | New |
| 37 | Air-Side Heat Recovery | C | New |
| 38 | Room Air Conditioner Recycling (EEM moved to TG 9, Residential Appliances) | - | New |
| 39 | Room Heat Pump Recycling (EEM moved to TG 9, Residential Appliances) | - | New |

Table 2-10 Ranked Residential Appliance and Plug Load EEM Categories

|  | Energy Efficiency Measure Category | Rank | DEER |
| --- | --- | --- | --- |
| 1 | Appliances: Clothes Washer | A | 2005 v2.01 |
| 2 | Appliances: Refrigerator & Freezer Recycling | A | 2008 v2.05 |
| 3 | Consumer Electronics: Televisions | A | New |
| 4 | Home Office: Power Supplies / Power Strips | A | New |
| 5 | Home Office: Computer CPU | B | New |
| 6 | Home Office: Computer Display | B | New |
| 7 | Home Office: Miscellaneous (Printers/Routers/MFD/etc.) | B | New |
| 8 | Consumer Electronics: Miscellaneous (Set-top Boxes/DVD/Audio/etc.) | B | New |
| 9 | Appliances: Self-Contained Refrigerator | C | 2008 v2.05 |
| 10 | Appliances: Dishwasher | C | 2005 v2.01 |
| 11 | Appliances: Self-Contained Freezer | C | 2008 v2.05 |
| 12 | Room Air Conditioner Recycling (EEM moved from TG 8, Residential HVAC) | C | New |
| 13 | Room Heat Pump Recycling (EEM moved from TG 8, Residential HVAC) | C | New |

Assigned Commissioner’s Ruling and Scoping Memo Regarding 2013-2014 Bridge Portfolio and Post-Bridge Planning, Phase IV.  October 25, 2011

The 2005 DEER deleted these measures due to code requirements and saturation of old tank replacements.  2004-2005 DEER Update, Final Report, pages 2-20 and 3-20.

There are no A categories for this measure because the energy savings for this category was less than 1% of total electricity savings and there were no evaluations of these technologies in 2006-08.

“Specialty” includes CFLs other than those that are medium screw base twister-style CFLs between 9 and 30 Watts. This includes 3-Way, Dimmable, Reflector, Small Screw Base, Pin-based, Globe, Candelabra, and other non-standard CFL styles.

A check on the commercial availability of units that meet the hot/dry specification is necessary for both packaged and split systems air conditioners.