# Recommended Updates to Net-To-Gross Ratios for Commercial Refrigeration

## Introduction

In this analysis, the Net-to-Gross ratio (NTGR) analysis results from the 2006 – 2008 impact evaluation studies that reviewed commercial refrigeration measures are compared to the matched NTGR values for similar energy efficiency measures in the current DEER data base. The goal of this comparison is to decide if an update to the existing NTGR values is warranted for the 2011 update.  The 2011 updates could serve as part of the bases for programs implemented after January 1, 2013.  For each measure, the DEER team compares the strengths and weaknesses of the latest evaluation methods used to derive the NTGR results with the prior studies’ evaluation methods used for the 2008 DEER update.  Some of the factors that may be considered in recommending a revised NTGR value include:

- The relative merits of the  methods and sampling plans used to derive the NTGR estimates in the existing DEER database and in the 2006 – 2008 evaluation studies,

- The potential changes in either the program’s design or measure’s minimum qualification levels between the previous evaluation efforts and the 2006 - 2008 evaluation, and

- The changing market conditions and measure’s saturation levels in the specific market segments.  There are two metrics that could serve as indicators of market saturation:

  - The rate of change in the measure’s market share over time, i.e., the measure’s adoption rate – very high rates, along with dramatic changes in the rate, may be indicative that the measure has reached commercial maturity and market acceptance with the expectation that the NTGR value will decline over time in mature markets, and

  - The trend in the measure’s incremental cost over time – the NTGR is expected to decrease as the incremental cost declines.

After recommending specific NTGR values for the 2011 DEER update, the DEER team identified possible program and market factors that could change the recommended values over the next three years.  We then consider whether there is sufficient data and an acceptable analytical approach to forecast the NTGR trend over the next three years.  If both conditions are met satisfactorily and absent any significant changes in either program design or qualifying levels for the rebated measures, an NTGR forecast is prepared and presented for use in 2013 - 2014.  However, if the assessment concludes that neither adequate data nor a good understanding of how market changes may affect the NTGR values over time, then no adjustments to the recommended 2011 NTGR estimates are made to account for market changes between now and 2013.

## Comparison of Current DEER NTGR Values to the Results from the 2006 – 2008 EM&V Studies

Table 11-1 lists the Commercial Refrigeration measures that the DEER team selected for the 2011 DEER update based upon the readily available data from the 2006-2008 evaluation reports.  The table summarizes and compares the NTGR values form the 2008 DEER v2.05 database and the results from the final 2006 – 2008 impact evaluation reports.  The recent evaluations found much higher levels of free-ridership for both measures, and consequently much lower NTGR values compared to those values in the 2008 DEER.  The evaluation report offered no explanations and possible reasons for the large increase in free-ridership between the two sets of evaluations.  One potential reason may be the low ex ante baseline issues identified in the recent study, in particular for the strip curtains measure.

Table 11-1:  Overview of NTGR results by Measure and Delivery Mechanism

| Energy Efficiency Measure | Delivery Mechanism | Existing DEER NTGR Values (Source) | 06-08 NTGR Results (Source) |
| --- | --- | --- | --- |
| Door Gaskets | Downstream Prescriptive | 0.46 (Itron, December 2008) | 0.19 (ADM, February 2010) |
| Strip Curtains | Downstream Prescriptive | 0.76 (Itron, December 2008) | 0.40 (ADM, February 2010) |

## Comparison of Methods and Sample Sizes

Both the 2004-05 and 2006-08 studies used the Self-Report Approach to derive their NTGR results using interviews with participating customers.  The 2006-08 evaluation study used a modified version of the standard net‐to‐gross battery especially for the door gaskets and strip curtains measures to eliminate obvious free-riders: participants with maintenance contracts for door gasket replacements (measure already adopted), and those that have an internal replacement schedule that is less than the measure’s estimate effective useful life (EUL) of four years.  The latest evaluation achieved larger sample sizes relative to the 2004-05 evaluation, as summarized in Table 11-2.   More detailed information on the sources and methods used are provided in Appendix A-4.

Due to the larger sample sizes, along with the improved, modified battery of questions specifically designed for the door gaskets and strip curtain measures, the recent evaluation appears to provide more precise information than the prior bases of the 2008 DEER based on our assessment of the sampling and analyses methodologies used.  Hence, the DEER Update Technology Group 5 team recommends the 2006-08 NTGR estimates summarized in Table 11-3 for use in the 2011 update.

Table 11-2:  Methods and Sample Sizes for Commercial Refrigeration Measures

| Measure | Market Segment and Program Year | NTGR Method Used | Sample Sizes DEER 2006-2008 |  | Delivery Mechanism |
| --- | --- | --- | --- | --- | --- |
| Door gaskets | All Nonresidential 2006-08 | Self Report | 24 | 71 | Downstream Prescriptive |
| Strip curtains | All Nonresidential 2006-08 | Self Report | 47 | 101 | Downstream Prescriptive |

## Recommended 2011 DEER Updates to the NTGR Values

Table 11-3 summarizes the recommended NTGR values for the 2011 DEER database update.

Table 11-3:  Recommended NTGR by Measure and Delivery Method

| EEM | EEM Characteristics | Delivery Methods | Applicable Sector and Building Types | Utility Specific or Statewide | Recommended NTGR Value |
| --- | --- | --- | --- | --- | --- |
| Door Gaskets | Applied to either walk-in cooler or freezer doors, Refrigerated Reach-in Display Cases | Downstream Prescriptive, Direct Install | Non-Residential: Supermarkets, Convenience Stores, Restaurants | Statewide | 0.19 |
| Strip Curtains | Walk-in coolers or freezers | Downstream Prescriptive, Direct Install | Nonresidential: Supermarkets, Convenience Stores, Restaurants, Refrigerated Warehouses | Statewide | 0.40 |

## Factors Likely to Lead to Changes in the Next Three Years

The DEER team reviewed the two most recent NTGR evaluations and the overall literature to identify key factors that may lead to changes in the next three years for the commercial refrigeration measures and the programs that promote them.  The principal factors that were identified for the commercial refrigeration measures include:

- Changes in the measures qualifying for the programs and their respective rebate levels,

- Potential federal and state minimum performance requirements for walk-in coolers and freezers,

- Changes in Program design, especially changes to the measure baselines,  and

- Business trends in the Vendor/Contractor markets and their impacts on program volume.

## Assessment of Current Data and Tools Available to Prepare a Forecast of NTGR Values

There is insufficient data on the market share of door gaskets and strip curtains to make adjustments to the 2008 NTGRs for use in program years 2013 and beyond.

## Recommended NTGR Values for Future Programs

Given the lack of data on price and limited incremental costs, and the percentage of customers with either maintenance contracts or with internal maintenance schedules for the two refrigeration measures reviewed, the DEER team finds that it cannot produce reliable adjustments to account for potential market conditions in 2013 - 2014.  Thus, we recommend no adjustments be made to the NTGR values presented in Table 11-3.

2004/2005 Statewide Express Efficiency and Upstream HVAC Program Impact Evaluation, Itron, Inc., December 2008.

Commercial Facilities Contract Group 2006-2008 Direct Impact Evaluation Study, Volumes 1 and 3, ADM and Associates, February 2010.