# Residential Lighting Impact Input Updates for DEER

## Introduction

The Database for Energy Efficient Resources (DEER) team was tasked with providing updated impact inputs for residential lighting measures based on the significant share of savings represented by these programs and the availability of high quality lighting logger data. The two key inputs that are being included in this short-term update are the Hours-of-Use profiles and the wattage-reduction ratios that go into the impact savings calculations for residential lighting measures. KEMA used data collected during the 2006-2008 Upstream Lighting Program (ULP) evaluation to develop the new savings inputs for the DEER calculations. These new inputs have been produced in collaboration with the DEER team and are being used in the interactive effects models to produce measure specific impact savings estimates. Below we provide an overview of the methods used for developing the values being suggested for the short-term updates.

## Wattage Reduction Ratio Estimates

The DEER team produced an updated wattage reduction ratio (WRR) by using the residential lighting inventory collected in the 2006-2008 ULP evaluation. The primary assumption used in developing the new wattage reduction ratio is that CFLs are replacing non-CFLs for residential savings. KEMA and the DEER team identified five different scenarios for interior lighting based on lamp shape; these scenarios reflect different aggregations of lamp types and are shown in Table 3-1.

Table 3-1: Wattage Reduction Ratio Interior Scenarios Sample Sizes

|  | CFL Lamp Shape | CFL Sample Size | Non-CFL Sample Size |
| --- | --- | --- | --- |
| Scenario 1 | A-LINE | 162 | 10287 |
|  | GLOBE | 173 | 971 |
|  | ALL OTHER | 391 | 877 |
|  | REFLECTOR | 426 | 2754 |
|  | SPIRAL | 6070 | - |
| Scenario 2 | A-LINE/SPIRAL | 6232 | 10287 |
|  | GLOBE | 173 | 971 |
|  | ALL OTHER | 391 | 877 |
|  | REFLECTOR | 426 | 2754 |
| Scenario 3 | A-LINE/SPIRAL | 6232 | 10287 |
|  | ALL OTHER | 990 | 4602 |
| Scenario 4 | REFLECTOR | 426 | 2754 |
|  | ALL OTHER | 6796 | 12135 |
| Scenario 5 | All | 7205 | 14889 |

The different scenarios for interior lighting were evaluated on the underlying samples sizes for each breakdown and for how well the different lamp shapes could be compared to their non-CFL equivalents. Scenario 1 show the most disaggregated set of lamp types, with other scenarios showing various aggregation strategies.  The DEER team decided that since A-lamp and spiral both used A-lamp as the non-CFL equivalents that those two categories could be combined (Scenarios 2 and 3). The small sample sizes for Globe CFLs, along with a hard-to-define non-CFL comparison group, was the deciding factor in combining them into the All Other category. Scenarios 3 and 4 show two aggregation strategies that break out A-lamp and spiral lamps (Scenario 3) and reflector lamps (Scenario 4) from all other lamps.  Reflectors were considered unique in their application, and had large enough sample sizes for both CFLs and non-CFLs that they were assigned their own WRR. Ultimately the DEER team decided upon Scenario 4 for interior lighting analysis. Exterior lighting was assessed across all lamp shapes due to limited sample sizes in the inventory. Table 3-2 shows the final wattage reduction ratios calculated using the data set from the 2006-08 evaluation and compares it to the current values in DEER. The DEER team recommends use of the 2011 DEER WRR for estimating energy savings for the three classes of CFLs described in this table.

Table 3-2: Wattage Reduction Ratio Recommendations for Short Term Update

| Location | Lamp Shape | 2011 DEER WRR | 2008 DEER WRR |
| --- | --- | --- | --- |
| INTERIOR | REFLECTOR | 4.09 | 3.53 |
| INTERIOR | ALL OTHER | 3.47 | 3.53 |
| EXTERIOR | All | 4.07 | 3.53 |

## Hours-of-Use Profile Estimates

The DEER team developed projections of lighting profiles for years 2012 through 2014.  The lighting profiles were estimated using the CPUC 2006-2008 evaluation data and models as a starting point and then adjusted these estimates with projected increments in CFL saturations for the years of interest (2010-2014).  The DEER team estimated saturation increments, by updating the Installation Analysis, which was previously constructed for the 2006-2008 evaluation report (KEMA 2010b) and updated for 2009.  Finally the DEER team estimated hourly lighting profiles by several levels of aggregation, including IOU, lamp type, dwelling unit type, space type, and by variables indicating different parts of a year.

### Installation Analysis and Saturation Increment

Underlying the projections of lighting profiles are changes in saturation of CFLs for the years of interest.  To estimate the saturation increment, we extended the installation analysis that was conducted for the 2006-2008 evaluation report and later updated for 2009.  In particular, we updated two key input variables: the number of program bulbs shipped and the number of residential households in the IOUs’ service territory.  Extending the analysis forward through 2014 produced estimates of number of bulbs in sockets per home for each year (saturation).

We obtained information on the number of program bulbs shipped each year from IOU tracking data.  We gathered information on the number of bulbs shipped from 2004 through 2010, as shown in Table 3-3. (Note that figures for 2010 do not include deferred installations.)  To estimate projected shipments for 2011 through 2014 (shaded in grey), we used two scenarios:

Constant: Assume that 2011-2014 shipments will be the same as 2010.

Half: Assume that 2011 will have the same amount of shipments as 2010, but that shipments will be half of that number from 2012 through 2014.

Table 3-3: Program Bulbs Shipped by IOU (in millions)

|  |  |  | Shipments per Year (in Millions) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Actual |  |  |  |  |  |  | Projected |  |  |  |
| IOU | Scenario |  | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 |
| Overall | Constant |  | 10.0 | 10.0 | 14.8 | 38.4 | 43.6 | 11.8 | 28.6 | 28.6 | 28.6 | 28.6 | 28.6 |
|  | Half |  | 10.0 | 10.0 | 14.8 | 38.4 | 43.6 | 11.8 | 28.6 | 28.6 | 14.3 | 14.3 | 14.3 |
| PG&E | Constant |  | 4.1 | 4.1 | 7.6 | 19.2 | 26.2 | 6.0 | 11.2 | 11.2 | 11.2 | 11.2 | 11.2 |
|  | Half |  | 4.1 | 4.1 | 7.6 | 19.2 | 26.2 | 6.0 | 11.2 | 11.2 | 5.6 | 5.6 | 5.6 |
| SCE | Constant |  | 4.7 | 4.7 | 6.3 | 15.4 | 13.6 | 4.0 | 14.1 | 14.1 | 14.1 | 14.1 | 14.1 |
|  | Half |  | 4.7 | 4.7 | 6.3 | 15.4 | 13.6 | 4.0 | 14.1 | 14.1 | 7.1 | 7.1 | 7.1 |
| SDG&E | Constant |  | 0.8 | 0.8 | 1.0 | 3.8 | 2.8 | 1.7 | 3.2 | 3.2 | 3.2 | 3.2 | 3.2 |
|  | Half |  | 0.8 | 0.8 | 1.0 | 3.8 | 2.8 | 1.7 | 3.2 | 3.2 | 1.6 | 1.6 | 1.6 |

The second updated parameter is the number of residential households served by the IOUs.  We developed tables with actual and projected sales for the Retail Energy Outlook, Issue 9, Q2 2011 (KEMA 2011).  Historical data, from 2003 through 2009, comes from the EIA Annual Electric Power Industry Report (Form EIA-861, File 2).  The remaining years are projected.  We estimated the number of customer accounts using data from EIA Short Term Energy Outlook (STEO) for 2010 through 2012, and growth rates from the EIA Annual Energy Outlook for 2013 and 2014.  Table 3-4 shows the actual and projected number of residential households for each IOU, from 2004 through 2014, with projected years shaded in grey.

Table 3-4: Projected Number of Residential Households by IOU (in millions)

|  | Residential Households by Year |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Actual |  |  |  |  |  | Projected |  |  |  |  |
| IOU | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 |
| Overall | 9.4 | 9.5 | 9.7 | 9.8 | 9.9 | 9.8 | 9.5 | 9.7 | 9.9 | 9.7 | 9.6 |
| PG&E | 4.3 | 4.3 | 4.4 | 4.5 | 4.5 | 4.5 | 4.3 | 4.4 | 4.5 | 4.4 | 4.4 |
| SCE | 4.0 | 4.0 | 4.1 | 4.1 | 4.2 | 4.2 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
| SDG&E | 1.1 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 |

With these updated household numbers and with the assumption that previous relationships remain constant through the period of interest, we produced the estimated number of CFLs in use per home at the end of each year.

Finally, we calculated the increment in CFL saturation for each year, Δsat,y, as the ratio of the increment in number of CFLs in use per home in that year, Δcfl,y, to the average number of sockets in each household.  The CFL increment is the projected number of CFLs in use in year y minus the number of CFLs in 2008, the base year.  The number of sockets is the average number of sockets in the household, for each IOU.  Note that the subscript for each scenario is omitted; in practice each value of y is a combination of year and scenario.

Δsat,y=Δcfl,y#sockets= #CFLsy- #CFLs2008#sockets

The estimated values for CFL saturation increment are shown in Table 3-5.

Table 3-5: CFL Saturation Increment by IOU and IOU

|  |  |  | Scenario |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Constant |  |  | Half |  |  |
| IOU | Year | Average Number of Sockets | CFLs in Use per Home | Increment in CFLs (Base 2008) | CFL Saturation Increment (Base 2008) | CFLs in Use per Home | Increment in CFLs (Base 2008) | CFL Saturation Increment (Base 2008) |
| PG&E | 2008 | 54 | 11.8 | 0.0 | 0.0% | 11.8 | 0.0 | 0.0% |
|  | 2009 | 54 | 14.4 | 2.6 | 4.8% | 14.4 | 2.6 | 4.8% |
|  | 2010 | 54 | 17.3 | 5.4 | 10.1% | 17.3 | 5.4 | 10.1% |
|  | 2011 | 54 | 19.0 | 7.2 | 13.3% | 19.0 | 7.2 | 13.3% |
|  | 2012 | 54 | 20.5 | 8.7 | 16.1% | 20.1 | 8.3 | 15.4% |
|  | 2013 | 54 | 22.6 | 10.8 | 20.0% | 21.9 | 10.0 | 18.6% |
|  | 2014 | 54 | 24.2 | 12.4 | 22.9% | 23.1 | 11.3 | 20.9% |
| SCE | 2008 | 49 | 9.9 | 0.0 | 0.0% | 9.9 | 0.0 | 0.0% |
|  | 2009 | 49 | 10.8 | 0.9 | 1.9% | 10.8 | 0.9 | 1.9% |
|  | 2010 | 49 | 12.9 | 3.1 | 6.3% | 12.9 | 3.1 | 6.3% |
|  | 2011 | 49 | 14.3 | 4.5 | 9.1% | 14.3 | 4.5 | 9.1% |
|  | 2012 | 49 | 15.5 | 5.6 | 11.5% | 14.8 | 4.9 | 10.1% |
|  | 2013 | 49 | 17.1 | 7.2 | 14.7% | 15.7 | 5.8 | 11.8% |
|  | 2014 | 49 | 18.3 | 8.4 | 17.2% | 16.2 | 6.4 | 13.0% |
| SDG&E | 2008 | 53 | 7.4 | 0.0 | 0.0% | 7.4 | 0.0 | 0.0% |
|  | 2009 | 53 | 8.3 | 0.9 | 1.8% | 8.3 | 0.9 | 1.8% |
|  | 2010 | 53 | 9.8 | 2.4 | 4.6% | 9.8 | 2.4 | 4.6% |
|  | 2011 | 53 | 10.7 | 3.3 | 6.3% | 10.7 | 3.3 | 6.3% |
|  | 2012 | 53 | 11.5 | 4.1 | 7.7% | 11.1 | 3.7 | 7.0% |
|  | 2013 | 53 | 12.6 | 5.2 | 9.8% | 11.8 | 4.4 | 8.4% |
|  | 2014 | 53 | 13.4 | 6.0 | 11.4% | 12.3 | 4.9 | 9.3% |

## Projections of Lighting Profiles

Even though a few of the saturation estimates may appear to be too high, it is worth emphasizing that the ultimate goal of the analysis is to provide projections of lighting profiles, not projections of CFL saturation.  The estimated CFL saturation increments were used to adjust our projections of lighting hours of use estimates, but had a limited impact on the adjusted lighting profiles.

### Cross-Sectional Model and Projected Lighting Profiles

A complete explanation of the approach used for developing lighting profiles can be found in the 2006-2008 ULP evaluation report and in the additional analysis memorandum of 2010 (KEMA 2010a).  A brief review of the lighting models is presented followed by an explanation of how the profiles were adjusted.  In the interest of clarity, and where ambiguity does not arise, subscripts will be omitted.  This omission is noted in the text.

The 2006-2008 ULP evaluation work that KEMA developed lighting profiles by means of two models: a seasonal and a cross-sectional model.  The seasonal model is used to provide annualization of the analyzed logger data.  Because each logger recorded only part of a year, KEMA extrapolated to the rest of the year by regressing hourly percent on (originally, hours of use) as a function of a sine wave with peak at winter solstice (December 21) and trough at summer solstice (June 21).  The sine wave approximates hours of daylight or darkness in a day.  This sinusoid model is estimated for each logger, for each day type, and for each hour of the day.  To simplify our discussion, the choice of day type will be omitted from all equations.  Hourly percent of time on for each logger, day of year, and hour is estimated by:

pctidh=αih+βihsinθd+ εidh

where

| pctidh | = | Percent on for logger i, on day of the year d, at hour h |
| --- | --- | --- |
| αih | = | Intercept coefficient for logger i, hour h |
| βih | = | Sinusoid coefficient for logger i, hour h |
| θd | = | Angle for day of the year d, with θd=0 at spring and fall equinox, θd=π/2 on December 21, and θd=-π/2 on June 21 |
| εidh | = | Residual error. |

Equipped with the sinusoid model and proper sampling weights, we are capable of estimating weighted usage for any time of the year directly from the sample, a method that we calls direct expansion.  This method is well suited for generating results at highly aggregated levels, but it is limited by the number of sample points that are available for any segment of interest.

The second model that the DEER team developed for lighting profiles aimed to mitigate the sample size issue for fine cuts of the data.  The idea is to leverage the inventory data that was collected at the same time as the metering study.  The inventory data contains complete information about all space types and lamp types in the households of the study.  The logger sample is a subset of the inventory.  The cross-sectional model is an analysis of covariance (Ancova) model estimated using the coefficients from the seasonal model as a function of a number of customer characteristics, including demographics, distribution of space types, and CFL saturation.  Here we restrict our attention to CFL saturation, which is the coefficient used for our adjustment.  The complete model specification can be found in Table 82 of the 2006-2008 ULP report.

The coefficients from the sinusoid model, αih and βih, are modeled separately as a function of CFL saturation and other customer characteristics.  It is the coefficients of CFL saturation, λα, h and λβ, h, that are used to adjust the projections of lighting profiles.

αih=intα,h+λα,hsat+…+μα,ih
βih=intβ,h+λβ,hsat+…+μβ,ih

where

| αih, βih | = | Coefficients from the sinusoid model |
| --- | --- | --- |
| intα,h, intβ,h | = | Ancova intercepts for α and β |
| sat | = | CFL Saturation in the household |
| λα, h, λβ, h | = | CFL Saturation coefficients for α and β |
| μα,ih, μβ,ih | = | Residual errors. |

The coefficients of CFL saturation provide a measure of how much a unit change in CFL saturation will affect the sinusoid coefficients, which in turn determine the lighting profiles.  Since the DEER team has projections of CFL saturation increment for all years of interest, one can calculate adjustments for the sinusoid coefficients by multiplying the saturation coefficient and the saturation increment:

adjα,yh=λα,h×Δsat,y
adjβ,yh=λβ,h×Δsat,y

These adjustments can be applied to profiles developed using either the direct expansion or Ancova methods.  When applied to the direct expansion results, we first calculate adjusted coefficients:

αiyh'=αih+adjα,yh
βiyh'=βih+adjβ,yh

We then calculate projections of percent on for each logger and each year by:

pctiydh'=αih'+βih'sin⁡(θd)

The calculation is analogous for the Ancova method, but it is performed for each fixture group in the inventory instead of each logger in the sample.  With either approach we can create lighting profiles by taking weighted averages of the individual profiles at the levels of aggregation of interest.

For the short-term DEER update, we produced projections of lighting profiles using the Ancova model, not the direct expansion approach.  This is because the DEER update required profiles at particular levels of analysis for which only a limited sample size was available.  The team emphasizes that in aggregate, both approaches give nearly indistinguishable results; it is only for finer cuts that we recommend using the Ancova model because it takes advantage of information from the inventory.

Because the lighting profiles span thousands of lines, we provide them in a separate appendix.  To give an overview, we present projections of annual hours-of-use for CFLs in Table 3-6 and Table 3-7.  As expected, the projected hours-of-use decrease across the years, but at a relatively small rate.

Table 3-6: Projected CFL Hours-of-Use by IOU and Space Type for Scenario “Constant”

|  |  | Space Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Overall |  | Interior |  | Exterior |  |
| IOU | Year | Num Obs | HOU | Num Obs | HOU | Num Obs | HOU |
| PG&E | 2009 | 3,946 | 1.60 | 3,479 | 1.40 | 467 | 3.32 |
|  | 2010 | 3,946 | 1.58 | 3,479 | 1.38 | 467 | 3.31 |
|  | 2011 | 3,946 | 1.57 | 3,479 | 1.37 | 467 | 3.29 |
|  | 2012 | 3,946 | 1.56 | 3,479 | 1.36 | 467 | 3.28 |
|  | 2013 | 3,946 | 1.54 | 3,479 | 1.35 | 467 | 3.27 |
|  | 2014 | 3,946 | 1.53 | 3,479 | 1.34 | 467 | 3.26 |
| SCE | 2009 | 3,519 | 1.96 | 3,108 | 1.76 | 411 | 3.71 |
|  | 2010 | 3,519 | 1.95 | 3,108 | 1.74 | 411 | 3.70 |
|  | 2011 | 3,519 | 1.94 | 3,108 | 1.73 | 411 | 3.69 |
|  | 2012 | 3,519 | 1.93 | 3,108 | 1.72 | 411 | 3.68 |
|  | 2013 | 3,519 | 1.92 | 3,108 | 1.71 | 411 | 3.67 |
|  | 2014 | 3,519 | 1.91 | 3,108 | 1.70 | 411 | 3.66 |
| SDG&E | 2009 | 1,679 | 1.39 | 1,456 | 1.18 | 223 | 3.11 |
|  | 2010 | 1,679 | 1.38 | 1,456 | 1.17 | 223 | 3.10 |
|  | 2011 | 1,679 | 1.37 | 1,456 | 1.16 | 223 | 3.10 |
|  | 2012 | 1,679 | 1.37 | 1,456 | 1.16 | 223 | 3.09 |
|  | 2013 | 1,679 | 1.36 | 1,456 | 1.15 | 223 | 3.08 |
|  | 2014 | 1,679 | 1.35 | 1,456 | 1.14 | 223 | 3.08 |

Table 3-7: Projected CFL Hours-of-Use by IOU and Space Type for Scenario “Half”

|  |  | Space Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Overall |  | Interior |  | Exterior |  |
| IOU | Year | Num Obs | HOU | Num Obs | HOU | Num Obs | HOU |
| PG&E | 2009 | 3,946 | 1.60 | 3,479 | 1.40 | 467 | 3.32 |
|  | 2010 | 3,946 | 1.58 | 3,479 | 1.38 | 467 | 3.31 |
|  | 2011 | 3,946 | 1.57 | 3,479 | 1.37 | 467 | 3.29 |
|  | 2012 | 3,946 | 1.56 | 3,479 | 1.36 | 467 | 3.29 |
|  | 2013 | 3,946 | 1.55 | 3,479 | 1.35 | 467 | 3.28 |
|  | 2014 | 3,946 | 1.54 | 3,479 | 1.34 | 467 | 3.27 |
| SCE | 2009 | 3,519 | 1.96 | 3,108 | 1.76 | 411 | 3.71 |
|  | 2010 | 3,519 | 1.95 | 3,108 | 1.74 | 411 | 3.70 |
|  | 2011 | 3,519 | 1.94 | 3,108 | 1.73 | 411 | 3.69 |
|  | 2012 | 3,519 | 1.93 | 3,108 | 1.73 | 411 | 3.68 |
|  | 2013 | 3,519 | 1.93 | 3,108 | 1.72 | 411 | 3.68 |
|  | 2014 | 3,519 | 1.92 | 3,108 | 1.72 | 411 | 3.67 |
| SDG&E | 2009 | 1,679 | 1.39 | 1,456 | 1.18 | 223 | 3.11 |
|  | 2010 | 1,679 | 1.38 | 1,456 | 1.17 | 223 | 3.10 |
|  | 2011 | 1,679 | 1.37 | 1,456 | 1.16 | 223 | 3.10 |
|  | 2012 | 1,679 | 1.37 | 1,456 | 1.16 | 223 | 3.09 |
|  | 2013 | 1,679 | 1.37 | 1,456 | 1.15 | 223 | 3.09 |
|  | 2014 | 1,679 | 1.36 | 1,456 | 1.15 | 223 | 3.09 |

## Possible Shortcomings of the Study

This short-term update to DEER benefits from a number of advances that were not available in earlier studies, including an extensive metering sample and statistical methods that take into account multiple customer characteristics, in contrast to simple averages.  However, to create projections of lighting profiles the DEER team had to make a number of assumptions.  Although THE DEER TEAM believes these are realistic assumptions, they may turn out not to hold in practice.  We discuss the possible shortcomings to the study in this section.

The installation analysis was developed for the 2006-2008 evaluation study and updated for 2009.  It relies on assumptions about the flow of CFL bulbs in California.  In particular, it assumes that sales of non-program bulbs remain fixed since 2008 and at the same time it assumes that the program net-to-gross ratio is fixed at the 2008 estimated level, independent of program volume.  These assumptions may be problematic, especially for distant years, but they are used for lack of better information.

The sinusoid annualization has a reasonable foundation, but it has some limitations that have already been discussed in the context of the 2006-2008 evaluation report.

The Ancova model was originally developed for daily hours of use. The DEER team used the same principle when estimating hourly percent on.  Although this approach produces results that are statistically indistinguishable from the direct expansion for most hours of the day, there are exceptions.  For early evening hours of the summer season, such as 6pm and 7pm, the estimates from the Ancova model are lower than the estimates calculated using the direct expansion model.

Lighting profiles are projected by adjusting the amount of CFL saturation in households.  All else is assumed to remain constant.  Furthermore, the adjustment is uniform in the sense that the analysis team assumes all households will experience the same amount of increase in CFL saturation.  In reality, we expect different segments to have different changes in saturation.  In aggregate, this may not be an issue, but it may warrant more refinement for small cuts of the data.

A more complete analysis could investigate the likelihood of each socket having a CFL as a function of household CFL saturation, socket hours of use, and other factors, with the intent of determining the distribution of CFLs in sockets given an increment in saturation.  In other words, assuming that hours of use are determined by the socket application, we will be able to predict what mix of sockets become CFLs as saturation increases.

## Comparison to Previous DEER Results

Table 3-8 compare the hours of use derived based on this analysis with the hours of use developed in the previous version of DEER updated in 2009 for internal and exterior lighting fixtures.  Daily hours of interior lighting use at the statewide level have declined by 32 % from 2.18 hours per day to 1.48 hours per day.  Estimated daily hour of use for exterior lighting fixtures has actually increased by 10%, from 3.42 hours per day to 3.10 hours per day. The decrease in lighting usage was expected by the DEER team given the increase saturation of CFL in residential dwellings but the explanation for why exterior lighting hours should have increased is not yet clear. The DEER team plans to continue to investigate this issue and may provide an updated estimate at a later date.

Table 3-8: Estimated Daily Hours of Use for Interior and Exterior Residential Light Fixtures

## Conclusion

The extensive data collection efforts conducted by the KEMA team in the residential lighting area have allowed the DEER team to develop much more robust estimates of lighting usage.  Whether the observed trends toward lower daily hours of usage between 2008 and 2011 are likely to continue depends on a variety of factors including the characteristics and lighting usage habits of the late adopters, new competitors to CFLs stimulated by the new EISA standards, and perhaps even future economic conditions. Comments on the current analysis of adjusted hours of use the key market factors that should be monitored because they may lead to further changes in hours of use.

## References

KEMA. (2010b). Additional Analysis of 2006-2008 Impact Evaluation Results for DEER Update.

KEMA. (2010a). Final Evaluation Report: Upstream Lighting Program, Vol 1, CALMAC CPU0015.01.

KEMA. (2011). KEMA Retail Energy Outlook, Issue 9, Q2.