# Load Impact Updates for Non-Residential Lighting

## Introduction

This section presents the general analysis that the DEER team has conducted to update the load impact estimates for non-residential lighting technologies.  In this analysis the DEER team reviewed new information on lighting system characteristics and hours of use gathered as part of the 2006-08 evaluation of non-residential lighting programs.  After comparing the revised HOU estimates by building type from this data set with the existing HOU values in the DEER data base, the DEER team decided not to recommend any changes to the HOU values because of uncertainties in the aggregation of data at the site level to DEER building prototypes and other technical issues.  However, the DEER team did decide to recommend minor changes to the baseline system inputs used to estimate load impacts for non-residential lighting systems due to changes in building codes and some changes to remaining useful life estimates for the same technologies.  These proposed changes to the baseline values discussed in Section 4.5.2. Below we provide an overview of the data sources and methods used for developing the values derived from the 2006-2008 analysis of small commercial and local government programs to illustrate the rationale for not updating the HOU at this time.

### Overview of this Section

This section is laid out as follows:

- Overview

  - Includes the Ranked List of Energy Efficiency Measures and other High-Level Background from the Analysis Plan.

- Data Sources

  - Describes the Data Sources for the 2008 DEER and Proposed 2011 DEER inputs.

- Hours of Use, Annual Usage Shapes and Coincident Factors

  - Presents and Compares the 2008 DEER and Proposed 2011 DEER HOU and CFs.

- Delta Watts Calculations

  - Presents and Compares the 2008 DEER and Proposed 2011 DEER HOU and CFs for CFL and linear fluorescents.

- Interactive Effects

- Appendix A-2.1 – Methodology for Calculating HOU and CFs

## Background/Overview of the DEER TG1 Analysis Plan

Table 4-1 summarizes the EEM Categories included in this update.  The measure groups were selected based upon their historical and perceived future energy savings potential statewide.  The EEM Categories listed appear in ranked order as determined by the Technology Group 1 team members.  The rankings were derived after consideration of both the available EM&V data to affect an update, and the feasibility of completing the analysis within the study’s allocated timeframe and budget.  EEMs ranked “A” are the highest priority updates, “B” are medium priority, and “C” are considered the lowest priority updates.  The DEER Update Study may include additional “C” ranked measure parameters in the formal DEER long-term update in 2012 based on data availability from ongoing EM&V studies and stakeholder feedback.  The table below summarizes the intended update timeframe for each specific parameter: an “S” indicates the EEM parameter is included in this short-term update and an “L” indicates inclusion in the long-term update (June 2012).

Table 4-1:  Commercial Lighting EEM Categories Parameter Updates

| Energy Efficiency Measure Category | Rank | HOU and CF | Load Shapes | Delta Watts | Interactive Effects |
| --- | --- | --- | --- | --- | --- |
| CFL, integral | A | S | S | S | S |
| Linear Fluorescent, de-lamping | A | S | S | S | S |
| Linear Fluorescent, lamp + ballast | A | S | S | S | S |
| Linear Fluorescent, fixture | A | S | S | S | S |
| HID, Mercury Vapor/HPS/LPS | B |  | L |  | S |
| HID, T5 HO | B |  | L |  | S |
| HID, Metal Halide | B |  | L |  | S |
| HID, LED | B |  | L* |  | S |
| CFL, fixture | C |  |  |  | S |
| Side Daylighting Controls | C |  |  |  |  |
| Top Daylighting Controls | C |  |  |  |  |
| Exit, LED | C |  |  |  | S |
| Incandescent Lamp | C |  |  |  |  |

Note: EEMs flagged with “L” also entail long term coordination efforts with the 2010 – 2012 EM&V studies.

This document provides the updates needed for annual energy savings and peak demand reduction for linear fluorescents and CFLs.  There is no data at this time to update savings values for other types of lighting.  High bay HIDs and occupancy sensors are listed as a priority for the IOUs.

### Parameter Updates

The following gross impact parameters were considered for updating for both CFL and linear fluorescent measures:

- Hours of Use/Operation and Coincidence Factors

The hours of use (HOU) and coincidence factors (CFs) were analyzed and are presented by building type and/or activity area.

- Measure Load Shape

The Technology Group developed load shapes for CFLs and linear fluorescents using a combination of the 0608 Small Commercial data and the 0608 Local Government Partnership data.  Load shapes for other measures will be updated during the next round of updates.  (Load shapes are available in an MS Excel file as an Appendix A-2.2.)

- Delta Watts

Delta watt estimates were developed for this short-term DEER update for linear fluorescents and CFLs.  The pre- and post-wattages were collected from the 0608 CPUC data sources for predefined measures.

### Interactive Effects Updates

Interactive effects for CFLs, linear fluorescents and high bay lighting will be updated using building simulation models (DOE-2.2).  After the release of the 2008 DEER v2.05 updates, the J.J. Hirsch & Associates simulation modeling team continued to work on significant impact improvements, bug fixes, and corrections collected under the unreleased DEER v3.02 update.  For example, the California 2008 Title 24 requirements are reflected in the revisions along with corrections to the Large Office primary lighting schedules.  The impacts of the various HVAC control and sizing changes in the unreleased DEER v3.02 have a profound effect on the secondary natural gas impacts.  In almost all cases, the heating energy “take-back” associated with commercial lighting measures is likely to decrease dramatically.

## Data Sources

### Data Sources – 2008 DEER

This section provides a brief explanation of the data sources used to estimate lighting hours of use in 2008 including sample sizes.  The 2008 DEER relied on a combination of the best available lighting logger data for non-residential buildings at the time and professional judgment.  The known data sources for the 2008 DEER HOU by building type were developed using data from the following sources:

- 2003 Express Efficiency Evaluation (logger study)

- 2004-05 Express Efficiency Evaluation (logger study)

- DOE2 Model using occupancy shapes

The 2003 and 2004-05 Express Efficiency evaluations included lighting logger studies.  These loggers were installed for approximately two months under the 2003 Express Study and three to four weeks under the 2004-05 Express Study.  The combined sample sizes of these studies are shown below:

- CFLs – Total of 179 sites, 414 loggers

- Linear fluorescents – Total of 111 sites, 234 loggers

In addition to the logger data available from these two studies, data from previous evaluation studies and occupancy shapes found in DOE2 were used to develop the HOU used in the 2008 DEER. The process used to develop analyze this data is summarized below.

Lighting profiles were updated to consider 2002-2003 and 2004-2005 state-wide express efficiency lighting logger studies. While not an exact process, the DEER team made its best effort to “map” activity areas in each DEER nonresidential prototype to the available lighting profiles. The DEER team started with preliminary shapes developed in April 2008 from the logger data collected for the 2004-2005 Express Efficiency evaluation. For business type and usage area combinations where the sample was large enough (e.g. Office - Office with window), the DEER team developed weekday and weekend load shapes at that level. Within a business type, usage areas with N<6 were collapsed together under the "Other" usage area label (e.g. School - Other). These M&V lighting logger study data sets showed that CFL operating hours and resulting equivalent full load hours (EFLH) were quite different and often lower than linear fluorescent and other general lighting.

During the review period for the 2009-2011 DEER program planning cycle, several lighting profiles were revised in response to comments from utilities. In general, these revisions resulted in increased equivalent full load hours for several activity areas. The effort to revise lighting profiles is summarized in the Excel workbook “DEER08_LightingProfiles_08101403.xls”. The workbook provides comparison of 2005 lighting profiles, 2008 lighting profiles and reference profiles from lighting logger studies.

### Data Sources – Proposed 2011 DEER Update

This section provides a brief overview of the data sources used to produce the HOU and CF estimates that the DEER Team proposes be used for the 2011 DEER Update.  The sample sizes are also presented and compared to those used in the 2008 DEER.

The latest EM&V analysis for non-residential lighting was performed during the 2006-2008 program cycle.  The analysis presented below uses data collected by two studies:

- 2006-08 Small Commercial Contract Group Evaluation

  - The results of the evaluation were reported in the 2006-08 Small Commercial Contract Group (SCCG) Direct Impact report prepared for the California Public Utilities Commission.

  - The Study included installation of over 7,000 lighting loggers at over 1,200 non-residential buildings.  Loggers were installed on linear fluorescents (including high-bay) and CFLs.

  - The logger effort included an Upstream CFL component, as well as the normal downstream/verification component.  For the Upstream effort, the sample was drawn from the entire population of commercial businesses.  However, only those businesses that installed a purchased CFL were recruited for the on-site visit.

- 2006-08 Local Government Partnership Contract Group Evaluation

  - The results of the evaluation were reported in the 2006-08 Local Government Partnership Contract Group (LGP) Direct Impact report prepared for the California Public Utilities Commission.

  - This study included two distinct lighting logger studies:

    - Pre-Post Study – Nearly 2,000 lighting loggers were installed at over 100 non-residential participant buildings.  Loggers were installed both before and after the rebated linear fluorescents (including high-bay) were installed.  For this DEER Update, only the data from the post phase (919 loggers) were used in the analysis.  (Note: these results were presented in the 2006-08 Small Commercial report.)

    - University/Community College – Over 550 lighting loggers were installed at over 14 universities and community colleges.  Loggers were installed after the rebated linear fluorescents were installed.

### Data Sources – Comparison of Sample Sizes

Table 4-2 and Table 4-3 present the number of sites and number of loggers used in the analysis by building type for linear fluorescents and CFLs respectively.  As shown, the number of loggers used in the analysis of the 2006-08 lighting logger studies was significantly higher than the logger measurements used to inform the 2008 DEER HOU estimates.

Data collected at these sites were used to update the HOU, CF, load shape and wattage estimates for the 2011 DEER update.

Table 4-2:  Comparison of Sample Sizes – Linear Fluorescents

| DEER Bldg Type | DEER 2008 | Proposed DEER 2011 |  |
| --- | --- | --- | --- |
|  | # Loggers | # Sites | # Loggers |
| Assembly | 21 | 16 | 156 |
| Education - Community College |  | 7 | 261 |
| Education - University |  | 7 | 294 |
| Grocery |  | 15 | 74 |
| Health/Medical - Clinic |  | 40 | 332 |
| Office - Small | 37 | 119 | 771 |
| Other | 125 | 118 | 633 |
| Restaurant |  | 25 | 75 |
| Retail - Small | 34 | 196 | 948 |
| Warehouse |  | 52 | 344 |
| Total | 234 | 595 | 3,888 |

Table 4-3:  Comparison of Sample Sizes – CFLs

| DEER Bldg Type | DEER 2008 | Proposed DEER 2011 |  |
| --- | --- | --- | --- |
|  | # Loggers | # Sites | # Loggers |
| Assembly | 79 | 64 | 280 |
| Health/Medical - Clinic | 29 | 61 | 215 |
| Lodging | 87 | 110 | 880 |
| Office - Small | 32 | 123 | 287 |
| Other | 72 | 141 | 437 |
| Restaurant | 54 | 110 | 381 |
| Retail - Small | 61 | 184 | 375 |
| Total | 414 | 793 | 2,855 |

In sum, these tables demonstrate there are significantly more information on hours of usage available from the loggers utilized in the 2006-08 evaluation relative to the loggers used in the 2004-05 studies.  These loggers collected data at more sites for each building type and for longer time periods than the loggers used for the DEER 2008 update.  However the team had some concerns about the building type assignments of the specific buildings metered within the 2011 sample for the assembly, small office, and restaurant building types.  In addition there may be differences in the activity area weights used in the last DEER update and the weights used by the DEER analysis team.  These differences are explored in more detail in subsequent sections.

## Hours of Use, Annual Usage Shapes and Coincident Factors

This section presents the HOUs and CFs currently in the 2008 DEER for commercial lighting systems and compares these to the values that were initially suggested for updates.

### 2008 DEER HOUs and CFs

As mentioned above, the current DEER HOU and CFs were developed using a combination of logger data, professional judgment, and the DOE2 model.  The HOUs in the official DEER 2008 documentation available on-line are presented by building type and activity area.  These HOUs are shown in Table 4-4 through Table 4-6.  However, since rebated lighting measures are not tracked at the activity area level (space use) in the program tracking data, these values are needed by building type.  The 2008 DEER Update documentation does not include a table of HOUs and CFs by building type, however these are available in an Excel file on-line.  These HOUs and CFs by building type are presented in Table 4-7.  Table 4-7 also identifies the DEER building types that we propose to update in the 2011 DEER for CFLs and linear fluorescents.

Table 4-4:  DEER 2008 – from Documentation (1 of 3)

Table 4-5:  DEER 2008 – from Documentation (2 of 3)

Table 4-6:  DEER 2008 – from Documentation (3 of 3)

Table 4-7:  DEER 2008 – HOU and CF by Building Type

| DEER Bldg Type | Linear Fluorescents |  |  | CFL |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | HOU | CF | Updating | HOU | CF | Updating |
| Assembly | 2,605 | 0.53 | X | 2,302 | 0.41 | X |
| Education - Community College | 2,416 | 0.81 | X | 2,509 | 0.67 |  |
| Education - Primary School | 2,061 | 0.58 |  | 2,173 | 0.59 |  |
| Education - Relocatable Classroom | 2,445 | 0.70 |  | 2,454 | 0.70 |  |
| Education - Secondary School | 2,317 | 0.71 |  | 2,339 | 0.71 |  |
| Education - University | 2,279 | 0.72 | X | 2,281 | 0.67 |  |
| Grocery | 4,891 | 0.69 | X | 3,879 | 0.50 |  |
| Health/Medical - Hospital | 5,182 | 0.80 |  | 4,081 | 0.68 |  |
| Health/Medical - Nursing Home | 4,308 | 0.68 |  | 3,616 | 0.56 |  |
| Lodging - Hotel | 1,964 | 0.24 | X | 1,660 | 0.20 | X |
| Lodging - Motel | 1,588 | 0.17 | X | 1,366 | 0.15 | X |
| Manufacturing - Bio/Tech | 3,963 | 0.85 |  | 3,572 | 0.78 |  |
| Manufacturing - Light Industrial | 3,227 | 0.92 |  | 2,578 | 0.78 |  |
| Office - Large | 2,641 | 0.71 |  | 2,996 | 0.63 |  |
| Office - Small | 2,594 | 0.69 | X | 2,962 | 0.67 | X |
| Restaurant - Fast-Food | 4,847 | 0.81 | X | 4,847 | 0.81 | X |
| Restaurant - Sit-Down | 4,827 | 0.80 | X | 4,827 | 0.80 | X |
| Retail - 3-Story Large | 3,372 | 0.78 |  | 3,703 | 0.64 |  |
| Retail - Single-Story Large | 4,171 | 0.85 |  | 4,245 | 0.69 |  |
| Retail - Small | 3,378 | 0.88 | X | 4,013 | 0.70 | X |
| Storage - Conditioned | 3,434 | 0.70 | X | 2,780 | 0.58 |  |
| Storage - Unconditioned | 3,434 | 0.70 | X | 2,780 | 0.58 |  |
| Warehouse - Refrigerated | 4,792 | 0.55 |  | 4,776 | 0.55 |  |

Source: DEER 08_Lighting-EFLH-DemFactor-v3-kjm.xlsx from Kevin Madison.

As mentioned, although the 2008 DEER relied on more information about usage than just the logger data, the 2003 and 2004-05 Express Efficiency logger data were a primary input for developing HOU estimates.  Therefore, the HOU values estimated directly from the 2003 and 2004-05 Express Efficiency logger data are also presented in Table 4-8 and Table 4-9 for comparison purposes.

Table 4-8:  DEER 2008 Analysis of Express Logger Data – Linear Fluorescents

| Bldg Type | Schedule Group | # Loggers | HOU |
| --- | --- | --- | --- |
| Community Service | Office_NoWindow | 7 | 2,796 |
| Community Service | Office_Window | 6 | 3,920 |
| Community Service | Other | 8 | 2,256 |
| Misc Commercial | Common_Area | 12 | 2,337 |
| Misc Commercial | Office_NoWindow | 7 | 3,498 |
| Misc Commercial | Office_Window | 29 | 2,590 |
| Misc Commercial | Other | 28 | 3,403 |
| Office | Office_Window | 21 | 2,515 |
| Office | Other | 16 | 2,451 |
| Other | Common_Area | 13 | 3,750 |
| Other | Other | 36 | 3,282 |
| Retail | Common_Area | 15 | 3,335 |
| Retail | Office_NoWindow | 10 | 2,530 |
| Retail | Other | 9 | 2,667 |
| School | Classroom | 10 | 2,039 |
| School | Other | 7 | 2,911 |

Table 4-9:  DEER 2008 Analysis of Express Logger Data – CFLs

| Bldg Type | Schedule Group | # Loggers | HOU |
| --- | --- | --- | --- |
| Community Service | Common_Area | 52 | 2,029 |
| Community Service | Other | 27 | 1,762 |
| Health Care | MedExam_Room | 10 | 2,516 |
| Health Care | Other | 19 | 2,595 |
| Lodging | Guest_Room | 52 | 812 |
| Lodging | Other | 35 | 2,868 |
| Office | Common_Area | 14 | 4,013 |
| Office | Office_Window | 8 | 3,047 |
| Office | Other | 10 | 1,361 |
| Other | Common_Area | 31 | 2,894 |
| Other | Other | 41 | 2,639 |
| Restaurant | Common_Area | 13 | 4,868 |
| Restaurant | Other | 41 | 3,069 |
| Retail | Common_Area | 47 | 3,838 |
| Retail | Other | 14 | 2,486 |

### 2011 DEER HOUs and CFs

This section presents initially suggested updates to hours-of-use (HOU) and coincident factors (CF) for linear fluorescents and CFLs.  As described in the Data Sources section above, the suggested values presented below were based on the lighting logger data collected as part of the 2006-08 Small Commercial Contract Group Evaluation and the 2006-08 Local Government Partnership Contract Group Evaluation.

Below is a brief description of the methodology used to estimate HOU and CF values.  The detailed methodology can be found in Appendix A-2.1.  Further, additional information about the on-site survey activities can be found in the 2006-08 Small Commercial Contract Group Evaluation Report available on CALMAC.

#### Logger Data Collection and QC

Time-of-use (TOU) loggers were installed to collect lighting usage data on-site.  The TOU loggers monitor on/off events.  The TOU data went through stringent quality control procedures.  Each logger was reviewed in addition to running the data through code to highlight possible problem loggers.  As a result of the validation process, logger data were dispositioned as either good or unusable.

The loggers were installed over the period between September 2008 through October 2009 with the majority of loggers installed in the second and third quarters of 2009.  The loggers were installed at each site for an extended period in most cases.  More than 75% of the loggers recorded lighting usage for more than eight weeks.  The bulk of the remaining loggers had between six to eight weeks of monitored data.

#### Logger Data Analysis

The validated loggers were then used to calculate hourly shapes for each logger and then were weighted to develop hours-of-use and CF estimates.  The results of this re-analysis of the Small Commercial and LGP logger data differs from the results presented in the final reports for those two studies.  The primary differences include:

- The Small Commercial report presents HOU and CFs by measure group, program delivery mechanism, building type and activity area.  The values proposed below aggregate the results across program delivery mechanisms for the DEER 2011.  The primary reason for this is to increase the sample sizes.

- The Small Commercial logger analysis used site weights in addition to the lamp count weights to weight up appropriately to the population of the program participants.  However, the team did not feel this was appropriate since the weighting can cause some sites to have large weights due only to the fact that they were originally misclassified in the program tracking database.

#### Initially Suggested HOU and CFs

The results have been calculated in several different formats.  The team has prepared results at the following levels of aggregation:

- Most Aggregated Level:  HOU and CF values by measure group and building type.  Presented in Table 4-10 and Table 4-11 for linear fluorescents and CFLs respectively.

- More Detailed Level:  HOU and CF values by measure group, building type, and activity area.  These estimates are available on request.

- Most Detailed Level:  8-day hourly load shapes by measure group, building type, and activity area.   These estimates are available on request.

Table 4-10:  Potential Updates for HOU and CF – Linear Fluorescents

| DEER Bldg Type | # Sites | # Loggers | HOU | CF |
| --- | --- | --- | --- | --- |
| Assembly* | 16 | 156 | 1,213 | 0.29 |
| Education - Community College | 7 | 261 |  |  |
| Education - University | 7 | 294 |  |  |
| Grocery | 15 | 74 | 5,292 | 0.90 |
| Health/Medical - Clinic | 40 | 332 | 3,501 | 0.71 |
| Office - Small | 119 | 771 | 2,545 | 0.66 |
| Other | 118 | 633 | 2,773 | 0.56 |
| Restaurant | 25 | 75 | 3,682 | 0.69 |
| Retail - Small | 196 | 948 | 2,691 | 0.79 |
| Warehouse | 52 | 344 | 2,268 | 0.65 |
| Total | 595 | 3,333 |  |  |

*  	One of the current items being discussed is which specific detailed building from the 2006-08 data set should be included in Assembly.  Depending on the outcome of this discussion, the sample sizes could change slightly.

Table 4-11:  Potential Updates for HOU and CF – CFLs

| DEER Bldg Type | # Sites | # Loggers | HOU | CF |
| --- | --- | --- | --- | --- |
| Assembly* | 64 | 280 | 970 | 0.14 |
| Health/Medical - Clinic | 61 | 215 | 1,703 | 0.27 |
| Lodging | 110 | 880 | 1,177 | 0.12 |
| Office - Small | 123 | 287 | 2,285 | 0.48 |
| Other | 141 | 437 | 2,060 | 0.34 |
| Restaurant | 110 | 381 | 3,803 | 0.63 |
| Retail - Small | 184 | 375 | 2,818 | 0.63 |
| Total | 793 | 2,855 |  |  |

* 	One of the current items being discussed is which detailed building types should be included in Assembly.  Depending on the outcome of this discussion, the sample sizes could change slightly.

#### Assessment of the Robustness of the revised Hours of Use Estimates compared to Current HOU Estimates

As discussed above, the substantial increase in data available from the recent EM&V logger studies supports using these results to update the DEER HOU and CF values.  These reasons are re-summarized below.

- Sample Sizes are significantly higher in the SmCom/LGP logger studies than the combined 2003 and 04-05 Express studies (used as a basis for DEER 2008).

  - SmCom/LGP logger analysis used data from over 8,000 loggers.

  - Express 03/04-05 logger analysis used data from approximately 650 loggers.

- The loggers installed as part of SmCom were installed for longer periods of time than those installed during the Express Evaluations.

- The DEER 2008 HOU estimates were partially based on assumed occupancy schedules in addition to logger data.

- However the team also identified some concerns with the whole sale substitution of HOU values from the 2006-08 studies which are described below.

#### Possible Concerns with the Proposed Updates

As discussed above, the suggested updates to HOUs for most building types are lower than the 2008 DEER values.  The team has investigated several possibilities for the lower HOU and describes these possibilities below.  In some cases, data was available to research the issue.

- Possible Reason:  The Small Commercial participants do not represent the general non-residential population.  Are the sites logged during the 2006-08 EM&V Studies smaller than the population?  Do larger sites stay open longer and therefore have higher lighting HOU?

  - First, this question is more relevant if DEER is intended to be used for estimating savings for the “general population” as opposed to program participants.  If the latter, the question however is still important, but changes slightly to: Are the sites logged during the 2006-08 EM&V Studies smaller than probable future participants.

  - The Small Commercial participant buildings were compared to the 2005 CEUS buildings.  (Draft memo on this analysis is available.)

  - High-level results:

    - The Small Commercial sites are, on average, as large as the 2005 CEUS sites except for: Grocery*, Retail, and Warehouses.

    - The Small Commercial sites are, on average, open as long as the 2005 CEUS sites except for: Lodging (N/A) and Retail.

    - According to this preliminary analysis, the only building type that appears to require an HOU adjustment is retail.  However, according to a simplistic regression model, the adjustment would be less than 1%.

- Possible Reason:  Increased saturation of occupancy sensors has reduced hours of use from the period 1995-2005 to current time period.

  - This is not supported by the Small Commercial logger data.  The saturation of occupancy sensors is less than a couple percent of the sites logged.

- Possible Reason:  Previous estimates of HOU depended on engineering judgment and were not clearly estimated and then weighted up by different usage areas in buildings.

- Possible Reason:  The recession has reduced hours of use from the period 1995-2005 to current time period.

- Possible Reason: The building type designation assigned to select sites as part of the 2006-08 logger study may differ in some cases from the definition of the DEER prototypes.  The DEER team is continuing discussions about how to characterize these sites and whether they can be included in the analysis.  The result of this conversation may end up proposing additional prototypes or a change in the definition of current prototypes to be more inclusive of similar types of buildings.  The concern is that a large percentage of the 2006-08 participant population does not fit into one of the current DEER prototypes.

- Possible Reason: Activity area weightings are also different.  The DEER team is working on how to resolve these differences.

## Delta Watts Calculations

This section presents the proposed delta watt values for CFLs and linear fluorescents including a comparison to the 2008 DEER values.

### Delta Watts – CFLs

#### 2008 DEER Delta Watts – CFLs

The 2008 DEER estimated delta watts as a multiplier (or ratio reduction of 2.53) of the installed CFL wattage.  Therefore, a 10 watt CFL would have a delta watts of 25.3 (or assumed baseline wattage of 35.3, or 3.53 times the installed wattage).

#### 2011 DEER Delta Watts – CFLs

The proposed delta watts (or ratio reduction) values were calculated using data collected during the 2006-08 Small Commercial Contract Evaluation.  For the evaluation, the delta watts were calculated separately for Upstream CFLs and Downstream CFLs.  However, for the 2011 DEER update, the Tech Group proposes to use one set of ratio reductions.  This section describes the data collected and the methodology used in the analysis to calculate these estimates.

For CFLs, manufacturer and model information were collected onsite for post-retrofit measures to estimate the wattage of the lamps installed, rather than simply relying on program tracking data.  Average wattages were estimated by measure.  In addition, self-report data was collected on-site on the wattage of pre-existing equipment.  Pre-existing wattages were estimated by post-retrofit wattage (as pre-retrofit wattages were gathered by post-retrofit measure).  Because the on-site data was not all encompassing of every possible post-retrofit wattage, regression analysis was used to develop a relationship between the pre- and post-retrofit wattages.  These pre- and post-wattages were reviewed for this DEER update.  The resulting delta watts and ratio reductions are shown below in Table 4-12.

As shown in Table 4-12, the proposed update would result in the delta watts increasing for some wattage ranges of CFL and decreasing it for others.  The CFLs most likely to be included in future programs are the “advanced” CFLs which include:

- Twister CFLs over 30 watts.

  - The proposed delta watts for these CFLs would decrease the overall UES value by approximately 33%.

- Specialty CFLs (globes, reflectors, dimmable, etc.).

  - The team recommends that future research be conducted to better estimate the delta watts for specialty CFLs since that was not the focus of the Small Commercial Evaluation Study.

Table 4-12:  Delta Watts – CFLs

| Post-Wattage/ Ranges | Proposed 2011 Updates |  |  | 2008 DEER |  | Delta Watts % Change |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ratio | Pre-Wattage | Delta Watts | Pre-Wattage | Delta Watts |  |
| 3 W - 11 W | 5.05 | Various | Various | Various | Various | 60% |
| 12 | 4.75 | 57.0 | 45.0 | 30.4 | 30.4 | 48% |
| 13 | 4.46 | 58.0 | 45.0 | 32.9 | 32.9 | 37% |
| 14 | 4.22 | 59.0 | 45.0 | 35.4 | 35.4 | 27% |
| 15 | 4.00 | 59.9 | 44.9 | 38.0 | 38.0 | 18% |
| 16 | 3.80 | 60.8 | 44.8 | 40.5 | 40.5 | 11% |
| 17 | 3.62 | 61.6 | 44.6 | 43.0 | 43.0 | 4% |
| 18 | 3.46 | 62.4 | 44.4 | 45.5 | 45.5 | -3% |
| 19 | 3.32 | 63.1 | 44.1 | 48.1 | 48.1 | -8% |
| 20 | 3.19 | 63.8 | 43.8 | 50.6 | 50.6 | -13% |
| 21 | 3.07 | 64.4 | 43.4 | 53.1 | 53.1 | -18% |
| 22 | 2.96 | 65.0 | 43.0 | 55.7 | 55.7 | -23% |
| 23 | 2.85 | 65.6 | 42.6 | 58.2 | 58.2 | -27% |
| 24 | 2.81 | 67.4 | 43.4 | 60.7 | 60.7 | -29% |
| 25 | 2.77 | 69.1 | 44.1 | 63.3 | 63.3 | -30% |
| 26 | 2.73 | 70.9 | 44.9 | 65.8 | 65.8 | -32% |
| > 26 W | 2.70 | Various | Various | Various | Various | -33% |

### Delta Watts – Linear Fluorescents

Summary of Ex Ante Value Revisions for T12 Linear Fluorescent Baseline Fixtures

- Baseline Fixture Wattage Revised to Assume Minimum Energy Saving Magnetic Ballasts: Minimum efficiency requirements for magnetic ballasts were adopted as part of EPACT in 1990. Ballasts covered by these standards are often called “Energy Efficient” or “ES” magnetic ballasts. These minimum efficiency requirements were updated again in 2005. The new requirements essentially prohibited the inclusion of ES magnetic ballasts in any new fixtures. However, the standard did allow the shipment of ES magnetic ballasts for repair or replacement purposes. Most importantly, EPACT 1990 required ES magnetic ballasts for all nonresidential applications. For this reason, the DEER team believes that older (or standard or pre-EPACT) magnetic ballasts cannot reasonably be used as a basis for determining baseline fixture wattage. For this reason, baseline wattages of fixtures in the DEER lighting fixture table that include pre-EPACT magnetic ballasts have been revised to assume ES magnetic ballasts.  Table 4-13 provides a sample of these fixture power revisions for 4 and 8 foot 2-lamp fixtures.

Table 4-13:  Comparison of 2008 and 2005 Fixed input Power for Linear Fluorescents

- RUL of T12 Fixture Reduced: EPACT will prohibit the shipment of most 4 and 8 foot T12 lamps as of July 1, 2012. This means that, at the end of a T12 lamp’s useful life, it will have to be replaced with a T8 lamp. The DEER team has therefore revised the remaining useful life (RUL) to be based on lamp life rather than ballast life for measures with baseline fixtures that include T12 lamps. For these cases, the RUL will be determined from the following equation:

RUL = 20,000 hr lamp life / bldg EFLH / 3 – 1

20,000 hours is considered a conservative (or long) estimate for T12 lamp life. With this revised approach, as can be seen in the Table 4-14 below, the RUL for T12 fixtures changes from a range of 3.11 to 5.0 years to 0.27 to 3.3 years depending on the DEER building type.  The current RUL in the DEER data base is six years.

Table 4-14: Impact of Proposed Changes on T-12 RUL by Building Type

## Non Residential Lighting – Interactive Effects

The DEER interactive effects of non-residential lighting were updated in 2010.  The documentation of these changes can be found in 2006-2008 Energy Efficiency Evaluation Report: Summary of Development of Interactive Effects, May 21, 2010.

As explained above, while data from sources in addition to the Express Efficiency lighting logger studies are believed to have been used for the 2008 DEER, the sample sizes are only available for these two lighting logger studies.

See Footnote #1.

Note: At the time of this draft (10/28/11), the method of calculating the distribution of lighting across activity areas may not be how the final HOU and CF estimates were calculated.  Once the proposed values are finalized, this methodology section will likely need to be updated.  These updates would likely be minor.

The TOU loggers installed were dominated by two Dent Instruments logger types: the LIGHTINGloggerTM (TOUL-3G) and the CTloggerTM (TOUCT-3G).

The detailed methodology can be found in Appendix A-1.