# Impact Updates for Residential Hot Water Heating

## Introduction

Based on the findings of the Technology Group 3 team’s data review effort, the DEER team has revised the EEM impacts for faucet aerators and low-flow showerheads for the 2011 update.   For the residential water heater measure that uses simulation approach, the DEER team provides input parameters to the simulation team to update the impacts.  The team will revise the remainder of the residential and non-residential hot water heating measures identified in the Technology Group 3 analysis plan and data assessment in 2012 for the long-term update.

## Residential Faucet Aerators and Showerheads

Currently, the DEER deemed savings estimates for faucet aerators and low-flow showerheads use a non-weather sensitive engineering calculation method.  The estimates use a fixed percent savings, also known as the Energy Savings Fraction (ESF), applied to the residential water heating unit energy consumption (UEC).  Based on the data assessment, the DEER team proposed to revise the residential water heating UEC values as part of the short-term update in 2011.  The energy savings fractions (ESF), used in the simplified engineering estimates, will be further reviewed and revised as part of the long-term effort in 2012.

The new residential water heating UEC values come from the 2009 Residential Appliance Saturation Study (RASS).  The team compared the new UEC values to the values used in the 2004-2005 DEER updates.  The 2008 DEER update did not revise the savings impacts for the faucet aerator and low-flow showerhead measures; hence, the 2005 DEER values are the most current set of deemed values.  After comparing the UEC values, the DEER team planned to update the UEC values and propose updated EEM impacts.  Table 5-1 compares the proposed UEC values and EEM impacts for faucet aerators and low-flow showerheads with the 2005 DEER values.  A Peak Demand factor (kW/kWh-unit) of 0.22 was used in the 2005 DEER to calculate the peak demand impacts.  Even though the conversion factor seems to be on the higher side, the same factor is recommended for the short term.  For the long term, it is recommended that the non-weather sensitive peak demand reduction be aligned with the weather sensitive peak demands.

Table 5-1:  Comparison of 2005 DEER IOU Electricity Impacts and Proposed Impacts for Faucet Aerators and Low-Flow Showerheads

| Utility | Measure Name | Building Type | 2005 DEER UEC (kWh) | 2009 UEC (kWh) | ESF Electric % | From 2005 DEER |  | Proposed/ Calculated |  | % change in impacts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Electric Impact (kWh/Unit) | Peak Demand (Watts/Unit) | Elec. Impact (kWh/Unit) | Peak Demand (Watts/Unit) |  |
| PG&E | Faucet Aerators | SF | 2,301 | 3,311 | 3% | 99.9 | 22.0 | 99.3 | 21.9 | -1% |
| PG&E | Faucet Aerators | MF | 1,896 | 1,495 | 3% | 52.2 | 11.5 | 44.9 | 9.9 | -14% |
| SCE | Faucet Aerators | SF | 2,512 | 2,868 | 3% | 90.6 | 19.9 | 86.0 | 18.9 | -5% |
| SCE | Faucet Aerators | MF | 1,906 | 1,645 | 3% | 47.3 | 10.4 | 49.4 | 10.9 | 4% |
| SDGE | Faucet Aerators | SF | 2,340 | 3,157 | 3% | 83.2 | 18.3 | 94.7 | 20.8 | 14% |
| SDGE | Faucet Aerators | MF | 1,940 | 1,690 | 3% | 43.4 | 9.5 | 50.7 | 11.2 | 17% |
| PG&E | Low Flow Showerhead | SF | 2,301 | 3,311 | 4% | 133.3 | 29.3 | 132.4 | 29.1 | -1% |
| PG&E | Low Flow Showerhead | MF | 1,896 | 1,495 | 4% | 69.6 | 15.3 | 59.8 | 13.2 | -14% |
| SCE | Low Flow Showerhead | SF | 2,512 | 2,868 | 4% | 120.7 | 26.6 | 114.7 | 25.2 | -5% |
| SCE | Low Flow Showerhead | MF | 1,906 | 1,645 | 4% | 63.0 | 13.9 | 65.8 | 14.5 | 4% |
| SDG&E | Low Flow Showerhead | SF | 2,340 | 3,157 | 4% | 110.9 | 24.4 | 126.3 | 27.8 | 14% |
| SDG&E | Low Flow Showerhead | MF | 1,940 | 1,690 | 4% | 57.9 | 12.7 | 67.6 | 14.9 | 17% |

The last column in the table provides the percentage difference between the proposed impact and 2005 DEER impact values.  Since there is not a significant variation in proposed savings across the utilities, the DEER team recommends that statewide unit energy saving impacts be used for both of the measures.  Table 5-2 provides the recommended statewide electric unit energy impacts for faucet aerators and low-flow showerheads.  The statewide numbers are obtained by performing a straight average of electric impacts for all the utilities.

Table 5-2:  Recommended Statewide Average Electric Unit Energy Impacts for Faucet Aerators and Low-Flow Showerheads

| Measure Name | Building Type | 2009 UEC | ESF | Proposed Elec Impact (kWh/Unit) | Proposed Peak Impact (Watts/Unit) |
| --- | --- | --- | --- | --- | --- |
| Faucet Aerators | SF | 3,112 | 3% | 93.4 | 21 |
| Faucet Aerators | MF | 1,610 | 3% | 48.3 | 11 |
| Low Flow Showerhead | SF | 3,112 | 4% | 124.5 | 27 |
| Low Flow Showerhead | MF | 1,610 | 4% | 64.4 | 14 |

Table 5-3 compares the proposed natural gas impacts for the water heating measures.  It also compares the proposed UEC values with the UEC values used in the 2005 DEER database.

Table 5-3:  Comparison of Proposed IOU Natural Gas Impacts and 2005 DEER Impacts for Faucet Aerators and Low-Flow Showerheads

| Utility | Measure Name | Bldg. Type | 2005 DEER UEC (Therms) | 2009 UEC (Therms) | ESF Gas% | 2005 DEER Gas Impact (Therms/Unit) | Proposed Gas Impact (Therms/Unit) | % change in impacts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PG&E | Faucet Aerators | SF | 111 | 189 | 3% | 5.6 | 5.7 | 2% |
| PG&E | Faucet Aerators | MF | 104 | 183 | 3% | 11.5 | 5.5 | -52% |
| SCG | Faucet Aerators | SF | 115 | 196 | 3% | 6.7 | 5.9 | -12% |
| SCG | Faucet Aerators | MF | 104 | 192 | 3% | 10.4 | 5.8 | -44% |
| SDGE | Faucet Aerators | SF | 103 | 182 | 3% | 5.6 | 5.5 | -2% |
| SDGE | Faucet Aerators | MF | 97 | 161 | 3% | 5.0 | 4.8 | -4% |
| PG&E | Low Flow Showerhead | SF | 111 | 189 | 4% | 7.5 | 7.6 | 1% |
| PG&E | Low Flow Showerhead | MF | 104 | 183 | 4% | 6.7 | 7.3 | 9% |
| SCG | Low Flow Showerhead | SF | 115 | 196 | 4% | 9.0 | 7.8 | -13% |
| SCG | Low Flow Showerhead | MF | 104 | 192 | 4% | 8.1 | 7.7 | -5% |
| SDG&E | Low Flow Showerhead | SF | 103 | 182 | 4% | 7.4 | 7.3 | -1% |
| SDG&E | Low Flow Showerhead | MF | 97 | 161 | 4% | 6.7 | 6.4 | -4% |

The last column in the table provides the percentage difference between the proposed impact and 2005 DEER impact values.  Since there is not a significant variation in proposed savings across the utilities, the DEER team recommends that statewide unit energy saving numbers should be used for both of the measures.  Table 5-4 provides the recommended statewide electric unit energy impacts for faucet aerators and low-flow showerheads.  The statewide numbers are obtained by performing a straight average of electric impacts for all the utilities.

Table 5-4:  Recommended Statewide Average Natural Gas Unit Energy Impacts for Faucet Aerators and Low-Flow Showerheads

| Measure Name | Bldg. Type | 2009 UEC | ESF | Proposed Gas Impact (Therms/Unit |
| --- | --- | --- | --- | --- |
| Faucet Aerators | SF | 189 | 3% | 5.7 |
| Faucet Aerators | MF | 179 | 3% | 5.4 |
| Low Flow Showerhead | SF | 189 | 4% | 7.6 |
| Low Flow Showerhead | MF | 179 | 4% | 7.1 |

## Residential Instantaneous and Storage Water Heater Measures

The 2008 DEER model incorporated the domestic hot water system into the residential simulation prototypes to calculate EEM impacts.  As part of the short-term update, the input parameters used in the simulation will be reviewed and updated.  The input parameters that are used in the simulations are readily available from the 06-08 evaluation studies and will be compared with the existing values by the simulation team.

The input parameters that are used in the simulation are:

- Heat Input Ratio,

- Tank Losses, and

- Water Heater Part Load Performance Curve

The simulation team will use the Energy Factor (EF) to update the heat input ratio, where as the storage tank size and the water heater temperature setting information will be used to update the tank losses.  Energy factor, storage tank size, and water heater temperature setting information are available from the 06-08 evaluation studies.  The values from the evaluations will be compared with the input parameters in the simulation model.  Table 5-5 shows the number of sites within the utility program for which the energy factor, storage size and temperature setting information are available.

Table 5-5:  Sample Size by Utility Program

| Utility Program | Number of Sites |
| --- | --- |
| PGE2000 | 75 |
| SDGE3024 | 75 |

Table 5-6 through Table 5-8 provide details on the energy factor, storage size and the temperature settings gathered during the site visits.  The simulation team will compare the input parameters with the values that are currently being used by the DEER model and update them if there is a significant difference.

Table 5-6:  Energy Factor for Rebated Hot Water Heaters

| Energy Factor (EF) | PGE2000 (n=75) | SDGE3024 (n=75) |
| --- | --- | --- |
| Gas EF of 0.59 | 0 | 1 |
| Gas EF of 0.62 | 43 | 46 |
| Gas EF of 0.63 | 18 | 23 |
| Gas EF of 0.64 | 2 | 0 |
| NA | 12 | 5 |
| Gas EF – Average | 0.623 | 0.622 |

Table 5-7:  Average Storage Size for Rebated Hot Water Heaters

| Storage Size (Gallons) | PGE2000 (n=75) | SDGE3024 (n=75) |
| --- | --- | --- |
| 30 Gallon Tank | 3 | 7 |
| 40 Gallon Tank | 43 | 37 |
| 50 Gallon Tank | 25 | 31 |
| NA | 4 | 0 |
| Average Size | 43.1 | 43.2 |

Table 5-8:  Average Temperature Setting for Hot Water Heaters

| Utility | Average Temperature Setting |
| --- | --- |
| PGE2000 (n=75) | 128 |
| SDGE3024 (n=75) | 125 |

## Recommendations for the Short-Term Update

As mentioned earlier in the report, the DEER team recommends statewide impact values for the faucet aerators and low-flow showerhead measures.  Table 5-9 and Table 5-10 summarize the recommended impact updates for the faucet aerators and low-flow showerhead measures.

Table 5-9:  Recommended Statewide Electric Unit Energy Impacts for Faucet Aerators and Low-Flow Showerheads

| Measure Name | Bldg. Type | 2009 UEC | ESF | Proposed Elec. Impact (kWh/Unit) | Proposed Peak Impact (Watts/Unit) |
| --- | --- | --- | --- | --- | --- |
| Faucet Aerators | Single Family | 3,112 | 3% | 93.4 | 21 |
| Faucet Aerators | Multi Family | 1,610 | 3% | 48.3 | 11 |
| Low Flow Showerhead | Single Family | 3,112 | 4% | 124.5 | 27 |
| Low Flow Showerhead | Multi Family | 1,610 | 4% | 64.4 | 14 |

Table 5-10:  Recommended Statewide Natural Gas Unit Energy Impacts for Faucet Aerators and Low-Flow Showerheads

| Measure Name | Bldg. Type | 2009 UEC | ESF | Proposed Gas Impact (Therms/Unit |
| --- | --- | --- | --- | --- |
| Faucet Aerators | Single Family | 189 | 3% | 5.7 |
| Faucet Aerators | Multi Family | 179 | 3% | 5.4 |
| Low Flow Showerhead | Single Family | 189 | 4% | 7.6 |
| Low Flow Showerhead | Multi Family | 179 | 4% | 7.1 |

For the residential water heater measures, the DEER team provides recommended input parameters to the simulation team.  The simulation team will compare the input parameters with the values that are currently being used by the DEER model and update the impacts if there is a significant difference.

2009 Residential Appliance Saturation Study- http://websafe.kemainc.com/rass2009/

These are direct impacts on the energy use of the water heater and do not include any estimate of embedded energy savings from reduced water deliveries or pumping.

Peak demand factor was calculated using a multiplying the energy impacts with peak demand conversion factor of 0.22 in 2005 DEER.

Residential Retrofit – High Impact Measure Evaluation Report, Prepared for the California Public Utilities Commission Energy Division, February 2010

HIR (Heat Input Ratio) = 1/Recovery Efficiency. 2008 DEER developed a table that shows the corresponding recovery efficiency to energy factor.

Formula used for calculating Tank Losses is provided in the 2008 DEER Report Version 2008.2.05.

When the EF was not available from site visits, the Evaluation Team conducted a make/model look-up (e.g., the Website: http://www.nipsco.com/energyprograms/eh/waterheaters_prod_list.html)