# Recommended Updates to Net-to-Gross Ratios for Non-Residential Lighting Measures

## Introduction

In this analysis, the most recent NTGR results from the 2006-2008 EM&V studies are compared to the existing values in the 2008 DEER database to derive a recommended value for the measures in this technology group selected for the DEER 2011 update.  For each measure, the DEER Team compares the strengths and weaknesses of the latest method used to derive NTGR results and contrast this to the relative strength of the method used to estimate NTGR for the 2008 DEER update.  The factors used to make a final NTGR recommendation include:

- The relative merits of the methods and sampling plans used to derive NTGR estimates in  for existing DEER data base and  those methods used to produce new NTG results in 2006-08 evaluation studies.

- Potential changes in program design or measure minimum qualification levels between the previous evaluation and 2006-08 evaluation.

- Rate of change in the market share of more efficient building systems or measures over time.

- Any available evidence on trends in the incremental cost of specific measures and how these could lead to changes in qualifying rebate levels or estimated NTGR over time.

After making a recommendation for a specific NTGR value to use in 2011 update, the DEER Team identified the program and market factors are likely to lead to changes in NTG results over the next three years (the first year when these new NTGR values might be used). The DEER Team then assessed whether there was sufficient data and or analysis tools to accurately forecast the likely trend in NTGR values over the next three years for this specific measure and program design.  If there is sufficient data to make an accurate forecast absent any significant changes in program design and or qualifying levels for rebated measures, the DEER Team produced a forecast of NTGR for use in 2013-2014.  Finally, the DEER Team notes that if either the program design or qualifying efficiency level for measures is changed significantly between 2009 and 2013, the forecast of NTGR should not be used in 2013 because it was based on the assumption of no major changes in design or qualifying levels.

## Comparison of Current DEER NTGR to Results from Latest EM&V Analyses in 2006-2008

The latest EM&V analysis for non-residential lighting was performed by Itron during the 2006-2008 program cycle.  The results of the evaluation were reported in the 2006-2008 Small Commercial Contract Group (SCCG) Direct Impact report prepared for the California Public Utilities Commission.  The objective of this analysis was to develop net-to-gross ratios (NTGR) for all HIMs and non-HIMs. A self-report methodology was utilized.  This methodology was developed by the Standard Non-residential NTGR working group, which was comprised of Energy Division and its technical consultants and evaluators.  The methodology estimated four separate measurements of free ridership from different inquiry routes and then averaged the values to derive the final free ridership estimate at the measure level.

The SCCG evaluation calculated NTGRs for CFLs, linear fluorescents, high bay fluorescents, and lighting controls.  Due to the limited sample size for high bays and similarities in results for the high bay fluorescents and linear fluorescents, these two measures were combined for the DEER recommendations.

Table 7-1 compares NTGR estimates for the leading lighting measures in the current DEER database to the latest results from the 2006-2008 evaluations.

Table 7-1:  Overview of NTGR Results by Measure, Delivery Mechanism and Vintage

| Energy Efficiency Measure | Delivery Mechanism | Existing DEER NTGR Values (Source) | 06-08 NTGR Results (Source) |
| --- | --- | --- | --- |
| CFLs | Downstream Prescriptive | .81 (0405 Express Efficiency) | 0.53 (0608 Small Commercial Report) |
| CFLs | Direct Install | .85 (0405 SDG&E Small Business) | 0.80 (0608 Small Commercial Report) |
| Linear Fluorescents | Downstream Prescriptive | .78 (0405 Express Efficiency) | 0.70 (0608 Small Commercial Report) |
| Linear Fluorescents | Direct Install | .85 (0405 SDG&E Small Business) | 0.89 (0608 Small Commercial Report) |
| Lighting Controls | Downstream Prescriptive | .84 (0405 Express Evaluation) | 0.60 (0608 Small Commercial Report) |
| Lighting Controls | Direct Install | .85 (0405 SDG&E Small Business) | 0.89 (0608 Small Commercial Report) |

## Comparison of Methods and Sample Sizes Used to Derive NTGR Results

Both the 2004-2005 Express Efficiency evaluation (used to support current NTGR values in DEER) and the SCCG Report used the self-report method to estimate NTG for CFLs, linear fluorescents, and lighting controls.  The 2008 DEER and the latest evaluation study also used discrete choice analysis to estimate the NTG for linear fluorescents.  The SCCG evaluation included 2,680 participants statewide in the self-report NTG analysis.  This greatly exceeds the 862 participants included in the 2004-2005 Express Efficiency Evaluation and the 150 participants included in the Evaluation of the SDG&E 2004-2005 Small Business Energy Efficiency Program.

The recommended NTGR values from the most recent evaluation study are up to 28 percent lower than existing NTGR values because of three factors:

- Diffusion of new product information and success stories over time- e.g. more commercial customers become aware of efficient lighting systems and as a result are willing to buy them as standard business practice.

- More accurate evaluation methods and larger sample sizes.

- Increases in availability and declines in cost of more energy efficient lighting systems over time reinforce likelihood that more efficient lighting systems will become standard business practice for a higher fraction of customers (who will be identified as free rider in NTG analysis.)

Overall, the DEER team finds that the most recent evaluation conducted in 2008 contains better, more precise information than the existing DEER based on a review of the methodologies, sample sizes and applicability. Table 7-2 below shows the methods and sample sizes of the NTG values from the SCCG Report.

Table 7-2:  Methods and Sample Sizes for Non-Residential Lighting

| Measure | Market Segment and Program Year | NTGR Method Used | Sample Size | Delivery Mechanism |
| --- | --- | --- | --- | --- |
| CFLs | Non-Residential 2006-2008 | Self Report | 115 | Downstream Prescriptive |
| CFLs | Non-Residential 2006-2008 | Self Report | 444 | Direct Install |
| Linear Fluorescents | Non-Residential 2006-2008 | Self Report/Discrete Choice | 521 | Downstream Prescriptive |
| Linear Fluorescents | Non-Residential 2006-2008 | Self Report/Discrete Choice | 1,479 | Direct Install |
| Lighting Controls | Non-Residential 2006-2008 | Self Report | 80 | Downstream Prescriptive |
| Lighting Controls | Non-Residential 2006-2008 | Self Report | 41 | Direct Install |

## Recommended NTGR by Measure and Applicable Market Segments or Building Types

Table 7-3 shows the recommended NTGRs to be used to update the DEER database in 2011.  It is recommended that the NTGRs are updated by measure and delivery mechanism for use statewide.

Table 7-3:  Recommended NTGR by Measure and Delivery Method

| EEM | EEM Characteristics | Delivery Mechanism | Applicable Sector and Building Types | Utility Specific or Statewide NTGR | Recommended NTGR (kWh) | Recommended NTGR (kW) |
| --- | --- | --- | --- | --- | --- | --- |
| CFLs | All | Downstream Prescriptive | Non- Residential | Statewide | 0.53 | 0.57 |
| CFLs | All | Direct Install | Non- Residential | Statewide | 0.80 | 0.80 |
| Linear Fluorescents | All | Downstream Prescriptive | Non- Residential | Statewide | 0.70 | 0.70 |
| Linear Fluorescents | All | Direct Install | Non- Residential | Statewide | 0.89 | 0.89 |
| Lighting Controls | All | Downstream Prescriptive | Non- Residential | Statewide | 0.60 | 0.59 |
| Lighting Controls | All | Direct Install | Non- Residential | Statewide | 0.89 | 0.74 |

## Factors Likely to Lead to Changes in NTGR Over Next Three Years

The DEER team reviewed the two most recent NTGR evaluations and the overall literature to identify key factors likely to lead to changes in NTGR over time for non-residential lighting measure programs. The principal factors were:

- Changes in the code will affect what measures are rebated in the future.

- Changes in rebate levels.

- Changes in incremental costs.

- Changes in Program Design or Sales Channel (Example – Target certain business sectors).

## Assessment of Current Data and Tools Available for Adjusting NTGR Over Time

The most likely impact on NTGRs in the future will be the upcoming changes to the codes for non-residential lighting.  As the currently rebated measures become standard, these NTGRs will no longer be representative of the new lighting technologies that replace them.  At this time, there is not sufficient data available to estimate a change in NTGR based on changes in code, rebate levels, or incremental costs.  The DEER team recognizes that future programs designs are likely to target different business sectors than past programs have targeted.  As a result, the DEER team recommends that NTGRs by measure, building type, and delivery mechanism are available for the program planners to customize a net to gross ratio by weighting the results by expected building types.

## Recommended NTGRs by Building Type to Assist Predicting NTGs for Future Programs

The DEER team analyzed the sample sizes by measure, building type, and delivery mechanism to determine which combinations have enough sample points to warrant a DEER value.  All combinations that did not have enough sample points were combined into the “ALL OTHER” category.  Table 7-4 shows the recommendations for CFLs.  The only building type with a downstream prescriptive recommendation is lodging.  Lodging was a large participant in the 2006-2008 programs.  As shown in the table, the lodging NTGR is lower than other business types.  Having this data would allow program planners designing a program to that did not include lodging to use a NTGR that was more consistent.  Similarly, a program planner that is designing a direct install program could weight these net to gross ratios to determine a more accurate ratio to represent the program.  Please note that these NTGRs by building type should only be used to create a customized NTGR with the approval of the Energy Division.  The DEER team does not recommend that these be the official DEER values.

Table 7-4:  CFL Sample Sizes and Net to Gross Ratios by Building Type and Delivery Mechanism

|  | Direct Install |  |  | Downstream Prescriptive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Sample Size | NTGR (kWh) | NTGR (kW) | Sample Size | NTGR (kWh) | NTGR (kW) |
| All Other | 96 | 0.80 | 0.75 | 74 | 0.61 | 0.63 |
| Assembly | 20 | 0.54 | 0.53 |  |  |  |
| Health/Medical - Nursing Home | 51 | 0.83 | 0.83 |  |  |  |
| Lodging | 32 | 0.77 | 0.77 | 41 | 0.40 | 0.40 |
| Office - Small | 59 | 0.89 | 0.89 |  |  |  |
| Retail - Single-Story Large | 22 | 0.89 | 0.88 |  |  |  |
| Retail - Small | 164 | 0.79 | 0.79 |  |  |  |

Table 7-5 shows the recommendations for net to gross ratios by building type and delivery mechanism for linear fluorescents.

Table 7-5:  Linear Fluorescent Sample Sizes and Net to Gross Ratios by Building Type and Delivery Mechanism

|  | Direct Install |  |  | Downstream Prescriptive |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Sample Size | NTGR (kWh) | NTGR (kW) | Sample Size | NTGR (kWh) | NTGR (kW) |
| All Other | 112 | 0.93 | 0.93 | 168 | 0.75 | 0.75 |
| Assembly | 55 | 0.89 | 0.89 |  |  |  |
| Grocery | 43 | 0.87 | 0.87 |  |  |  |
| Health/Medical - Nursing Home | 63 | 0.85 | 0.85 |  |  |  |
| Manufacturing - Light Industrial | 125 | 0.91 | 0.91 | 127 | 0.69 | 0.70 |
| Office - Large | 37 | 0.93 | 0.93 | 58 | 0.70 | 0.70 |
| Office - Small | 330 | 0.89 | 0.89 | 35 | 0.58 | 0.58 |
| Restaurant - Fast-Food | 37 | 0.83 | 0.82 |  |  |  |
| Retail - Single-Story Large | 109 | 0.88 | 0.88 |  |  |  |
| Retail - Small | 521 | 0.88 | 0.88 | 44 | 0.74 | 0.74 |
| Storage - Unconditioned | 47 | 0.94 | 0.94 | 34 | 0.76 | 0.77 |
| Storage - Conditioned |  |  |  | 55 | 0.77 | 0.77 |