# Recommended Updates to Net-to-Gross Ratios for Residential Lighting (CFLs)

This report contains the results of an assessment of net-to-gross ratio (NTGR) estimates for CFLs distributed through upstream retail channels.  The goal of this assessment was to:

- Compare current DEER NTGR estimates to CFL NTGR estimates from the 2006-2008 impact evaluation;

- Compare methods and sample sizes used to derive CFL NTGR estimates (current DEER v. 2006-2008 impact evaluation);

- Review CFL NTGR estimates estimated by channel and product type (basic v. specialty, or “advanced”), assess whether changes in market conditions are likely to change these values over time and assess the pros and cons of adopting NTGR estimates at the sales channel , product type or technology level

- Recommend a final NTGR estimate for the 2011 update in the near term

- Identify factors likely to lead to changes in CFL NTGR estimates over next three years; and

- Assess current data and tools available for adjusting CFL NTGR estimates over time.

We begin with a brief summary of the NTGR results from the 2006-2008 impact evaluation followed by a comparison of these results to the current DEER values.  We then describe some relevant changes to the Upstream Lighting Program from 2006-2008 to 2010 through Q1 2011 and assess their possible effects on NTGR.    After performing some sensitivity analysis on the channel level NTGR estimates, we present our final recommendation for a technology level NTGR of 0.54 for upstream CFL programs in the residential sector.   The final two sections describe factors that will lead to changes in CFL NTGR estimates beyond the current program cycle (2013-2014) and our recommended approach to collecting the needed data to adjust CFL NTGR estimates over time.

## NTGR Results from 2006-2008 Impact Evaluation

In the 2006-2008 impact evaluation, NTGR estimates were developed for all CFLs using multiple methods which produced a range of results, as shown in Table 8-1.  As part of the evaluation effort, the validity of each method/estimate was considered and assessed, at the channel level where available.  The relative strengths and weaknesses of each were presented and discussed in the 2006-2008 impact evaluation report.

The final recommended CFL NTGR estimates for the 2006-2008 Upstream Lighting Program are also presented in Table 8-1 by sales channel.  These estimates reflect the evaluators’ best judgment based on a preponderance of the evidence found using different methods of estimating the net impact of the program in each sales channel.

Table 8-1: CFL NTGR Estimates by Method and Final Recommended CFL NTGR Estimates from 2006-2008 Upstream Lighting Program Impact Evaluation

| Channel | % of CFL Shipments by Channel | CFL NTGR Estimates by Evaluation Method |  |  |  |  |  | Final Recommended CFL NTGR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Self-Report |  | Econometric Models |  |  | Total Sales (Market-Based) Approach |  |
|  |  | Supplier | Consumer Stated Preference | Conjoint Model | Revealed Preference Purchase Model | Stated Preference Purchaser Elasticity Model |  |  |
| Discount | 16% | 1.00 | 0.18 | n/a | n/a | 0.52 | n/a | 0.90 |
| Drug | 9% | 0.73 | 0.36 |  | 0.33 | 0.31 |  | 0.32 |
| Grocery - chain | 15% | 0.81 | 0.14 |  | 0.33 | 0.29 |  | 0.33 0.90 |
| Grocery - small | 21% |  | 0.11 |  | n/a | 0.51 |  |  |
| Hardware | 5% | 0.60 | 0.06 |  | 0.20 | 0.50 |  | 0.35 |
| Home Improvement | 8% | 0.46 | 0.20 |  | 0.20 | 0.52 |  | 0.36 |
| Lighting and Electronics | 1% | 0.83 | n/a |  | n/a | n/a |  | 0.36 |
| Mass Merchandise | 5% | 0.37 | 0.10 |  | 0.33 | 0.48 |  | 0.41 |
| Membership Club | 19% | 0.63 | 0.12 |  | 0.33 | 0.32 |  | 0.33 |
| All IOUs |  | 0.74 | 0.15 | 0.06 | n/a | 0.42 | 0.23 (low) – 0.65 (high) | 0.54 |
| PG&E |  | 0.71 | 0.18 | 0.06 |  | 0.40 | n/a | 0.49 |
| SCE |  | 0.80 | 0.14 | 0.06 |  | 0.44 |  | 0.64 |
| SDG&E |  | 0.71 | 0.30 | 0.08 |  | 0.41 |  | 0.48 |

In general, the revealed preference results were favored over the other approaches mainly because these were the only methods that used data derived from actual observations of participating retail store environments, average prices for all available, comparable products, and average prices for actual purchased products.  In addition, the revealed preference surveys provided the only source of data for actual observations of IOU-discounted CFL purchases as a percent of all CFL purchases.  Therefore, the recommended NTGR estimate for most channels was taken as the average of the two revealed preference model results.

There were two important channels for which the revealed preference models did not produce direct NTGR estimates for the 2006-2008 program – that is, discount stores and small, independent grocery stores.  For discount stores we observed mostly IOU-discounted CFLs on the retail shelves and, as a result, the revealed preference models could not be run.  In addition, manufacturer, retail buyer and retail store manager survey responses were in agreement that nearly 100% of the CFLs sold through this channel are discounted by the program (i.e., close to zero non-program sales), which was confirmed through the revealed preference surveys (i.e., we observed near 100% stocking of IOU-discounted CFLs and near 100% sales of IOU-discounted CFLs in this channel), and manufacturers and retail buyers were in agreement in terms of their independently-generated estimates of 100% program attribution.  Therefore, for this important channel (it accounted for 16% of all CFLs rebated through the program during 2006-2008), we recommended a NTGR estimate of 0.90.

For similar reasons, we also recommended a 0.90 NTGR estimate for small, independent grocery stores.  Interviews with manufacturers and retailers attribute a high percentage of the sales through these channels to the program, and the revealed preference surveys indicated that 100% of all CFL purchases were IOU-discounted CFLs.  According to 2006-2008 program tracking records, nearly 20 million IOU-discounted CFLs were distributed by less than ten manufacturers to more than 700 small, independent grocery stores located in hard-to-reach segments throughout the state.  These suppliers indicated that had it not been for the program incentives, they would not have been able to sell CFL products through these stores in anywhere near the volume they experienced during 2006-2008.

The   NTGR estimates presented in Table 8-1 represented the most robust, well-constructed estimates available for attributing net impacts to the 2006-2008 Upstream Lighting Program.  The main reason for the difference between the IOUs had to do with variations in distributions by retail channel – i.e., during 2006-2008, SCE shipped a much greater portion of the rebated measures through channels for which the NTGR estimates were highest (e.g., discount stores, small grocery stores).

## Comparison of 2006-2008 Evaluation Results to DEER Values

The last DEER update resulted in recommended NTGR estimates of 0.60 for basic CFLs and 0.85 for specialty CFLs.  At that time, the recommended NTGR estimates for basic CFLs were projected to reflect conservative estimates for the 2009-2011 time period.  The key driver for these NTGR estimates was increased sales nationwide beginning in 2007 combined with decreasing CFL prices.

The overall NTGR estimate from the 2006-2008 impact evaluation was somewhat lower than the DEER estimate for basic CFLs (0.54 v. 060).  The 2006-2008 results were significantly lower than 0.60 for PG&E (0.49) and SDG&E (0.48), whereas for SCE the 2006-2008 results were somewhat higher (0.64).  As mentioned above, these differences are largely due to differences in shipments by channel, which was not taken into account in the last DEER update.

Separate NTGR estimates for specialty CFLs were not produced as part of the 2006-2008 impact evaluation. Therefore, a comparison to the DEER NTGR estimate of 0.85 for specialty CFLs cannot be made.

## Changes to Upstream Lighting Programs Over Time

As stated in the 2006-2008 Upstream Lighting Program Impact Evaluation final report, it is likely that the final recommended NTGR estimates may not represent the best estimates going forward since the market for energy efficient lighting will continue to change and the effects of ongoing IOU interventions, new standards, and changes in the broader California economic conditions will need to be considered in future analyses.

Through Q1 2011, the 2010-2012 Upstream Lighting Program has changed in two significant ways:

- The distribution of discounted lighting products by channel has shifted slightly toward those channels with higher expected attribution, and

- Specialty (or “advanced”) CFLs make up a more significant percentage of all CFLs discounted through the program.

### Changes to Channel Distribution

According to IOU program tracking data, over 150 million CFLs have been discounted through the upstream programs beginning in 2006 through first quarter 2011  Table 8-2  presents the distribution of CFLs discounted through various channels by year.

Table 8-2: Upstream Lighting Program CFL Shipments by Channel and Year

| All CFLs | 2006 | 2007 | 2008 | 2010* |
| --- | --- | --- | --- | --- |
| Discount | 13% | 15% | 19% | 19% |
| Drug | 13% | 8% | 8% | 4% |
| Grocery | 40% | 33% | 37% | 39% |
| Hardware | 4% | 5% | 6% | 8% |
| Home Improvement | 7% | 10% | 7% | 5% |
| Lighting and Electronics | 1% | 1% | 2% | 2% |
| Mass Merchandise | 4% | 5% | 6% | 4% |
| Membership Club | 19% | 22% | 17% | 20% |
| Other | 0% | 0% | 0% | 0% |
| All Channels | 100% | 100% | 100% | 100% |

*2010 refers to the period January 2010 through March 2011.

### Changes to Product Type Distribution

Table 8-3 shows the basic CFLs as a percent of all CFLs discounted through the Upstream Lighting Programs by channel and year.  In all channels, the percentages have decreased from 2008 to 2010 and, overall, basic CFLs have been scaled back to represent 70% of the all CFL shipments by Q1 2011.

Table 8-3: Basic CFL as a Percent of All CFL Shipments by Channel and Year

| All CFLs | 2006 | 2007 | 2008 | 2010* |
| --- | --- | --- | --- | --- |
| Discount | 95% | 97% | 96% | 68% |
| Drug | 99% | 70% | 83% | 80% |
| Grocery | 97% | 93% | 84% | 71% |
| Hardware | 95% | 98% | 94% | 78% |
| Home Improvement | 73% | 92% | 82% | 67% |
| Lighting and Electronics | 99% | 96% | 91% | 68% |
| Mass Merchandise | 79% | 88% | 87% | 84% |
| Membership Club | 70% | 87% | 69% | 61% |
| Other | 100% | 99% | 83% | 64% |
| All Channels | 90% | 90% | 84% | 70% |

*2010 refers to the period January 2010 through March 2011.

## Recommended NTGR Estimates

### Basic CFLs

Using the results from the 2006-2008 impact evaluation, the overall NTGR estimate for basic CFLs discounted in 2010 through Q1 2011 would be 0.57, reflecting a shift in CFL shipments toward channels with higher expected program attribution.  Estimates for each IOU are as follows: SCE, 0.59; SDG&E, 0.50 and PG&E, 0.55.  Table 8-4  presents the distribution of basic CFL shipments by channel for each IOU as well as overall.  Applying the 2006-2008 CFL NTGR estimates by channel to these IOU-specific shipment distributions produces the basic CFL NTGR estimates by IOU for the 2010 through Q1 2011 program period.

Table 8-4: Basic CFL Distributions by Channel and NTGR Estimates 
(2010 through Q2011)

| Channel | Percent of Basic CFL Shipments (2010-Q1 2011) |  |  |  | NTGR by Channel (2006-2008) |
| --- | --- | --- | --- | --- | --- |
|  | SCE | SDG&E | PG&E | All IOUs |  |
| Discount | 18.9% | 16.9% | 14.8% | 17.1% | 0.90 |
| Drug | 5.2% | 0.8% | 3.2% | 4.0% | 0.32 |
| Grocery | 45.7% | 19.5% | 30.9% | 37.3% | 0.65 |
| Hardware | 8.3% | 4.5% | 8.6% | 8.1% | 0.35 |
| Home Improvement | 3.3% | 1.2% | 6.2% | 4.3% | 0.36 |
| Ltg & Electronics | 1.4% | 1.2% | 1.5% | 1.4% | 0.36 |
| Mass Merchandise | 6.0% | 11.1% | 2.4% | 5.0% | 0.41 |
| Membership Club | 11.1% | 44.3% | 16.3% | 16.3% | 0.33 |
| Other | 0.0% | 0.5% | 0.9% | 0.4% | 0.54 |
| Missing | 0.0% | 0.0% | 15.2% | 6.1% | 0.54 |
| Basic CFL NTGR Estimate by IOU (2010-Q1 2011) | 0.59 | 0.50 | 0.55 | 0.57 |  |

Recent NTGR research completed in MA (using similar methods to what was used in CA for 2006-2008) produced a NTGR estimate of 0.43 for basic CFLs during 2010.  A recent study completed for ComEd (Illinois) using some of these similar methods determined that the NTGR estimate for a similar upstream CFL program implemented from mid-2009 through mid-2010 was 0.58.  Table 8-5 compares the NTGR results from the 2006-2008 CA study with these other studies from MA and IL.

Table 8-5: CFL NTGR Estimates: Comparison Across Recent Studies

|  | CA (2006-2008) |  | MA (2010) |  | ComEd (2009-2010) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Basic | Specialty | Basic | Specialty | Basic | Specialty |
| Final Recommended NTGR Estimate | 0.54 |  | 0.43 | 0.60 | 0.58 |  |
| Conjoint Model | 0.06 - 0.44 | 0.14 - 0.66 | n/a | 0.59 | n/a |  |
| Multistate Model | 0.23 - 0.63 |  | 0.45 |  | n/a |  |
| Revealed Preference Model | 0.20 - 0.33 | 0.60 | 0.37 | 0.31 | 0.21 |  |
| Stated Preference Model | 0.42 |  | n/a |  | n/a |  |
| Consumer Self-Report | 0.15 |  | 0.49 | 0.31 | 0.57 - 0.60 |  |
| Supplier Self-Report | 0.74 | 0.72 | 0.39 | 0.49 | 0.44 |  |
| Program Discounted CFLs (millions) | 105.1 | 20.5 | 1.4 | 1.2 | 7.4 | 0.8 |

#### MA Study

As mentioned above, the MA study used similar methods to determine NTGR estimates but it also included a Delphi panel to integrate the results.  The overall result of 0.43 for basic CFLs was generally in line with all of the other individual NTGR results from each specific method.

In MA, the 2010 programs differed from prior programs in that hard-to-reach households were targeted by expanding CFL distributions in dollar stores, discount stores, bargain stores, thrift stores, ethnic markets, and stores of various types located in low-income neighborhoods or market areas.  However, the MA study did not produce overall NTGR estimates by channel.  However, the MA study did produce NTGR estimates by channel using the supplier self report method.  This method produced a NTGR estimate of 0.72 for basic CFLs for the discount/HTR channel, which was much higher than any of the other channel-specific estimates produced using the supplier self report method.

Despite targeting HTR households through expanded distributions in HTR channels (including discount stores), only about 9% of all basic CFLs distributed through MA 2010 program were distributed through HTR/discount channels.  In contrast, the CA programs distributed about 17% of all basic CFLs through discount channels.

Given the relatively larger emphasis placed on basic CFLs distributed through discount channels in CA, and the consistently higher NTGR estimate derived from supplier self-reports for this channel, it is reasonable to assume that CA’s overall NTGR estimate for all basic CFLs would be higher than the overall NTGR estimate produced through the MA study.  Said another way, the MA 2010 program distributed proportionally more basic CFLs through channels expected to have lower NTGR estimates and as such it is reasonable to expect that the overall NTGR estimate for the MA 2010 program would be lower than an estimate derived for the CA programs.

#### ComEd Study

The ComEd program in 2009-2010 was similar to the CA IOU programs in 2006-2008 in that it was almost exclusively an upstream buy-down type of program in which specialty CFLs represented less than 10% of total CFLs discounted through the program.  However, the ComEd program differed somewhat significantly from CA in that 90% of the CFLs discounted through the program were distributed through the “big box,” “DIY” and membership club channels.  In comparison, during a similar time period, the CA programs distributed less than 30% of basic CFLs through these channels.

The overall NTGR estimate of 0.58 was determined by averaging the customer self report methods (0.57 and 0.60, average 0.58).  The evaluators rejected the revealed preference results because of instability in the model, as well as other data issues and constraints.  The evaluators also rejected the supplier self-report results due to concerns over potential bias from a small sample.

#### Final Recommended NTGR Estimate for Residential CFLs

The DEER team reviewed the range of NTGR estimates produced for each sales channel in KEMA’s 2006-08 evaluation of the upstream lighting programs. (See Table 8-1) The group identified two specific sales channels, discount stores and small independent grocery stores that had the greatest variance in NTGR estimates within the same channel.  For example NTGR estimates from different methods ranged from 1.0 to 0.18 in the discount sales channel. In this case KEMA decided on using a final NTGR estimate of 0.90 in its 200608 evaluation because of the compelling evidence presented by suppliers that CFL bulbs would not have been sold in these stores in the absence of the program.  Other analysts on the DEER team gave more weight to the NTGR estimates derived from customer interview in this channel because of the possibility that some small fraction of the customers who purchased CFL in the discount stores would have been free riders if they purchased CFLs in other sales channels    Because all of these approaches contain some uncertainty, the DEER team decided to bound the uncertainty by developing high, medium and low NTGR values for each of the key sales channels.

Table 8-6 shows the impact of selecting different NTGR results for these key sales channels on the overall Program level NTGR. The table shows that small changes in the judgment used to derive a final NTGR by channel led to a range of overall program NTGR from 0.57 to 0.45.

Table 8-6: Effect of Channel Level NTGR on Overall Program NTGR for Residential CFL’s

| Team | NTGR by Channel |  |  | Program NTGR |  |
| --- | --- | --- | --- | --- | --- |
|  | Discount | Small Grocery | Grocery-Chains |  | Weighted by 2010 shipments |
| High | 0.9 | 0.9 | 0.33 |  | 0.57 |
| Medium | 0.8 | 0.7 | 0.33 |  | 0.50 |
| Low | 0.6 | 0.6 | 0.33 |  | 0.45 |
| Final Recommended Value |  |  |  |  | 0.54 |

The DEER team recommended that the 2011 DEER update only include a program weighted NTGR because of concerns that these channel specific NTGR estimates are likely to change over time as prices and market conditions change. The DEER management team reviewed these results and the supporting evidence used to derive the program weighted NTGR estimates for these sales channels.    In the final analysis the DEER team decided there was too much uncertainty in the NTGR estimates developed for this table and in any event there were based on channel shipment weights that are likely to change in the future. The DEER management concluded that any overall NTGR value within this range could be supported by the available data and evaluation evidence

As a result of this sensitivity analysis and consideration of a variety of policy concerns, the DEER management team decided to recommend using the original program level NTGR of 0.54 produced in the 2006-08 study for the 2011 update, This represents a reduction of 10% relative to the current NTGR value of 0.60 contained in the current DEER data base but falls within the range of uncertainty identified by the DEER team.  The NTGR of 0.54 was estimated across all types of CFLs, both basic spiral CFL and specialty CFLs and as such would be applied to all upstream programs promoting all types of residential CFL’s

## Factors Leading to Changes in CFL NTGR Estimates Over Time

There are numerous factors leading to changes in CFL NTGR estimates over time.  These include the following listed below.

- Channel Shift.  Channel shift refers to sales through one retail channel that, in the absence of an energy-efficiency program, would have occurred through another channel.  To meet other program objectives – such as trying to increase CFLs sales in “hard-to-reach” market sectors – lighting rebate programs may promote CFL sales in certain retail channels (e.g., discount, ethnic grocery) over others.  However, some observers have wondered whether such policies might be simply shifting some CFL sales from traditional lighting channels such as large home improvement or mass merchandise to these presumably more targeted channels.

- Economic Recession/Recovery.  During 2009, many retailers saw CFL sales decline in comparison to sales during 2008, with some recovery in overall sales during 2010 and 2011.  Most suppliers attribute this decline and subsequent recovery to changes in the economy.  As the general economic climate changes, so too do consumer spending habits – and likely consumer willingness or ability to pay the incremental cost associated with CFLs and other energy-efficient lamps.

- Unpredictability of Lighting Market Reaction in Response to New Lighting Standards.  The U.S. Energy Independence and Security Act of 2007 (EISA) establishes increased energy efficiency standards for general service lamps and will effectively phase out traditional medium screw-base incandescent lamps over time, starting with the standard 100-Watt lamp on January 1, 2012.  Through California Assembly Bill 1109 (AB1109), the Lighting Efficiency & Toxics Reduction Act of 2007, the state of California has begun implementing the increased efficiency standards of EISA a year ahead of the rest of the country.  Neither set of standards prohibits retailers from selling through their existing stock of these products – so conceivably, the 100-Watt incandescent bulb may be available to Californians after January of 2011.  Because these changes have just begun, it is difficult to assess their effects on the lighting market in general and their possible resultant effects on NTGR.

- Increased Presence of Halogen Lamps.  Attributable in part to EISA and AB1109, there is an increased presence of halogen lamps on the market.  Each of country’s largest incandescent lamp manufacturers (GE, Philips, and Sylvania) released new EISA-compliant halogen products during 2010.  It is unclear how the presence of these products – which are markedly similar to standard incandescent lamps in appearance and price as compared to CFLs – will affect consumer choices in a post-EISA world.

- Commercialization of LED Lamps.  Again possibly attributable in part to EISA and AB1109, there is an increased presence of LED lamps in the marketplace.  These products – as well as the EISA-compliant halogen products described above – may dilute consumer understanding of CFLs as “the energy-efficient alternative” to incandescent lamps, as both of these types become more familiar as energy-efficient alternatives themselves.

- Increased CFL Penetration of Lighting Sockets.  CFL saturation among CA households has increased from less than 1% in 2000 to more than 20% from by 2009.  This increasing saturation of California lighting sockets – along with other related factors such as the long lifetime of the typical CFL and consumer concerns about the suitability of CFLs for lighting sockets that require quick start-up or dimming capability – could dampen demand for CFL sales going forward.

- California Policy and Related Changes in IOU Policy.  The lighting chapter of California’s Long-Term Energy Efficiency Strategic Plan was adopted by the commission in late 2010.  One of the key strategies in the Plan includes phasing out IOU incentives for CFLs.  The Plan, in concert with declining NTGR and other factors, has already lead one IOU (SCE) to stop providing incentives for basic CFLs in late 2011.  As the IOUs phase out these incentives, it is unclear whether/how the prices of these products will change.

- Price of Rare Earth Minerals and Other Raw Materials.  Within the past year, China has taken steps to reduce pollution in the rare earth minerals mining and processing industries.  As a result, the country has dramatically reduced supply of these products on the international market.  Because China produces the vast majority of the world’s rare earth materials, the resultant shortage has caused prices for CFLs and other lamp types to surge.  NEMA reports that the average fluorescent lamp price has increased by 37 percent during 2011, and the price increases likely extend to LED lamps as well.  These increased prices widen the incremental price gap between efficient lamp technologies and incandescent lamps, which has implications with regard to the level of the program discount’s influence on energy-efficient lamp purchases and other elements of NTG.

## Approach to Adjusting CFL NTGR Estimates Over Time

The planned evaluation and market studies of the 2010-2012 of the California lighting programs have included a number of approaches for adjusting the CFL NTGR estimates over time.  Most of this evaluation activity is being done under Work Order 28 (WO28): the Upstream and Residential Downstream Lighting Impact Evaluation.  The WO28 research team plans to produce a number of interim reports that will provide updated estimates of CFL NTGR estimates at regular intervals.  The following is the schedule for the interim and final reports.

- 2010-2011 Interim Results Report:  This report will cover 2010 through Q2 2011 program activity.  The final version of this report is expected in mid-2012.

- 2010-2012 Interim Results Report:  This report will cover 2010 through Q2 2012 program activity.  The final version of this report is expected in March 2013.

- 2010-2012 Final Results Report:  This report will cover 2010-2012 program activity.  The final version of this report is expected in August 2013.

In addition to these interim and final reports, the WO28 evaluation will also generate early feedback memorandums throughout the process that can provide new/preliminary findings as they become available.

While the WO28 evaluation effort will collect most of the information needed to produce regular revisions of the NTGR estimates, other planned CPUC-sponsored evaluation activities will also provide useful information.  The most prominent of these is Work Order 13 (WO13): Lighting Programs Process Evaluation and Market Characterization.  The WO13 evaluation will be particularly useful for shedding light on some of the factors such as the EISA legislation and LED market penetration which were discussed in the previous section.  The evaluation will achieve this most immediately through a series of three “early feedback” research tasks.  These tasks include:

- Fast-track lighting retail store shelf surveys:  The WO13 team plans to conduct lighting shelf surveys in 200 California retail stores in the second half of 2011.  These surveys will be conducted primarily to provide both time-series and cross-sectional information on lighting product availability, diversity and pricing, as well as to allow for comparisons over time.

- LED market characterization study:  This study will provide a high-level/preliminary assessment of LED market characteristics and a snapshot of LED product availability and diversity.  The study will also attempt to provide details on screw-base LED lamp and fixture sales in California as well as data on market share and pricing for these technologies.  The market characterization will focus on California’s market to the extent that data will allow, and is expected to include data on the national market where California data is not available.

- Study to assess the early effects of EISA/AB1109:  The purpose of this study is to assess the possible early effects of EISA and California Assembly Bill 1109 mentioned above.  The study will assess the impacts of this legislation on the availability of baseline technologies (i.e., traditional general purpose incandescent lamps) and provide preliminary insights regarding consumer purchasing behaviors in light of possible changes in product availability.

“Advanced” refers to the category of CFLs that are not basic bare spirals less than 30 watts.  Many of these products are not necessarily advanced so, to be more accurate, throughout this report we refer to them as specialty (“advanced”) CFLs.

See Sections 2.3, 3.3 and 6.1.3 in Final Evaluation Report: Upstream Lighting Program, Volume 1, February 2010, KEMA.

Only the supplier self-report and conjoint methods were designed to produce estimates for specific types of CFLs (i.e., basic v. specialty CFLs). Results from those methods were ultimately not used to inform the final recommended NTGR estimates for the 2006-2008 program.

See Table 12 in Itron’s “NTFR Res CFLs 050208.doc.”

Itron indicated that its recommendation of 0.60 as a forward-looking NTGR estimate for basic CFLs was, at that time, conservative due to the significant reported increase in basic CFL sales in states without active programs and their observation of decreasing/converging basic CFL prices across different states.  It should be noted that there was very little evidence provided in the documentation accompanying the last DEER update to support the recommended NTGR estimate of 0.85 for specialty CFLs.

While NTGR estimates were produced using the supplier self-report and the conjoint methods, these estimates were deemed unreliable and thus not considered as part of the final set of recommended NTGR estimates.

Throughout this report, we refer to program tracking data collected for the following IOU program periods: 2006-2008, and 2010 through Q1 2011.  For simplicity, the heading “2010” is often used to represent data reported for the 2010 program year through Q1 2011.  Limited information is available from the 2009 program period; we estimate that 30 million CFLs were distributed in 2009 through the IOU upstream programs.

Massachusetts Energy Star Lighting Program: 2010 Annual Report, Volume 1, submitted to Energy Efficiency Advisory Council Consultants, Cape Light Compact, NSTAR, National Grid, Unitil and Western Massachusetts Electric, June 2011, NMR Group, Inc.

Energy Efficiency / Demand Response Plan: Plan Year 2 (6/1/2009-5/31/2010), Evaluation Report: Residential Energy Star ® Lighting, prepared for Commonwealth Edison Company, December 2010, Navigant Consulting.

Bradsher, K., 2011. China Consolidates Grip on Rare Earths. The New York Times. New York Edition, Page B1. September 15, 2011.