# Recommended Updates to Net-To-Gross Ratios for Industrial and Agricultural End Uses (Except Lighting)

## Introduction

Net-to-Gross ratio (NTGR) values were last updated in 2008.  In late 2010, the CPUC requested that the DEER team develop new NTGR ratios based on more recent research results to increase confidence in their application of NTG ratios to estimate net energy savings for current and future programs.  To meet this objective, a comprehensive literature review was recently conducted of recently completed studies relevant to these customer segments. A key data source was the Program Year 2006-2008 EM&V studies covering both Industrial and Agricultural energy efficiency measure (EEM) categories. For certain categories, values were averaged across related studies to obtain the recommended value.

Some of the factors considered in making a final NTGR recommendation may include:

- The relative merits of the methods and sampling plans used to derive NTGR from past and the most current 2006-08 evaluation studies

- Potential changes in either the energy efficiency program’s design or the EEM’s minimum qualification levels between the previous evaluation and the 2006-08 evaluation, and

- The changing market conditions and saturation levels of the EEMs in the industrial and agricultural sectors.  There are two metrics that could serve as indicators of market saturation

  - Rate of change in the market share of efficient technologies over time

  - Any available evidence on trends in the incremental cost of the targeted energy efficiency measures

After recommending specific NTGR values for the 2011 DEER update, the DEER team identified the program and market factors that could most likely lead to changes in the recommended values over the next three years. We then consider whether there is sufficient data and an acceptable analytical approach to reasonably forecast the NTGR trend over the next three years.  If both conditions are met absent any significant changes in program design and/or qualifying levels for rebated measures, a NTGR forecast for use in 2013-2014 is presented. However, if the assessment concludes that there is neither adequate data nor a good understanding of how market  changes may affect the NTGR values over time, then no adjustments to the estimated 2011 NTGR update values are recommended to account for market changes between now and 2013.

## Comparison of Current DEER NTGR Values to the Results from the 2006 – 2008 EM&V Studies

Table 9-1 compares NTGR estimates for all industrial and agricultural measure categories.  The table is segmented by measure type and fuel.

Table 9-1:  Overview of NTGR Results by Measure and Delivery Approach

Table 9-1 (continued): Overview of NTGR Results by Measure and Delivery Approach

Table 9-1 (continued): Overview of NTGR Results by Measure and Delivery Approach

## Comparison of Methods and Sample Sizes

The studies used to estimate the current DEER values and the latest evaluation studies from 2006-2008 both relied on the Self-Report Approach to derive NTGRs using interviews with participating customers and vendors. The studies underlying the current DEER values were generally those from the PY2004-2005 program evaluations and used a NTG question and scoring rubric that had been in effect for many years.  The studies performed for the 2006-2008 program cycle used a newly-developed set of questions and scoring algorithm, as described below.

Methods. The self-report methodology used in the 2006-2008 EM&V studies was developed by a nonresidential net-to-gross ratio working group that was composed of experienced evaluation professionals. The main purpose of this group was to develop a standard methodological framework, including decision rules, for integrating, in a systematic and consistent manner, the findings from both quantitative and qualitative information in estimating net-to-gross ratios.

The methodology was developed to address the unique needs of Large Nonresidential customer projects developed through energy efficiency programs offered by the four California investor-owned utilities and third-parties.  This method relies exclusively on the Self-Report Approach (SRA) to estimate project and program-level Net-to-Gross Ratios (NTGRs), since other available methods and research designs are generally not feasible for large nonresidential customer programs.

The method introduces a 0 to 10 scoring system for key questions used to estimate the NTGR, rather than using fixed categories that were assigned weights (as was done previously).  It asks respondents to jointly consider and rate the importance of the many likely events or factors that may have influenced their energy efficiency decision making, rather than focusing narrowly on only their rating of the program’s importance.  This question structure more accurately reflects the complex nature of the real-world decision making and helps to ensure that all non-program influences are reflected in the NTGR assessment in addition to program influences.

There are three levels of free-ridership analysis.  The most detailed level of analysis, the Standard – Very Large Project NTGR, is applied to the largest and most complex projects (representing 10 to 20% of the total) with the greatest expected levels of gross savings The Standard NTGR, involving a somewhat less detailed level of analysis, is applied to projects with moderately high levels of gross savings. The least detailed analysis, the Basic NTGR, is applied to all remaining projects.  Evaluators must exercise their own discretion as to what the appropriate thresholds should be for each of these three levels.

Data Sources. There are five sources of free-ridership information in this study.  Each level of analysis relies on information from one or more of these sources.  Table 1 below shows the data sources used in each of the three levels of free-ridership analysis. Although more than one level of analysis may share the same source, the amount of information that is utilized in the analysis may vary.  For example, all three levels of analysis obtain core question data from the Decision Maker survey.

Table 9-2: Information Sources for Three Levels of NTGR Analysis

|  | Program File | Decision Maker Survey Core Question | Vendor Surveys | Decision Maker Survey Supplemental Questions | Utility & Program Staff Interviews | Other Research Findings |
| --- | --- | --- | --- | --- | --- | --- |
| Basic NTGR | √ | √ | √1 |  | √2 |  |
| Standard NTGR | √ | √ | √1 | √ | √ |  |
| Standard Very Large NTGR | √ | √ | √3 | √ | √ | √ |

1 	Only performed for sites that indicate a vendor influence score (N3d) greater than maximum of the other program element scores (N3b, N3c, N3g, N3h, N3l).

2	Only performed for sites that have a utility account representative

3 	Only performed if significant vendor influence reported or if secondary research indicates the installed measure may be becoming standard practice.

NTGR Questions and Scoring Algorithm. The NTGR is calculated as an average of three scores.  Each of these scores represents the highest response or the average of several responses given to one or more questions about the decision to install a program measure.

- A Timing and Selection score that reflects the influence of the most important of various program and program-related elements in the customer’s decision to select the specific program measure at this time. Program influence through vendor recommendations is also incorporated in this score.

- A Program Influence score that captures the perceived importance of the program (whether rebate, recommendation, training, or other program intervention) relative to non-program factors in the decision to implement the specific measure that was eventually adopted or installed. This score is determined by asking respondents to assign importance values to both the program and most important non-program influences so that the two total 10. The program influence score is adjusted (i.e., divided by 2) if respondents say they had already made their decision to install the specific program qualifying measure before they learned about the program.

- A No-Program score that captures the likelihood of that the customer might have taken the same program qualifying action at this time and in the future if the program had not been available (the counterfactual). This score also accounts for deferred free ridership by incorporating the likelihood that the customer would have installed the program-qualifying measure at a later date if the program had not been available.

When there are multiple questions that feed into the scoring algorithm, as is the case for the Timing and Selection score, the maximum influence rating is used.  The rationale for using the maximum value is to capture the most important element in the participant’s decision making.  Thus, this score is always based on the strongest influence indicated by the respondent. However, high scores that are inconsistent with other previous responses trigger consistency checks and can lead to follow-up questions to clarify and resolve the discrepancy.

The self reported core NTGR is simply the average of the Program Influence, Timing and Selection, and No-Program Scores, divided by 10.

Sample sizes. As Table 9-3 summarizes, the 2006-2008 studies reached significantly larger sample sizes and were measure specific.  For steam traps and custom measures, interviews with vendors were conducted to support the NTG analysis for those industrial applications where the vendor was an influential factor in the decision to install the measure.  In addition, utility account reps were interviewed in all cases to capture any possible influence they may have had on the decision to install the project.

The larger sample sizes, coupled with a more comprehensive battery of questions and a strong focus on measure specific conditions that affect estimates of free ridership resulted in more robust NTGR values in the 2006-08 program evaluations.

Table 9-3:  Methods and Sample Sizes for Agricultural and Industrial Measures

| Measure | Market Segment and Program Year | NTGR Method Used | Sample Sizes DEER 2006-2008 |  | Delivery Mechanism |
| --- | --- | --- | --- | --- | --- |
| Agricultural– Heat curtains | Agricultural greenhouse – 2006-08 | Self Report | 50 | 51 | Downstream Prescriptive |
| Agricultural– Infrared Film | Agricultural greenhouse – 2006-08 | Self Report |  | 35 | Downstream Prescriptive |
| Industrial Steam Traps | Industrial – 2006-08 | Self Report | 113 PG&E–34 SCE E–67 SDG&E E–12 | 126 | Customized |
| Industrial Pump Off Controllers | Industrial - 2006-08 | Self Report |  | 250 | Customized |
| Custom Electric Measures | Industrial – 2006-08 | Self-Report |  | 408 |  |
| Industrial Pipe Insulation | Industrial –2006-08 | Self Report |  | PG&E–35 SCG E–240 | Customized |

Overall the DEER team finds that the recently completed PY2006-2008 evaluations contain more robust and comprehensive NTGR estimates than the values in the existing DEER database, based on our review of methodologies and sample sizes.

## Industrial/Agricultural NTGR Sensitivity Analysis

For the PY2010-12 evaluation, a dual baseline approach is being implemented, which incorporates the effect of project timing into the gross savings calculation.  Previously, this effect has been captured in the deferred free ridership component of the NTG ratio.

The DEER team ran a sensitivity analysis in which the timing effect was removed from the NTG ratio calculation for the Industrial and Agricultural sector studies.  The resulting NTGRs are very slightly lower, as shown below.

Table 9-4: Industrial/Agricultural NTGR Sensitivity Analysis Results

| Evaluation Study | Sampling Domain | 2008-2008 NTGR | Sensitivity Analysis with Timing Effect Eliminated |
| --- | --- | --- | --- |
| PG&E Fabrication, Process and Manufacturing Contract Group | Pump-Off Controllers (POCs) | kWh =0.45, kW=0.44 | kWh =0.45, kW=0.44 |
| PG&E Fabrication, Process and Manufacturing Contract Group | Non-POCs | kWh =0.60, kW=0.59 | kWh =0.58, kW=0.57 |
| PG&E Fabrication, Process and Manufacturing Contract Group | Overall electric | kWh=0.53, kW=0.52 | kWh=0.51, kW=0.51 |
| PG&E Fabrication, Process and Manufacturing Contract Group | Gas | 0.31 | 0.30 |
| Southern California Industrial and Agricultural Contract Group | Agricultural measures | kWh=0.59, kW=0.63 | kWh=0.56, kW=0.59 |
| Southern California Industrial and Agricultural Contract Group | Industrial measures | kWh=0.63, kW=0.65 | kWh=0.60, kW=0.61 |

## Recommended 2011 DEER Updates to NTGR Values

Table 9-5 shows the recommended NTGRs to be used to update the DEER database in 2011 for use in ex ante values in 2013 programs and beyond. Because of the large sample sizes and multi–tiered approach used to estimate free ridership, the DEER team recommends use of the 2006-2008 study results shown in Table 9-3 for the reviewed measures.

Table 9-5:  Recommended NTGR by Measure and Delivery Method

| EEM | EEM Characteristics | Delivery Mechanism | Applicable Sector and Building Types | Utility Specific or Statewide NTGR | Recommended NTGR Values |
| --- | --- | --- | --- | --- | --- |
| Pump off controllers |  | Downstream, Custom | Major oil companies | SW | 0.42 |
| Pump off controllers |  | Downstream, Custom | Independent oil companies | SW | 0.74 |
| Custom Electric Measures |  | Downstream, Custom | Large Commercial and Industrial | SW | 0.60 |
| Custom Gas Measures |  | Downstream, Custom | Large Commercial and Industrial | SW | 0.35 |
| Pipe Insulation | Various diameters and pressures | Downstream, Custom | Industrial | SW | 0.71 |
| Steam Traps | Pressure levels: High and Low | Customized | Industrial | SW | 0.52 |
| Greenhouse Heat Curtain |  | Downstream Prescriptive, | Agricultural greenhouse | SW | 0.63 |
| Infrared Film |  | Downstream Prescriptive | Agricultural greenhouse | SW | 0.46 |
| kWh, kW and therms |  | Downstream, Custom | Agricultural | SW | 0.70 |

## Factors Likely to Lead to Changes in NTGR Over Next Three Years

The DEER team reviewed the three most recent NTGR evaluations and the overall literature to identify key factors likely to lead to changes in NTGR over time for the two Agricultural and four Industrial measure programs.

- Changes in qualifying and rebate levels for measures that have an effect on payback periods

- Changes in Program design and delivery as it pertains to vendors

  - e.g., the installation of commercial steam traps involves a more sophisticated approach to program delivery and setting of rebate levels.

- Changes in industry standard practice

  - e.g., for particular measures suspected of becoming standard practice such as pump-off controllers and injection molding machines

- Non-energy factors (e.g., regulatory mandates)

  - e.g., As applicable to projects involving pollution control equipment

- Business trends in the vendor/contractor program volume.

## Assessment of Current Data and Tools Available to Prepare a Forecast of NTGR Values

The most recent study and previous evaluations of the SPC and industrial programs not only have derived similar NTGRs over the past several years, but have consistently raised concerns regarding the relatively high levels of free-ridership among customers who installed the four industrial measures. These evaluations have also recommended changes in program design to reduce these high free –ridership levels. Following the PY2004-2005 evaluation, these recommendations were not implemented and it is unclear the extent to which they have been implemented in this program cycle.  In addition, the only difference in rebate levels has been the addition of a per-kW incentive on top of the traditional per-kWh incentive.  However, this was largely done to help meet kW goals, not to cut back on free ridership.  On the other hand, for pump-off controller measures, the IOUs did in fact suspend rebates on POCs for new well applications starting in 2009 based on the 2006-2008 Early Feedback Memo.

The trends in NTGRs have been relatively flat over the past several evaluation cycles. The 2006-2008 Study NTGRs are virtually identical to the estimate of corresponding net-to-gross ratios for the statewide Standard Performance Contracting (SPC) program in the PY2004-2005 evaluation. In addition, they are very similar to the NTG estimates made in prior SPC evaluations conducted for each program year since the program’s inception in 1998, as shown in Table 9-6.  This implies it is appropriate to apply current NTGRs in future cycles.

Table 9-6:  Statewide Custom Electric Measures Net to Gross Ratios for the Period 1998-2008

| (1-FR) | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004-2005 | 2006-2008 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weighted | 0.53* | 0.51 | 0.41 | 0.65 | 0.45 | 0.59 | 0.57 | .059 |
| Unweighted | 0.49 | 0.48 | 0.46 | 0.55 | 0.45 | 0.60 | 0.54 |  |

* Weighted by incentives rather than by kWh savings.

## Recommended NTGR Values for Future Programs

Given the stability of NTGR values over the past several evaluation cycles, and the fact that many of the recommendation for program improvements have not been implemented, the DEER team advises using the recommended NTGR values (based on the 2006-08 EM&V studies) for future program years. Thus we recommend no adjustment be made to the NTGR values presented in Table 9-5.

Note that we do not refer to an Enhanced level of analysis, since this is defined by the Protocols to involve the application of two separate analysis approaches, such as billing analysis or discrete choice modeling.

The kW Engineering study focused on the overall Agricultural program, and did not use a HIM-based approach.  Therefore, the NTGR is representative of the program as a whole, not any particular measure.