CPUC Energy division

DEER Scaled and Weighted Measures

Part of the Ex-Ante data Submittal for 2013-2014 Planning and Reporting

Paul Reeves

5/20/2012

A description of the scaled and weighted measure types in the DEER2011 database, with a guide to creating new scaled and weighted measures to accompany IOU workpapers.

Contents

Scaled DEER Measures	2

Indoor and Outdoor Lighting Measures	2

Outdoor Lighting Measures	3

Residential Refrigerator/Freezer Measures	3

Residential Dishwashers and Clothes Washers	3

Using READI to create new Scaled Measures	4

Weighted DEER Measures and Impacts	6

Cross-Measure Weighted Measures	6

Weighted DEER Impacts	6

Creating new Weighted DEER results	7

The development and submittal of scaled DEER measures and weighted DEER measures requires far fewer data tables and documentation than standard deemed measures.  These types of measures utilize approved DEER methodologies in the development of the energy savings values.  Once approved, these measures can be referenced as any other DEER measure.

# Scaled DEER Measures

Many DEER measures utilize a set of “normalized” energy impacts that are scaled to accommodate the specific technologies associated with the measure.

## Indoor and Outdoor Lighting Measures

All DEER energy impacts associated with lighting measures are scaled based on the measure definition of base case technology total Watts and measure technology total Watts along with a set of normalized energy impacts.  The DEER2011 database includes scaled lighting measures for the following measure categories:

- Residential Indoor lighting measures

- Residential Outdoor lighting measures

- Commercial Indoor Linear Fluorescent and HID Fixture measures

- Commercial Indoor Exit Fixture measures

- Commercial Indoor Lamp replacement measures

Energy impacts for these measures are calculated by READI using data from three tables:

kWh savings = 	Dir_EnImpact (kWh/ΔWatt) * Meas_ΔWatt * IE_kWh

kW savings = 	Dir_DemImpact (kW/ΔWatt) * Meas_ΔWatt * IE_kW

therm savings = Dir_EnImpact (kWh/ΔWatt) * Meas_ΔWatt * IE_therm

where:

Dir_EnImpact = normalized direct electricity savings (kWh/ΔWatt) as stored in the energy impacts table.

Dir_DemImpact = normalized direct electric demand savings (kW/ΔWatt) as stored in the energy impacts table.

Meas_Δwatt = Measure “delta” Watts, defined as (Base technology Watts – Measure technology Watts); there are separate values for above-code and above pre-existing cases.  These values are stored as part of the measure definition or are calculated based on the technology references that are part of the measure definition.

IE_kWh = HVAC Interactive Effects factor for electricity use (kWh per kWh)

IE_kW = HVAC Interactive Effects factor for electricity demand (kW per kW)

IE_therm = HVAC Interactive Effects factor for gas use (therm per direct kWh)
HVAC interactive effects values are dependent on the specified building type, building vintage, location and HVAC system type; the values are store in tables based on the measure type.

## Outdoor Lighting Measures

HVAC interactive effects are not included in the calculation of outdoor lighting measure energy impacts, so the equations simplify to:

kWh savings = 	Dir_EnImpact (kWh/ΔWatt) * Meas_ΔWatt

kW savings =	 Dir_DemImpact (kW/ΔWatt) * Meas_ΔWatt

## Residential Refrigerator/Freezer Measures

The energy impacts associated with residential refrigerator and freezer measures are determined by the measure technology DOE-rated annual energy use (kWh/yr):

kWh savings = 	WB_EnImpact (kWh/ΔRatedkWh) * Meas_ ΔRatedkWh

kW savings = 	WB_DemImpact (kW/ ΔRatedkWh) * Meas_ ΔRatedkWh

therm savings =	WB_GasImpact (therm/ ΔRatedkWh) * Meas_ ΔRatedkWh

where:

WB_EnImpact = normalized whole building electricity savings (kWh/ΔRatedkWh) as stored in the energy impacts table.

WB_DemImpact = normalized whole building electric demand (kW/ΔRatedkWh) as stored in the energy impacts table.

WB_GasImpact = normalized whole building gas savings (therm/ΔRatedkWh) as stored in the energy impacts table.

Meas_ΔRatedkWh = Measure “delta” rated annual kWh, defined as (Base technology rated annual kWh – Measure technology rated kWh); there are separate values for above-code and above pre-existing cases.  These values are stored as part of the measure definition or are calculated based on the technology references that are part of the measure definition.

## Residential Dishwashers and Clothes Washers

The energy impacts associated with residential dishwasher and clothes washer measures are determined by the normalized impacts and the specified number of use cycles per year:

kWh savings = 	WB_EnImpact (kWh/cycle) * NumCycles

kW savings = 	WB_DemImpact (kW/ cycle) * NumCycles

therm savings =	WB_GasImpact (therm/ cycle) * NumCycles

where:

WB_EnImpact = normalized whole building electricity savings (kWh/cycle) as stored in the energy impacts table.

WB_DemImpact = normalized whole building electric demand (kW/cycle) as stored in the energy impacts table.

WB_GasImpact = normalized whole building gas savings (therm/cycle) as stored in the energy impacts table.

NumCycles = the measure specified number of use cycles per year.

## Using READI to create new Scaled Measures

When one downloads the Measures table from READI, a comma delimited text file is created, with each measure definition occupying a single line in the text file.  This file can be edited directly (as an imported spreadsheet) to create new measure definitions.  However, since many of the values have a limited set of options and are interconnected, it is typically best to use READI to create the new measure definitions.

When imported into READI, new measures will function like all other scaled measures in the database, but they will not be added to the DEER database or be viewable to other users. The new measures can be used to determine the associated energy impacts and can be submitted as part of IOU work papers. Only scaled measures (including all lighting measures) can be created with this process.

Follow these main steps to create a new scaled measure with READI:

#### Start with a measure that is similar to the desired new measure.

On the “Measures” tab of READI, select a measure that has the same Use Category, Use Sub-category, Technology Group and Technology Type as the measure you want to create.  Additionally, select a measure that utilizes as many of the technologies that will be in associated with the new measure.

With the starting measure selected, go to the main menu “Tools => Create new Scaled Measure” option (or right click on the row in the measure list).  Note: the “Create New Scaled Measure” option is only enabled when a qualifying measure is selected.

#### Use the “Add Measure” tab of READI to specify the new measure.

Adding a new measure is a three-step process, with each step using a new screen of options. Each step has “required actions” that must be completed before the new measure is fully defined and can be saved to your computer.

###### Step 1: Specify Technologies

On the "Specify Technologies" tab you can either select or describe the new technologies associated with this measure. It is always preferable to select pre-defined technologies from the DEER technologies table when possible. To select a new technology from the DEER tables, follow these steps:

- Make sure the "link" is checked next to the technology case of interest. This will enable the options needed to link a measure technology to the DEER technology tables.

- Click the "Find" button next to the technology case you want to replace. You will be taken to the technology list with the current technology selected.

- In the Technology Table, browse to the desired replacement technology record. You can filter and sort the list by right-clicking on the column titles.

- Right-click on the new technology and select the technology case that this new technology will replace (Measure, Code or Pre-Existing)

If the technology needed for the new measure definition is not in the technology tables, you can "unlink" the description from the technology table by unchecking the "link" option for the appropriate technology case. When you do this, you will be able to enter a technology description and a technology Scale Value.  Be sure the entered scale value is consistent with the scale value of the other technologies in the measure definition.

If you choose a measure technology with associated code technologies, you will be able to choose which code technology by double-clicking on the technology from the list at the bottom of the “Specify Technologies” screen.

###### Step 2: Update Measure Information

On the "Update Measure Info" tab, enter a new Measure ID, Measure Description and Comment.

The MeasureID should be unique, especially if the new measure definition will be submitted as part of IOU workpapers.  Click the update button to have READI automatically create an ID based on the measure classification and associated technology IDs.

The Measure Description should include enough details to differentiate it from similar measures. Click the update button to have READI automatically create a description based on the associated technologies.

Note: whenever you link to a new technology in Step 1, the program will automatically create a new Measure ID and Description; you can accept these new values or edit them as appropriate.

###### Finally, select the applicable IOU from the pull-down list.  IOU submitted measures that are of general interest can specify “All”.

###### Step 3: Review Measure Data

Review all of the other measure information on the last tab.  None of the values on this screen can be altered, but calculation and classification data should be examined to assure that the measure being created corresponds with the intended new measure.

Only after you have completed all three steps and all sections with an "Action Required" title are completed will the "Save" button at the bottom of the “Add Measure” screen be enabled.  When you press the Save button, you can add the new measure definition to an existing file or write it to a new file.

#### Import the new measure into READI

To and examine the new measure definition and its associated energy impacts use the main menu “Tools => Import Local Data => Measure Definitions”.  Specify the CSV file that you saved new measure definitions to.  If you choose a file that READI cannot interpret correctly, you will be given a chance to stop the import process.

READI will attempt to interpret each line in the specified import file as a measure definition.  Lines that begin with “!” or “$” or “;” will be skipped and can be used to add comments to the new measure definition file.

After the measures are imported, select the measure from the measure list to see the measure definition and double-click on the row in the measure list to see the associated energy impacts.

# Weighted DEER Measures and Impacts

There are two types of weighting that can be done with DEER measures: weighting that creates new measures and associated energy impacts from existing measures and weighting that creates new energy impacts for a given measure.

## Cross-Measure Weighted Measures

Measures from the same technology group can be combined and weighted to create new measure definitions and new energy impacts.  This process is referred to as Cross-Measure Weighting.

Cross-measure weighting can accomplish two things:

- Combine Energy Impacts: energy impacts from different measures are combined, or merged, into a single measure.  The energy impacts are “selected” based on applicability.  Example: residential ARP and duct sealing.  In this case, measures that are specific to an IOU or to a building type are combined into a single measure.

- Weight Energy Impacts: energy impacts from different measures but for the same applicability are weighted together into a single measure. Example: residential refrigerant charge.

A simple example of this latter type of cross-measure weighting is the weighting of two HVAC measures to produce a new measure with a new efficiency parameter.  Example: Residential SEER 14.5 AC measure. In this example case, the SEER 14 measure and the SEER 15 measure are given equal weighting to produce a SEER 14.5 measure.  This type of weighting can represent two different concepts: a new measure technology that is represented by the weighted efficiency parameter, or, a measure that represents the utilization of two separate measure technologies.

Some rather complex processing can be accomplished via the relatively simple weight tables. Residential clothes washer cross-measure weighting, for example, combines results from different building types (each of which has a different assumed number of cycles per year) and weights results within each building type based on the combination of clothes washer and hot water heater fuel type.

The weights that this process uses are viewable within READI on the “Cross-Measure Wts” tab and are documented in the “DEER2011-Weights-Development_1May2012.xls” workbook.

The weighting process is executed once, and the new results are stored as part of the ex-ante database.  That is, the weighted results are not calculated “on-the-fly”, and all cross-measure weighted measures are stored in the DEER database as a standard energy impact calculation type.

## Weighted DEER Impacts

Inter-Measure Weighting refers to the weighting of energy impacts within a single measure to create a new building type, building vintage or location.  DEER standard processing uses “inter-measure” weighting to create the “Existing” vintage, “IOU” climate zone and “Res” or “Com” building type energy impacts for DEER measures.   READI also produces the weighted HVAC type using this method.

The weights that this process uses are viewable within READI on the “Inter-Measure Wts” tab and are documented in the “DEER2011-Weights-Development_1May2012.xls” workbook.

## Creating new Weighted DEER results

The DEER weighting process was designed to be flexible and easily augmented, without the need for additional programming or new program versions.  Once the weights definitions and weight values are added to the database tables, the new weights can be applied to any number of existing measures.

Since the weighted results are stored in the DEER database, the process must be executed by a user with administrative privileges.  At this time, the only method to add weighted measures or energy impacts to the DEER database is to send the new cross-measure or inter-measure weights to Energy Division.  Energy Division will review and add the new weighted results as appropriate.