# Recommended Updates to the Net-to-Gross Ratios for Residential HVAC Systems and Building Envelopes

## Introduction

The DEER team reviewed the Net-to-Gross Ratio (NTGR) results from the 2006 – 2008 EM&V studies for a select group of Residential HVAC energy efficiency measure (EEM) categories and compared them the California Public Utilities Commission’s (CPUC) adopted values in the 2008 DEER database, Version 2.05.  The objective was to arrive at a recommended set of NTGR values for the 2011 DEER update.  For each measure, we compared the strengths and weaknesses of the methods and datasets used to derive the NTGR results and contrasted them to the relative strength of the methods and datasets used to establish the values for the 2008 DEER update.  Some of the factors that may be considered in recommending a revised NTGR value include:

- Methods and Sample Sizes.  The relative merits of the methods and sample sizes used to derive the NTGR estimates in the existing DEER database and new evaluation studies.

- Program Design Changes.  The potential changes in either the energy efficiency program’s design or the EEM’s minimum qualification levels.

- Market Changes.  The changing market conditions and saturation levels of the EEM in the specific market segments.  There are two metrics that serve as indicators:

  - The rate of change in the measure’s market share over time,  and

  - The trend in the measure’s incremental cost over time.

After recommending specific NTGR values for the 2011 DEER update, the DEER team identified possible program and market factors that could change the recommended values over the next three years.  We then consider whether there is sufficient data and an acceptable analytical approach to forecast the NTGR trend over the next three years.  If both conditions are met satisfactorily and absent any significant changes in either program design or measure qualifying levels, an NTGR forecast is prepared and presented for use in 2013 – 2014.  However, if the assessment concludes that there is neither adequate data nor a good understanding of how market changes may affect the NTGR values over time, then no adjustments to the estimated 2011 NTGR update values are recommended to account for market changes between now and 2013.

## Comparison of the Current DEER NTGR Values to the Results from the 2006 - 2008 EM&V Studies

Table 12-1 lists the Residential HVAC measure categories that were selected for the 2011 DEER updates based upon the 2006-2008 impact evaluation reports.  The table summarizes the NTGR values from the 2008 DEER v2.05 database and the results from the final evaluation reports.

Table 12-1:  Overview of NTGR Results by Measure and Delivery Method

| Energy Efficiency Measure | Delivery Method in 2008 DEER | NTGR Values in 2008 DEER (Source) | ‘06-‘08 NTGR Results (Source) |
| --- | --- | --- | --- |
| Room Air Conditioners | Downstream Prescriptive Rebate | 0.70, Default (Source: ‘04-‘05 Res. Retrofit) | Program NTGR PGE2000 0.41 SCE2501 0.36 SDGE3024 0.31 (Source: Res. Retrofit) |
| HVAC Maintenance: Duct Sealing | Free Tune-up/Repair | 0.78, HVAC Diagnostic (Source: Unknown) | Program NTGR PGE2000 0.54 PGE2078 0.85 SCE2501 0.79 SCE2507 0.96 SDGE3035 0.80 (Source: HVAC HIM and Specialized Commercial) |
| HVAC Maintenance: Refrigerant Charge Adjustment | Free Tune-up/Repair | 0.78, HVAC Diagnostic (Source: Unknown) | Program NTGR PGE2000 0.63 PGE2078 0.78 SCE2501 0.78 SCE2507 0.97 SDGE3035 0.78 (Source: HVAC HIM and Specialized Commercial)3 |
| Roof and Wall Insulation | Downstream Prescriptive Rebate | 0.70, Wall and Ceiling Insulation (Source: ‘04-‘05 Res. Retrofit) | Prgm kWh kW Therms PGE 0.25 0.28 0.26 SCG 0.30 0.30 0.29 SDGE 0.25 0.26 0.25 (Source: Res. Retrofit)2 |
| Air Cooled Packaged and Split System Air Conditioners and Heat Pumps | Downstream Prescriptive Rebates | 0.67, Central AC >14 SEER (Source: ‘04-‘05 Res. Retrofit ) 0.80, Central AC >15 SEER (Source: Unknown) 0.55, Heat Pump – Energy Star (Source: ‘04-‘05 Res. Retrofit) | Program NTGR SCE2507 0.56 SDGE3029 0.53 (Source: HVAC HIM and Specialized Commercial)3 |

## Comparison of Methods and Sample Sizes

The sample sizes and methods used to determine the free-ridership and NTGR estimates in the 2006 – 2008 impact evaluation studies are summarized in Table 12-2.

Table 12-2:  Methods and Sample Sizes for Residential HVAC in the ‘06-‘08 EM&V Studies

| EEM | Market Segment and Program Year | NTGR Method Used | Sample Sizes | Delivery Methods in Studies |
| --- | --- | --- | --- | --- |
| Room Air Conditioners | Residential 2006 – 2008 | Self Report Approach | PGE2000 91 SCE2501 629 SDGE3024 377 Total: 1,097 | Downstream |
| HVAC Maintenance: Duct Sealing | Residential 2006 – 2008 | Self Report Approach | PGE2000 211 PGE2078 103 SCE2502 102 SCE2507 112 SDGE3035 102 Total: 630 | Midstream |
| HVAC Maintenance: Refrigerant Charge Adjustment | Residential 2006 – 2008 | Self Report Approach | PGE2000R 135 SCE2507 94 CMMHP 309 Total: 538 | Midstream |
| Roof and Wall Insulation | Residential 2006 – 2008 | Self Report Approach | PGE2000 448 SCG3517 779 SDGE3024 530 Total: 1,797 | Downstream |
| Air Cooled Packaged and Split System Air Conditioners and Heat Pumps | Residential 2006 – 2008 | Self Report Approach | SCE2507 204 SDGE3029 153 Total: 357 | Upstream, Downstream |

The 2008 DEER does not list a specific NTGR value for Energy Star qualified Room Air Conditioners (RAC).  Hence, the default value of 0.70 applies to this measure.  For the 2006 – 2008 programs, the investor owned utilities (IOUs) used a default NTGR value of 0.80 for their ex ante claimed results.  However, the 2004 – 2005 Residential Retrofit impact evaluation study estimated an NTGR value of 0.69, listed in the report’s Table 9-47, based on a Self Report evaluation of 50 surveys.

The HVAC Maintenance Duct Sealing measure, also referred to as Duct Test and Seal (DTS) in some instances, is categorized as an HVAC Diagnostic measure in the 2008 DEER update.  The 2004 – 2005 Statewide Residential Retrofit Study did not evaluate this measure.  Likewise the HVAC Maintenance Refrigerant Charge Adjustment measure was not evaluated as part of the 2004 – 2005 Statewide Residential Retrofit Study.  A search of the available reports on the CALMAC website did not find an impact evaluation with NTGR values that approximate the values adopted in the 2008 DEER updates, and the available DEER update documentation does not describe how the adopted values were established.

The Roof and Wall Insulation measures were evaluated in the 2004 – 2005 Residential Retrofit Study and recommended an NTGR value of 0.70, listed in the report’s Table 9-47.  The study evaluated the insulation measure’s free-ridership using both the Self Report and Discrete Choice approaches.  The study’s Self Report approach estimated the NTGR at 0.53 based on 253 observations.  The report’s recommended NTGR for this measure used the Discrete Choice method based on 243 observations.  The report states that “…the final NTG ratios used to calculate the net Program impacts are based on the discrete choice results since they are generally thought to be more robust than the self-report methods.”  The insulation Discrete Choice approach was a one-stage model used to determine the probability of installing insulation.

There are two NTGR values listed in the 2008 DEER for Air Cooled Packaged and Split System Air Conditioners and one for Heat Pumps.  Both the 0.67, for Central AC > 14 SEER, and the 0.55 for Heat Pumps, may be found among the recommended values in Table 9-47 in the 2004 – 2005 Residential Retrofit Study.  The recommended Central AC estimate was derived from a two-stage Discrete Choice model using 248 participant surveys.  The Heat Pump NTGR estimate was derived from a Self Report evaluation based on 55 observations as listed in Table 9-35 in the ’04-’05 Residential Retrofit Study report.  The 2008 DEER adopted NTGR value of 0.80 for Central AC > 15 SEER is not found in the ’04-’05 Residential Retrofit Study.  The 2008 DEER update documentation does not specify how the adopted value was established.

## Recommended 2011 DEER Updates to the NTGR Values

Overall, the DEER team found that the most recent evaluations had larger sample sizes and improved Self Report evaluation methods over the Self Report methods used in the 2004 – 2005 Residential Retrofit Study.  The 2006 – 2008 impact evaluation studies did not use Discrete Choice models to estimate free-ridership for the measures selected for the 2011 DEER Updates.  Hence, we cannot conclude unequivocally that the current set of Self Report Approach NTGR estimates would be close in value to a set of values derived using updated Discrete Choice models and the same dataset of surveys.  The expectation is that the changed market conditions and baselines for the evaluated measures from the prior evaluation to the circumstances for the 2006 – 2008 timeframe would lead to similar results.  It is important to point out that at least one Discrete Choice analysis performed for non-residential packaged and split system air conditioners encountered a number of problems and resulted in a lower value than the corresponding Self Report Approach.  With this limitation in mind for Central Air Conditioner and Insulation measures, we recommend using the NTGR values the team has derived and summarized in Table 12-3 for the 2011 DEER update.  The values were derived through weight averaging the utility level estimates from the ’06-’08 evaluations to arrive at statewide recommended values.  The analyst’s summaries and data collection notes are found in the Appendices A-5.1 and Appendix A-5.2 provides the embedded spreadsheet calculations that derived the recommended statewide averages.

Table 12-3:  Recommended NTGR by Measure and Delivery Method

| EEM | EEM Characteristics | Delivery Methods | Applicable Sector and Building Types | Utility Specific or Statewide NTGR | Recommended NTGR Values for 2011 Update |
| --- | --- | --- | --- | --- | --- |
| Room Air Conditioners | Energy Star Qualified | Downstream | Residential: All Building Types | Statewide | 0.36 |
| HVAC Maintenance: Duct Sealing | n/a | Midstream | Residential: All Building Types | Statewide | 0.78 |
| HVAC Maintenance: Refrigerant Charge Adjustment | n/a | Midstream | Residential: All Building Types | Statewide | 0.78 |
| Roof and Wall Insulation | Existing Roof Insulation Level ≤ R-11 and EEM Level ≥ R-30 if 24” of space available or ≥ R-19 if less than 24” available No Existing Wall Insulation | Downstream | Residential: Single Family | Statewide | 0.28 |
| Air Cooled Packaged and Split System Air Conditioners and Heat Pumps | SEER ≥ 14 | Upstream, Downstream | Residential: All Building Types | Statewide | 0.55 |

The Room Air Conditioner NTGR value is intended for Energy Star Qualified units rebated through a downstream program delivery approach.  The 2011 NTGR value is an interim recommendation only.  As stated in the ’06-’08 Residential Retrofit report “…In 2006-2008 the National ENERGY STAR retailer partners reported the national market share data for ENERGY STAR room air conditioners was 36%, 50%, and 43%, respectively.  While this is not an estimate of free-ridership, it is an indication that sales of ENERGY STAR room air conditioners were in the 36%-50% range throughout the U.S., substantially lower than the self-reported estimate of free-ridership in this study.”  In 2006, the reported California share of sales of Energy Star RACs stood at 21%, well below the national average.  In 2007, the reported California sales of Energy Star units went up to 51%, slightly higher than the national level.  In 2008, the reported California Energy Star share of sales had dropped to 38%, below the levels reported at the national level by five percentage points.  In the last available Energy Star sales dataset for 2009, the California and national share of sales of Energy Star RACs both stood at 36%.  In addition, a small survey conducted as part of the process evaluation of the SCE 2006 – 2008 Home Energy Efficiency Rebate program found for room air conditioners that “…Fifty-six percent of those who received a rebate on their recent air conditioner purchase said they would have been very unlikely to have bought the same model without this rebate. A third (33%) said they would have been somewhat likely to make the same decision without the rebate, and only 11 percent said they would have been very likely to buy the same model without rebates. Once again, the handful of respondents who reported knowingly buying a non-rebated model could not provide a reason for their decision.”  The Technology Group 8 team believes that a further time-series trend review of incremental costs, adoption rates and market conditions, along with a comparative analysis of retailer stocking practices by capacity and efficiency levels is necessary to adjust the final recommendation for the 2013 – 2014 time periods.

For both the Duct Sealing and Refrigerant Charge Adjustment HVAC Maintenance measures, the team considers that the future of these measures as standalone offerings in IOU programs will be limited due to unresolved uncertainties surrounding the field measurements and the launch of the Quality Maintenance HVAC program in June 2011 that bundles these and other measures into a single service offering.  Also, these measures were the only ones among the examined residential HVAC measures whose NTGR values failed to cluster about a central value across IOU programs in the ’06-’08 evaluation studies.  The studies failed to satisfactorily explain the occurrence of extreme high and low NTGR results for very similar programs across California.  Therefore, the team recommends no adjustments to the 2008 DEER NTGR values for both of these HVAC maintenance measures based on the ’06-’08 evaluation results.

The residential Roof and Wall Insulation measure is a long standing conservation option within the IOU programs that has been offered on an on-again/off-again basis for close to 30 years.  The mature markets for these measures are reflected in the increasing free-ridership observed in successive evaluations using different evaluation methodologies.  The team considers that the measures will continue to be part of programs that encourage Whole Building retrofits and take advantage of home renovation opportunities. The recommended value of 0.28 should not be applied to these insulation measures when they are bundled as part of a whole building retrofit but are appropriate if rebates are offered directly for these measures on a standalone basis.   The team considers the recommended statewide value for the stand alone insulation measures a reasonable reflection of the mature laggard market and stringent Building Energy Code requirements the measures face in California.

Lastly, the team recommends the adoption of the Air Cooled Package and Split System Air Conditioner and Heat Pump NTGR value listed in Table 12-3 with two exceptions.  First, it appears that only one IOU portfolio continues to offer rebates for central air conditioner and heat pump replacements through the newly launched Quality Installation HVAC Program.  Given the undisputed problems associated with the operating performance of many newly installed residential central air conditioning systems, the team does not recommend applying the statewide value derived from prior AC replacement programs to the Quality Installation HVAC Program due to the significant program changes.  This program should be subjected to early EM&V to evaluate its effectiveness and potential free-ridership.  Second, the 2008 DEER adopted a high NTGR value for systems with efficiency levels above SEER 15.  The team considers that very high performance systems may still warrant a high default NTGR value.  The team recommends continuing the policy and that the SEER level be raised to 16 with a corresponding EER greater or equal to 13 for split system AC units (2009 CEE Tier 3) for the 2013 – 2014 time periods.

## Factors Likely to Lead to Changes in the Next Three Years

The DEER team reviewed the two most recent NTGR evaluations and the overall literature to identify key factors that may lead to changes in the next three years for the residential HVAC EEMs and the programs that promote them.  The principal factors that were identified include:

- Potential changes in the level of rebates and qualification tiers for residential HVAC equipment,

- Significant changes in EEM incremental costs,

- Potential federal and state code updates including the time lag from the previous set of code updates (effective date requirements in EISA and Title 20/24 for code changes that are already approved),

- Significant changes in HVAC equipment, controls and system designs not common in the California residential market,

- Business trends in the contractor market, and customer disposable income and attitude towards major purchases (effect on program volumes and market share), and

- Changes in either Program Design or Sales Channels, e.g., target a narrow market sector.

## Assessment of Current Data and Tools Available to Prepare a Forecast of NTGR Values

The NTGR values for the residential HVAC EEM Categories underwent significant declines at the time when policy and practice shifted away from applying an NTGR value wholesale to an energy efficiency program to determining measure specific values per program delivery method and market segment that began with the 2004 – 2005 impact evaluation studies.  The IOU residential programs as a whole tended to use 0.80 as their default NTGR value.  Thus, central air conditioners declined from an NTGR of 0.80 to values that range from 0.63 to 0.67 depending on the equipment’s performance rating and delivery approach in the ’04-’05 evaluation studies.

There are no published analyses focused on either free-ridership or NTGR trends as a function of market adoption rates, equipment sales, incremental costs, efficiency levels, or program incentives, etc.  Data to inform such analysis will have to be gathered from a broad range of sources and compared.  For example, annual Energy Star qualified equipment market share data is available and covers several years as well as current lists of qualified equipment.  However, the Energy Star sales share information may only be useful as a comparative source of sales data.  The team recognizes that it is problematic to use the Energy Star collected information for anything beyond a general gauge of sales trends of energy efficient equipment.  Nonetheless, the information should be informative when compared to data and projections from other commercial data sources on equipment sales and shipments.  The California Energy Commission’s appliance database may also be a useful resource.  Other common residential data sources in California such as the Residential Appliance Saturation Studies (RASS) and the California Statewide Lighting and Appliance Efficiency Saturation Study (CLASS) focus on existing residential equipment and may contribute little to the proposed analysis.  Further review and consultation with analysts familiar with the detailed data fields in both the RASS and CLASS are needed to judge the usefulness of the datasets.  The Residential Market Share Tracking (RMST) studies may serve as a resource, but further consultation with the RMST analysts is needed.  Lastly, several Measure Cost studies have been conducted in California and they should provide adequate information for trending incremental costs against a number of different equipment characteristics.  Also, the program information on rebates over the years is possible to retrieve.  A number of statistical analysis and forecasting methods are readily available that can be used to determine whether there are any significant correlations between estimated free-ridership levels and the available data the team may gather and compare.

## Recommended NTGR Values for Future Programs

The majority of the 2011 NTGR values listed in Table 12-3 should not be adjusted to account for changed market conditions in 2013.  The one adjustment that is necessary is for the Energy Star Room Air Conditioner measure.  The team believes that the result from the 2006 – 2008 impact study may not accurately portray the present and future free-ridership levels because of recent data that suggest the market share of Room air conditioners is declining in recent years.  Unfortunately, there is insufficient time to complete a full analysis before the end of 2011.  Hence, we recommend increasing the NTGR rate from the 2006-08 studies from 0.36 to 0.46 to reflect the recent decline in the observed market adoption rate of Energy Star qualifying Room AC unit.

2004/2005 Statewide Residential Retrofit Single-Family Energy Efficiency Rebate Evaluation, Final Report, Itron/KEMA, October 2, 2007.

Residential Retrofit High Impact Measure Evaluation Report, prepared for the California Public Utilities Commission Energy Division, The Cadmus Group, Inc., February 8, 2010.

Evaluation Measurement and Verification of the California Public Utilities Commission HVAC High Impact Measures and Specialized Commercial Contract Group Programs, 2006 – 2008 Program Year, Final Consultant Report, Volumes 1 and 2, KEMA, February 10, 2010.

The Comprehensive Manufactured & Mobile Home Program (CMMHP) includes the following utility programs: PGE2078, SCE2502 and SDGE3035.

The HVAC HIM report indicates that for the “…SDG&E 3029 Residential AC Replacement, there were 322 survey sample completes, but only 153 were used to calculate the savings weighted NTG ratio.”

2004/2005 Statewide Express Efficiency and Upstream HVAC Program Impact Evaluation, Prepared for the California Public Utilities Commission and California’s Investor Owned Utilities, Submitted by Itron, Inc.; December 31, 2008.

From within Microsoft Word, you may double-click on a table in Appendix A-5.2 to launch the spreadsheet application on your computer to examine the underlying formulas and calculations.

Process Evaluation of Southern California Edison’s 2006-2008 Home Energy Efficiency Rebate (HEER) Program, Final Report, Report ID SCE0278, KEMA, Inc.; November 30, 2009; page 5-56.