# Recommended Updates to Net-to-Gross Ratios for Residential Water Heating Systems and Energy Efficiency Measures

## Introduction – Residential Hot Water Systems and Water Flow Restriction Measures

In this analysis, the most recent net-to-gross ratio (NTGR) results for more efficient water heating systems and low flow energy efficiency measures are compared to the existing NTGR values in the current DEER data base.  These NTGR estimates were part of a larger evaluation of residential retrofit energy efficiency programs conducted by various contractors for the Energy Division of the California PUC for the 2006-08 program cycle.  Based on these comparisons, the DEER Team derived a recommended NTGR value for the three measures in this technology group; residential aerators, low flow showerheads, and more efficient residential gas water heater systems.  For each measure, the Team compared the strengths and weaknesses of the methods used to derive NTGR results in the previous DEER and contrasted this to the methods used to estimate NTGR in previous evaluations of utility programs in California.  The factors considered by the DEER Team in making a final NTGR recommendation are the following:

- The relative merits of the methods and sampling plans used to derive NTGR for existing DEER values and results from new studies;

- Potential changes in program design or measure minimum qualification levels between the previous evaluation and 2006-08 evaluation;

- Rate of change in the market share of efficient measures or products over time; and

- Any available evidence on trends in rebates offered as a fraction of the incremental cost of the measures or more efficient hot water systems.

After making a recommendation for a specific NTGR value to use in this DEER 2011 update, the DEER Team identified the program and market factors that are likely to lead to changes in NTGR results between now and the first potential ex ante application of these values in 2013.  The Team then assess whether there is sufficient data and or analysis tools to accurately forecast the likely trend in NTGR values between 2008 and post-2012 for this specific measure and program design.  If there is sufficient data to make an accurate forecast absent any significant changes in program design and or qualifying levels for rebated measures, the Team produced a forecast of NTGR for use in program fielded post-2012.  These forecasts are based on the assumption that program design or qualifying efficiency level are not likely to significantly change between programs deployed in 2008 and those in post-2012.  If there are significant changes to these factors, the Team recommends the use of our unadjusted NTGR values based on the most recent evaluation research.

## Comparison of Current DEER NTGR to Results from Latest EM&V Analyses in 2006-2008

Table 13-1 compares the NTGR estimates in the existing DEER database to recent NTGR results from the 2006-2008 Cadmus study for similar measures and programs designs. The Current DEER data base contains default values for faucet aerators and low flow showerheads and values from the 2004-05 evaluation studies for residential gas water heaters.  These differences in NTGR for each of the three major measures are significant (greater than a 10% change) and could reflect changes in program design, evaluation methods, or the underlying market structure between 2005 and 2008.  Each potential cause is explored below.

Table 13-1: Overview of NTGR Results by Measure, Delivery Mechanism and Source

| Energy Efficiency Measure | Delivery Mechanism | Existing DEER NTGR (Source) | 06-08 NTGR (Source) |
| --- | --- | --- | --- |
| Faucet Aerators | Direct Install | 0.85 (Default NTGR for direct install in hard to reach, See 2008 DEER report ) | 0.59 SF, 0.65 MF (Cadmus, Res Retrofit 2006-08) |
| Low Flow Showerheads | Direct Install | 0.85 (Default NTGR for direct install) | SCG (MF) 0.72 SDGE (MF) 0.68 SDG&E (SF) 0.70 (Cadmus, Res Retrofit 2006-08) |
| Residential Gas Storage /Instantaneous Water Heater EF >0.62 >30 gallons | Downstream Rebate-Prescriptive | 0.58 (all res bldg types) Itron, 2004-5 Single Family Impact Evaluation) | 0.23 SDGE 0.18 PGE (Cadmus, Ibid) |

## Differences in Program Design

The Direct Install Program delivery method was used to promote low flow measures (faucet aerators and low flow showerheads) in both the 2004-05 and 2006-08 programs.  Similarly, the Prescriptive Rebate Program design used to promote more efficient gas water heaters from 2004-05 has not changed significantly relative to the design used and evaluated in 2006-08 programs.  Thus, the differences in NTGR estimates in this table are not likely related to changes in program design between the two program vintages.

## Comparison of Methods and Sample Sizes Used to Derive NTGR Results

Table 13-2 and Table 13-3 compare the methods and sample sizes used to estimate NTGRs for 2004-05 programs compared to 2006-08 programs.  Table 13-2 provides information on low flow showerheads and aerators.  Table 13-3 provides the corresponding information for residential gas water heater systems.  We discuss the potential impact of changes in method and different sample sizes before making a final conclusion on the final NTGR for each measures.

## Differences in Evaluation Methods

The 2006-08 evaluation for these water heating measures used the standardized set of self report questions adopted by the CPUC to estimate NTGR.  The 2006-08 evaluation survey batteries are relatively comprehensive relative to the shorter set of questions used in the 2004-05 evaluation of the same measures.  The more comprehensive set of questions was designed to tease out different types of impacts that the rebate offer might have on customer decisions including the potential to accelerate customer purchases of more efficient systems and influence customers to choose slightly levels of efficiency than they would have done in the absence of the program.  The large drop in NTGRs observed for these measures from 2005 to 2008 may be related to the use of these different survey batteries to determine NTGR in the 2006-08 survey.  Alternatively, the observed drop could be simply due to changes in market share of more efficient products over time.

## Differences in Sample Size

Table 13-2 and Table 13-3 show that a significantly larger sample was used to estimate NTGR in the 2006-08 program evaluations compared to the sample sizes used for the 2004-05 program evaluations.  Higher sample sizes should lead to more robust estimates of NTGR assuming an efficient sampling plan was used in each study.  This fact coupled with the earlier observation that the NTGR batteries used in 2008 were more comprehensive than the survey batteries from 2005 suggest that the NTGR ratios estimated for the 2006-08 programs should be used to replace the values currently in the DEER data base.

Table 13-2:  Methods and Sample Sizes for Aerators and Low-Flow Showerheads

| Measure | Market Segment and Program Year | NTGR Method Used | Sample Size and Precision if Known | Delivery Mechanism |
| --- | --- | --- | --- | --- |
| Aerators | All-2004-05 | Self Report or Default | 25 | Direct Install |
| Aerators | MF-2006-08 | Self Report | 43 | Direct Install |
| Aerators | SF- 2006-08 | Self Report | 150 | Direct Install |
| Low Flow Showerheads | All-2004-05 | Self Report | 25 | Direct Install |
| Low Flow Showerheads | MF-2006-08 | Self Report | 379 | Direct Install |
| Low Flow Showerheads | SF-2006-08 | Self Report | 15 | Direct Install |
| Low Flow Showerheads | MF-2006-08 | Self Report | 160 | Direct Install |

Table 13-3:  Methods and Sample Sizes for Residential Gas Water Heating Systems

| Measure | Market Segment and Program Year | NTGR Method Used | Sample Size and Precision if Known | Delivery Mechanism |
| --- | --- | --- | --- | --- |
| Gas Water Heaters | SF/MF- 2004-05 | Self report-simple | 53- no confidence interval reported | Downstream Customer Rebates |
| Residential Gas Storage Instantaneous Water Heater | SF/MF-PG&E-2006-08 | Self Report-comprehensive | 392—“ exceeded 90% confidence &10 precision” | Downstream Customer Rebates |
| Residential Gas Storage Instantaneous Water Heater | SF/MF-SDG&E-2006-08 | Self Report-Comprehensive | 456-- , “exceeded 90% confidence &10 %precision” | Downstream Customer Rebates |

## Recommended NTGR by Measure, Applicable Market Segments, and Building Types

Table 13-4 presents the recommended NTGR values for each measure and compares them to the NTGR values in the current DEER data base.  The DEER team finds that the most recent evaluation conducted for 2006-08 programs are likely to contain better, more precise information than the existing DEER based on the aforementioned review of methods, sample sizes, and changes in program delivery mechanisms.  Accordingly, the Team recommends use of the 2006-08 NTGR values as shown in Table 13-4 below.

Table 13-4:  Recommended NTGR by Measure and Delivery Method

| EEM | EEM Characteristics | Delivery Mechanism | Applicable Sector and Building Types | Utility Specific or Statewide NTGR | Recommended NTGR Value | Current DEER NTGR Value |
| --- | --- | --- | --- | --- | --- | --- |
| Faucet Aerators | NA | Direct Install | Single Family | Statewide | 0.59 | 0.85 (default for HTR) |
| Faucet Aerators | NA | Direct Install | Multifamily | Statewide | 0.65 | 0.85 (default for HTR) |
| Low Flow Showerheads | Flow less than 1.5 gallons per minute | Direct Install | Multifamily | SDGE | 0.72 | 0.85 (default for HTR) |
| Low Flow Showerheads | Flow less than 1.5 gallons per minute | Direct Install | Multifamily | SCG | 0.70 | 0.85 (default for HTR) |
| Low Flow Showerheads | Flow less than 1.5 gallons per minute | Direct Install | Single Family | SDGE | 0.68 | 0.85 (default for HTR) |
| Low Flow Showerheads | Flow less than 1.5 gallons per minute | Direct Install | All Re All Residential building types | Statewide | 0.70 | 0.85 |
| Residential Gas Storage Instantaneous Water Heater | Energy factor > 0.62 | Downstream Prescriptive Rebate | All Residential building types | SDG&E | 0.23 | 0.58 |
| Residential Gas Storage Instantaneous Water Heater | Energy factor > 0.62 | Downstream Prescriptive Rebate | All Residential building types | PG&E | 0.18 | 0.58 |

The most significant changes in NTGR values between the current DEER and the evaluation findings were for the residential gas water heaters.  For these water heating systems, NTGR values decreased by more than 50% for both utilities (from 0.59 to 0.23/0.18 for PGE and SDGE respectively).  The Team reviewed the survey design for 2006-08 to determine if there were any anomalies in design or weighting that could help explain this large and unexplained drop.  No obvious flaws in the survey design or questionnaires were discovered. More details on the methods used and recommendations from these evaluations can be found in Appendix A-6.

The changes in NTGR observed for faucet aerators and low flow showerheads were primarily because the current DEER estimate of 0.85 was a default value while the 2006-08 values are based on a self report analysis. The differences in estimated NTGR for low flow showerheads of 0/72, 0.70, and 0.68 for Multifamily and Single family units are not statistically significant. Thus the Team recommends the use of a statewide “average” NTGR value of 0.70 for all residential building types where low flow showerheads are installed.

The 2006-08 study for water heating systems contained NTGR estimates for each of the three program years that gradually declined from 2006 to 2008 by roughly 1% per year. This slow decline suggests a stable market where a gradually larger percentage of customers were adopting efficient gas water heating systems with Energy Factors above 0.62 in the absence of the program.  The most obvious potential explanation for a significant drop in the NTGR value between 2004 and 2008 would be the introduction of a new efficiency standard for these systems in the period right before the program was launched in 2005.  Another possible explanation would be if the rebate level had been kept the same at 0.62 Energy factor level for many years.

## Factors Likely to Lead to Changes in NTGR Over the Next Three Years

The DEER team reviewed the two most recent evaluations in California and the overall NTGR literature to identify key factors likely to lead to changes in NTGR over time for water heating measures.  In the previous DEER update factors 1&2 below were identified as having a significant impact on NTG; efficiency market share and rebates as a fraction of incremental cost.  In this update we identify and discuss two more factors (3 and 4 below). The four principal factors are:

- Rate of growth in the market share of more efficient hot water systems or specific measures such as low flow showerheads,

- Trend in rebates as a share of incremental costs of efficient measures,

- Changes in the structure of the contractor service offerings, and

- Changes in Program Design or Sales Channel – example:  move from downstream to upstream for HVAC system rebates.

## Assessment of Current Data and Tools Available for Adjusting NTGR Over Time

There is insufficient data on the market share of low flow showerheads or the saturation of aerators to make adjustments to the 2008 NTGRs for use in post 2012 program years.  Thus, the discussion focuses on potential changes for NTGR for efficient gas water heaters below.

## Efficient Market Share of Gas Water Heating Systems

Table 13-5 provides data on changes in market share of more efficient gas water heaters is available for in California from 2000 to 2005 and at the national level from 2000 to 2010.

Table 13-5:  Trends in Efficient Market Share for Gas Water Heater Systems

| Measure | 2000 | 2005 | 2010 | Source |
| --- | --- | --- | --- | --- |
| Gas Water Heater- Average Energy Factor Mkt Share >.64 EF | 23.1% | 26.8% | NA | California Data |
| Electric Water Heaters Market share >.92 EF | 6.3% | 12.6% | NA | None |

These historical trends in market share suggest the market share of more efficient gas water heaters will continue to increase at a slow but stable rate over the next three to five years.  Holding all other factors constant, we would expect that the NTGR for this program is likely to continue to fall in correlation with increases in market share for more efficient product if qualifying levels for rebated equipment are not adjusted upward.  However the observed drop in NTGR from 0.58 in 2005 to 0.23 in 2008 suggests that NTGR are not strongly correlated with the relatively small increases in market share for efficient water heaters with an EF above 0.64 in this table.  Hence we would not recommend making an adjustment to NTGR for the 2013 program year and beyond based on forecast of market share alone.

## Trends in Rebate Levels as a Fraction of Incremental Cost

Rebate levels were decreased from $40 to unit in the 2005 program to $30/ unit for the 2008 program.  At the same time the incremental cost of more efficient water heaters has declined from $175/unit in 2005 to $ 150/unit in 2008.  Thus, the fraction of rebates compared to incremental cost has remained relatively constant, from 23% to 20% over four years.  This small change is not likely to cause significant changes in NTGR over time and there is insufficient data available to project future trends in the ratio or rebates to incremental costs.  As a result no adjustment in NTGR is recommended.

## Contractor Market

Contractor market for water heater installation has fallen to record low levels of replacement sales and sales in the new construction market are also quite low.  In addition, customer disposable income has been falling for four years in California.  There have been no significant changes in the methods used by contractors to sell water heaters.  Given these facts, no adjustments to NTGR are recommended based on this factor.

## Program Design or Sales Channel

No significant changes in program design were observed between 2005 and 2008 programs.  Review of the commission’s strategic plan for 2011 and beyond suggest that some changes in program design may be on the way but there is insufficient data to forecast the timing of any changes in program design with certainty.  As a result we recommend no adjustment to the NTGR based on potential changes in the program channel used to promote these water heating systems.

On the other hand there is a plethora of new types of water heating systems coming to market over the last three years which could reduce the incremental costs of these more efficient gas water heaters because of increased competition.  A recent study from ACEEE lists 16 different types of water heating systems more efficient than the water heating systems promoted in the utility programs and reports incremental costs ranging from $200 to $3000 per system at a cost of conserved gas ranging from $ 0.75 to $3.26 per therm.  The study claims that seven of these sixteen water heating systems are cost effective at current prices.  If even a few of these new systems are able to achieve market share over 5%, it is likely that the new products will induce changes in program design or qualifying requirement for efficient gas systems.  We lack confidence in how these two competing factors (competition from new technologies and declining incremental costs will interact over time to influence net NTGR’s.  As a result we recommend no adjustment to the NTGR based on this factor.

## Synthesis of NTGR Forecasting and Adjustment Analysis

Given the lack of data on trends in the market share of more efficient gas systems and rebates as a function of incremental costs, and the overall decline in the sales volume contractor market, the DEER team finds there is insufficient data or forecasting tools available to reliably adjust the latest NTGR from 2008 values for program years in 2013 and beyond.  Thus, the Team recommends no adjustments be made to the NTGR values presented in Table 13-4 for use in residential water heater programs fielded after 2013.

The Cadmus Group, Residential Retrofit High Impact Measure Evaluation Report and Appendices, (Prepared for the Energy Division of the CPUC, February 8, 2010).

Itron Inc, 2004/2005 Statewide Residential Retrofit Single-Family Energy Efficiency Rebate Evaluation; CPUC-ID#:1115-04, September 2007.

Harvey Sachs, Jacob Talbot and Nate Kaufman, Emerging Hot Water Technologies and Practices as of 2011 (ACEEE, Research Report A112, October, 2011) see Table 1.