# Recommended Updates to Net-To-Gross Ratios for Residential Appliances

## Introduction

In this analysis, the most recent net to gross ratio (NTGR) results for more energy efficient appliances are compared to the existing NTGR values in the current DEER data base. These NTGR estimates were part of the 2006-08 evaluation of residential retrofit energy efficiency programs conducted by various contractors for the Energy Division of the California PUC.   Based on these comparisons, the DEER team derives a recommended NTGR value for the three measures in this technology group: residential clothes washers, refrigerators and freezers.  For each measure, we compare the strengths and weaknesses of the methods used to derive NTGR results in the previous DEER and contrast this to the methods used to estimate NTGR in previous evaluations of utility programs in California.  The factors considered by the DEER team in making a final NTGR recommendation are:

- The relative merits of the methods and sampling plans used to derive NTGR for existing DEER values and results from new studies.

- Potential changes in program design or measure minimum qualification levels between the previous evaluation and 2006-08 evaluation.

- Rate of change in the market share of efficient measures or products over time. Faster rates of change are likely to lead to reduced NTGRs over time.

- Any available evidence on trends in rebates offered as a fraction of the incremental cost of the measures. NTGRs are expected to decrease as the ratio of rebates to incremental cost increase.

After making a recommendation for a specific NTGR value to use in this DEER 2011 update, the DEER team identified the program and market factors are likely to lead to changes in NTG results between now and the first potential ex ante application of these values in 2013.  The team then assesses whether there is sufficient data and or analysis tools to accurately forecast the likely trend in NTGR values between 2008 and 2013 for this specific measure and program design.  If there is sufficient data to make an accurate forecast absent any significant changes in program design and or qualifying levels for rebated measures, we produce a forecast of NTGR for use in program fielded in 2013-2014. These forecasts are based on the assumption that program design or qualifying efficiency level are not likely to significantly change between programs deployed in 2008 and those in 2013. If there are significant changes to these factors, we recommend the use of our unadjusted NTGR values based on the most recent evaluation research.

## Comparison of Current DEER NTGR Values to Results 2006-2008 EM&V Analyses

Table 14-1 compares the NTGR estimates in the existing DEER database to recent NTGR results from the 2006-2008 Cadmus study for similar measures and programs designs. These differences in NTGR for each of the three major appliance measures are significant (greater than a 10% change) and could reflect changes in program design, evaluation methods, or the underlying market structure between 2005 and 2008.  Each potential cause is explored below.

Table 14-1:  Overview of NTGR Results by Measure and Delivery Mechanism

| Energy Efficiency Measure | Delivery Mechanism | Existing DEER NTGR Values (Source) | 06-08 NTGR Results (Source) |
| --- | --- | --- | --- |
| Clothes Washers >1.72 MEF | Downstream prescriptive | 0.81 (Itron, 2007) | PGE2000 = 0.31 SDGE3023 = 0.31 SCG3517 = 0.29 (CADMUC, 2010) |
| Clothes Washers 15% Above Standard | Downstream prescriptive | 0.85 (Itron, 2007, ibid) |  |
| Refrigerator, efficiency characteristics of recycled unit | Downstream prescriptive | 0.614 (ADM, 2008) | PGE2000 0.51 SCE2500 0.56 SDGE3028 0.58 (CADMUC, ibid) |

*2006-08 evaluation is limited to only the refrigerators recycled through each utility’s ARP.

### Differences in Program Design

The prescriptive rebate program design used to promote more energy efficient clothes washers and to prevent the continued operation of older, inefficient refrigerators and freezers from 2004-05 has not changed significantly relative to the design used and evaluated in 2006-08 programs.  The only program change, beginning in 2006, involved expanding eligibility to include small commercial businesses, including office complexes, industrial customers, schools, and municipalities.  Consequently the differences in NTGR estimates in Table 14-1 are not likely related to changes in program design between the two evaluation periods.

### Differences in Evaluation Methods

Table 14-2 compares the methods and sample sizes used to estimate NTGRs for 2004-05 programs compared to 2006-08 programs. Table 14-2 provides information on clothes washers, refrigerators and freezers.  The potential impact of changes in method and different sample sizes are discussed here before making a final conclusion on the final NTGR for each measure.

The 2006-08 evaluation for the measures reviewed used the standardized set of self report questions adopted by the CPUC to estimate NTGR. The 2004-05 evaluation interviewed participants and nonparticipants and also used discrete choice analyses to determine NTGRs while the 2006-08 evaluation interviewed only participants.  However, the 2006-08 survey batteries are comprehensive relative to the shorter set of questions used in the 2004-05 evaluation of the same measures. The more comprehensive set of questions was designed to tease out different types of impacts that the rebate offer might have on customers to discard inefficient refrigerator/freezers and influence customers to choose slightly higher levels of clothes washer efficiency than they would have done in the absence of the program.  The large drop (50%-52%) in NTGRs observed for clothes washers from 2005 to 2008 may be related to the use of these different survey batteries to determine NTGR in the 2006-08 survey.  Alternatively the observed drop could be simply due to changes in market share of more efficient products over time.

### Differences in Sample Sizes

For clothes washers, all ENERGY STAR / CEE Tier 1-3 machines were grouped together in the evaluation analysis.  For refrigerators and freezers, only recycled units were included in the evaluation analysis.  A significantly larger sample was used to estimate NTGR in the 2008 program evaluations compared to the sample sizes used for the 2004-05 program evaluations. Higher sample sizes should lead to more robust estimates of NTGR assuming an efficient sampling plan was used in each study. This fact coupled with the earlier observation that the NTGR batteries used in 2008 were more comprehensive than the survey batteries from 2005 suggest that the NTGR ratios estimated for the 2006-08 programs should be used to replace the values currently in the DEER data base.

Table 14-2:  Methods and Sample Sizes for Residential Appliances EEMs in the 2006-2008 EM&V Studies

| Measure | Market Segment and Program Year | NTGR Method Used | Sample Size | Delivery Mechanism |
| --- | --- | --- | --- | --- |
| Clothes Washers >1.72 MEF | Residential – 2006-08 | Self Report and Discrete Choice | PGE2000 551 SCG3517 323 SDGE3023 116 Total: 990 | Downstream Prescriptive |
| Clothes Washers 15% Above Standard | Residential – 2006-08 | Not evaluated |  | Downstream prescriptive |
| Refrigerator, efficiency characteristics of recycled unit | Residential – 2006-08 | Self Report | 1,173 non-participants PG&E – 505 SCE – 248 SDGE – 420 1857 participants PG&E – 712 SCE – 573 SDG&E – 572 81 Market actors | Downstream Prescriptive |
| Freezer, efficiency characteristics of recycled unit | Not evaluated Of the three appliances (refrigerators, freezers, and room air conditioners), only the recycling of refrigerators was identified as a high impact measure (HIM) by the California Public Utilities Commission (CPUC). As a result, the evaluation is limited to only the refrigerators recycled through each utility’s ARP. |  |  |  |
| Refrigerator/Freezer*, 15% >current standard |  |  |  |  |

## Recommended NTGR by Measure and Applicable Market Segments or Building Types

Table 14-3 presents the recommended NTGR values for each measure and compares them to the NTGR values in the current DEER data base.  The most significant changes in NTGR values between the current DEER and the evaluation findings were for the residential clothes washers.  For clothes washers, NTGR values decreased by more than 50% for both utilities (from 0.81 to 0.31/0.29 for PG&E, SDG&E, and SCG respectively).  The Team reviewed the survey design for 2006-08 to determine if there were any anomalies in design or weighting that could help explain this large and unexplained drop. No obvious flaws in the survey design or questionnaires were discovered.  The most obvious potential explanation for a significant decline in the NTGR value between 2004 and 2008 would be the introduction of a new efficiency standard for these systems in the period right before the program was launched in 200. More details on the methods used and recommendations from these evaluations can be found in Appendix A-7.

Table 14-3:  Recommended NTGR by Measure and Delivery Method

| EEM | EEM Characteristics | Delivery Mechanism | Applicable Sector and Building Types | Utility Specific or Statewide NTGR | Recommended NTGR Value | Current DEER NTGR Value |
| --- | --- | --- | --- | --- | --- | --- |
| Clothes Washers | CW >1.72 MEF | Downstream Prescriptive | All residential | Utility specific | PGE2000 - 0.31 SDGE3023 - 0.31 SCG3517- 0.29 | 0.81 |
| Clothes Washers | 15% > DOE Standard | Downstream Prescriptive | All residential | Statewide | Current DEER value | 0.85 |
| Refrigerator | Recycled unit | Downstream Turn-in Recycling | All residential | Utility specific | PGE2000 -0.51 SCE2500 - 0.56 SDGE3028 - 0.58 | 0.614 |
| Freezer | Recycled unit | Turn-in Recycling | All Residential | Statewide | Current DEER Value (not evaluated in the 0608 study) | 0.702 |
| Refrigerator/Freezer | 15% > current standard | Downstream Prescriptive | All residential | Statewide | Current DEER Value (not evaluated in the 0608 study) | 0.75 |

## Factors Likely to Lead to Changes in NTGR Over Next Three Years

The DEER team reviewed the three most recent NTGR evaluations and the overall literature to identify key factors likely to lead to changes in NTGR over time for Residential appliance measures.  The four principal factors were:

- Program-qualifying efficiency levels

- Program design, (including incentive levels

- Trends in market share of Clothes Washers

- Share of the incremental equipment cost being paid for by the program rebate

## Assessment of Current Data and Tools Available for Adjusting NTGR Over Time

The DEER team finds that the most recent evaluation conducted in 2006-08 are likely to contain better, more precise information than the existing DEER based on the aforementioned review of methods, sample sizes and changes in program delivery mechanisms. Accordingly the Team recommends use of the 2006-08 NTGR values, where a 0608 evaluation was conducted, as shown in Table 3 below, with no adjustment to NTGRs over time.

## Synthesis of NTGR Adjustment Analysis

The DEER team believes that there is inadequate information to make any significant adjustments to the recommended NTGR values derived from the 2006 - 2008 evaluations.  Thus, the DEER team recommends that no adjustments be made to the NTGR values presented in Table 14-3.

The Cadmus Group,  Residential Retrofit High Impact Measure Evaluation Report and Appendices,(Prepared for the Energy Division of the CPUC, February 8, 2010,)

Itron Inc, 2004/2005 Statewide Residential Retrofit Single-Family Energy Efficiency Rebate Evaluation; CPUC-ID#:1115-04, September 2007.

Evaluation Study of the 2004-2005 Statewide Residential Appliance Recycling Program, ADM, April, 2008